3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210263

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1590 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to cads_NR_CellConfig_C_RNTI_REQ |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently, the TTCN-3 Core Languague definition "ETSI ES 201 873-1"does not clearly differenciate between "explicit omit" / "implicit omit" context. But the TTCN-3 code in RAN5 ATS is written under the assumption that "implicit omit" is used. For other context, this may result in optional template fields being left uninitialized when template modification mechanism as per "ETSI ES 201 873-1 clause 6.2.1 – Record type and values" is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added “implicit omit” to template declarations to initialize optional template fields. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation not in line with TTCN-3 Core Language definitions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Nearly all, as common NR template definitions are affected (see cl. 1.2). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to template cads_NR_CellConfig_C_RNTI_REQ	3

1.2	Similar changes are required for other templates	4

# Corrections

## Correction to template cads_NR_CellConfig_C_RNTI_REQ

| Template name | cads_NR_CellConfig_C_RNTI_REQ |
| --- | --- |
| Reason for change | In the template definition of cads_NR_CellConfig_C_RNTI_REQ modifying cas_NR_CellConfigAllOmit_REQ, parameters CellTimingInfo and InitialCellPower are not initialized. This result in optional template fields being left uninitialized. |
| Summary of change | Added “implicit omit” to template declarations to initialize optional template fields. |
| TTCN module | NR_CellCfg_Templates |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfig_C_RNTI_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) RNTI_Value_Type p_RNTI_Value, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5-206369: added p_FollowOnFlag sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := omit, CellConfigCommon := { C_RNTI:= p_RNTI_Value } } } } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) NR_SYSTEM_CTRL_REQ cads_NR_CellConfig_C_RNTI_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) RNTI_Value_Type p_RNTI_Value, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ //@sic R5-206369: added p_FollowOnFlag sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag, p_FollowOnFlag), Request := { Cell := { AddOrReconfigure := { StaticResourceConfig := omit, CellConfigCommon := { C_RNTI:= p_RNTI_Value } } } } } with { optional "implicit omit" }; <<SKIPPED CODE>>

## Similar changes are required for other templates

in module NR_CellCfg_Templates

- cads_NR_ServingCellConfig_REQ

- cads_NR_RachProcedureConfig_REQ

- cads_NR_CellConfigPhysicalLayerUplink

- cads_NR_UplinkBWP_ConfigCommon_REQ

- cads_NR_ULGrantAllocation_REQ

- cads_NR_PDSCH_Config_REQ

- cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ

- cads_NR_SearchSpaceUlDciAssignment_REQ

module NR_CommonTemplates_Beams

- cads_NR_SSB_BeamArray_REQ

- cads_NR_CSI_Config_REQ

module NR5GC_SysinfoPaging

- cads_NR_ModifySysinfo_Def_REQ

- cads_NR_DciTriggerShortMessage_REQ

module MAC_BWP_Common_NR

- cads_NR_BWP_CONFIG_DL_REQ

- cads_NR_BWP_CONFIG_UL_REQ

modeule MAC_TC_Common_NR

cads_NR_DownlinkBWP_ConfigCommon_REQ

cads_NR_UL_DL_BWP_ConfigCommon_REQ

cads_NR_DciTriggerPDCCHOrder_REQ

cads_NR_TDD_UL_DL_ConfigCommon_REQ