3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210265

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1591 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In this function the second parameter is defined as "ENDC_Bearer_Type", this is defined as type ENDC_DRB_Configuration_Type ENDC_Bearer_Type (MCG_SCG, MCG_Split, MCG_Only); In some cases (8.2.2.4.1 for .e.g) the second parameter that is passed into this function is not limited to ENDC_Bearer_Type this results in Runtime exception OutOfValueConstraintException f_TC_8_2_2_4_1_ENDC_EUTRA-> fl_TC_8_2_2_4_1and5_ENDC_EUTRA_TestBody -> f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only) -> v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the second parameter to ENDC_DRB_Configuration_Type |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execution will result in runtime error due to OutOfValueConstraintException |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.4.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated	3

# Corrections

## Correction to f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated

| Template name | f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated |
| --- | --- |
| Reason for change | In this function the second parameter is defined as "ENDC_Bearer_Type", this is defined as type ENDC_DRB_Configuration_Type ENDC_Bearer_Type (MCG_SCG, MCG_Split, MCG_Only); In some cases (8.2.2.4.1 for .e.g) the second parameter that is passed into this function is not limited to ENDC_Bearer_Type this results in Runtime exception OutOfValueConstraintException f_TC_8_2_2_4_1_ENDC_EUTRA-> fl_TC_8_2_2_4_1and5_ENDC_EUTRA_TestBody -> f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, RBConfig_Only) -> v_RadioResourceConfigDedicated := f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(p_CellId, p_DRB_Type, p_RRC_State); |
| Summary of change | Changed the second parameter to ENDC_DRB_Configuration_Type |
| TTCN module | EUTRA38_ENDC_CommonProcedures |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(EUTRA_CellId_Type p_CellId, ENDC_Bearer_Type p_DRB_Type, EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE) runs on EUTRA_PTC return template (omit) RadioResourceConfigDedicated { var template (omit) RadioResourceConfigDedicated v_RRCMsg := omit; var template (value) DRB_ToAddModList v_DRBList; var template (omit) EPS_BearerIdentityList v_ListOfIds := f_EUTRA_MobileInfo_GetEPS_BearerList (); // @sic R5-199103 sic@ var EPS_BearerIdentityList v_BearerIds; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_CellId); //Get FDD or TDD mode from Cell-configuration var template (omit) PhysicalConfigDedicated v_PhysicalConfigDedicated; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (p_CellId); var integer i; v_PhysicalConfigDedicated := cs_508_PhysicalConfigDedicated_Default_RBC(v_EUTRA_FDD_TDD_Info.cqi_ReportConfig, v_EUTRA_FDD_TDD_Info.soundingRS_UL_Config, v_AntennaInfo); if (isvalue(v_ListOfIds)) { v_BearerIds := valueof(v_ListOfIds); } else { FatalError(__FILE__, __LINE__, "Bearer Id not stored"); } for (i:= 0; i < lengthof(v_BearerIds); i := i+1) { v_DRBList[i] := cs_508_DRB_ToAddMod_DEFAULT_AM (f_EUTRA_NB_EpsBearerAssociatedDRB(v_BearerIds[i])); } select (p_RRC_State) { case (RRC_IDLE) { // Need to configure SRB2 + MAC + Phy too select (p_DRB_Type) { case (MCG_Split) { // add DRB2 to the end of the list v_DRBList[lengthof(v_DRBList)] := cs_508_DRB_ToAddMod_ENDC_Split; v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); //@sic R5-200989 sic@ } case(MCG_Only){ // Same configuration on LTE side - no dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL, v_PhysicalConfigDedicated); } case else { // Same configuration on LTE side - with dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); } } } case else { // RRC_CONNECTED select (p_DRB_Type) { case (MCG_Split) { // only the new DRB to be included v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC); //@sic R5-200989 sic@ } } } } return v_RRCMsg; }

After Change:

function f_EUTRA38_BuildEUTRA_RadioResourceConfigDedicated(EUTRA_CellId_Type p_CellId, ENDC_DRB_Configuration_Type p_DRB_Type, //WA# changed from ENDC_Bearer_Type p_DRB_Type, as the types passed are based on ENDC_DRB_Configuration_Type EUTRA_RRC_STATE_Type p_RRC_State := RRC_IDLE) runs on EUTRA_PTC return template (omit) RadioResourceConfigDedicated { var template (omit) RadioResourceConfigDedicated v_RRCMsg := omit; var template (value) DRB_ToAddModList v_DRBList; var template (omit) EPS_BearerIdentityList v_ListOfIds := f_EUTRA_MobileInfo_GetEPS_BearerList (); // @sic R5-199103 sic@ var EPS_BearerIdentityList v_BearerIds; var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_CellId); //Get FDD or TDD mode from Cell-configuration var template (omit) PhysicalConfigDedicated v_PhysicalConfigDedicated; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (p_CellId); var integer i; v_PhysicalConfigDedicated := cs_508_PhysicalConfigDedicated_Default_RBC(v_EUTRA_FDD_TDD_Info.cqi_ReportConfig, v_EUTRA_FDD_TDD_Info.soundingRS_UL_Config, v_AntennaInfo); if (isvalue(v_ListOfIds)) { v_BearerIds := valueof(v_ListOfIds); } else { FatalError(__FILE__, __LINE__, "Bearer Id not stored"); } for (i:= 0; i < lengthof(v_BearerIds); i := i+1) { v_DRBList[i] := cs_508_DRB_ToAddMod_DEFAULT_AM (f_EUTRA_NB_EpsBearerAssociatedDRB(v_BearerIds[i])); } select (p_RRC_State) { case (RRC_IDLE) { // Need to configure SRB2 + MAC + Phy too select (p_DRB_Type) { case (MCG_Split) { // add DRB2 to the end of the list v_DRBList[lengthof(v_DRBList)] := cs_508_DRB_ToAddMod_ENDC_Split; v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); //@sic R5-200989 sic@ } case(MCG_Only){ // Same configuration on LTE side - no dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL, v_PhysicalConfigDedicated); } case else { // Same configuration on LTE side - with dualConnectivityPHR configured in MAC-MainConfig @sic R5-200989 sic@ v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig({cs_508_SRB2_ToAddMod_DEFAULT}, v_DRBList, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC, v_PhysicalConfigDedicated); } } } case else { // RRC_CONNECTED select (p_DRB_Type) { case (MCG_Split) { // only the new DRB to be included v_RRCMsg := cds_RadioResourceConfigDedicated_SrbDrbConfig(-, {cs_508_DRB_ToAddMod_ENDC_Split}, cs_508_MAC_MainConfig_Explicit_RBC_DrxL_DC); //@sic R5-200989 sic@ } } } } return v_RRCMsg; }