3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210282

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1598 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During intercell HO at steps 8-10 and 23-25, when only 1 UL grant is provided by TTCN for MSG5 and for receiving ReconfigurationComplete, it might lead to instability as TTCN needs to provide exact timing of UL grants. However, it depends on the timing of when UE will start random access procedure for intercell HO. It is proposed to provide continuous grants during HO until PDCP status report is received. Once PDCP status report is received in TTCN, continuous grants will then be stopped |  |  |  |  |  |  |  |  |
| Summary of change: |  | Start continuous cyclic grants for receiving RRCReconfigurationComplete and PDCP status report When PDCP status report is received, stop transmission of UL grants. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | 1. During intercell HO from nr_Cell1 to nr_Cell2 at steps 8-10, when only 1 UL grant is provided by TTCN for MSG5 and for receiving ReconfigurationComplete, it might lead to instability as TTCN needs to provide exact timing of UL grants. However, it depends on the timing of when UE will start random access procedure for intercell HO. It is proposed to provide continuous grants during HO until PDCP status report is received. 2. Once PDCP status report is received in TTCN, continuous grants will then be stopped. |
| Summary of change | 1. Start continuous cyclic grants for receiving RRCReconfigurationComplete and PDCP status report 2. When PDCP status report is received, stop transmission of UL grants. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

…

// for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {

v_Grants:=  2;

}

//@siclog "Step 10" siclog@

// The SS assigns  UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and

//@sic R5s201484 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20);

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@    //Anritsu TTCN CR R5s210032 MCC

interleave {//@sic R5s201484 sic@

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(5)})){}

}

}           else{   //Anritsu TTCN CR R5s210032 MCC

…

}

After change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

…

//@siclog "Step 10" siclog@

// The SS assigns  UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and

// 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@

//@sic R5s201484 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20);

// for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {  // if NR5GC

f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20);

} else {  // if ENDC

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

}

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@    //Anritsu TTCN CR R5s210032 MCC

interleave {//@sic R5s201484 sic@

//Receive RRCReconfigurationComplete

[]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){}

//@siclog "Step 11" siclog@

//Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS:

//D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5?

[]DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2,

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(5)})){

f_NR_ULGrantConfiguration_Stop(nr_Cell2);

}

}

}           else{   //Anritsu TTCN CR R5s210032 MCC

…

}

## 2.2 Change 2

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | 1. During intercell HO from nr_Cell2 to nr_Cell1 at steps 23-25, When only 1 UL grant is provided by TTCN for MSG5 and for receiving ReconfigurationComplete, it might lead to instability as TTCN needs to provide exact timing of UL grants. However, it depends on the timing of when UE will start random access procedure for intercell HO. It is proposed to provide continuous grants during HO until PDCP status report is received. 2. Once PDCP status report is received in TTCN, continuous grants will then be stopped. |
| Summary of change | 1. Start continuous cyclic grants for receiving RRCReconfigurationComplete and PDCP status report 2. When PDCP status report is received, stop transmission of UL grants. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_7_1_3_4_1_NR_Step25to30 (inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

…

// for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {

v_Grants:=  2;

}

//@siclog "Step 24" siclog@

// The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report.

// 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20);

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@    //Anritsu TTCN CR R5s210032 MCC

interleave {//@sic R5s201484 sic@

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(5)})){

f_NR_ULGrantConfiguration_Stop(nr_Cell2);

}

}

}           else{   //Anritsu TTCN CR R5s210032 MCC

…

}

After change:

function f_TC_7_1_3_4_1_NR_Step25to30 (inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

…

//@siclog "Step 24" siclog@

// The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report.

// 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@

//@sic R5s201484 sic@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20);

// for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU

// For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU

if ( p_WaitforRRCConCmp) {  // if NR5GC

f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20);

} else {  // if ENDC

f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants);

}

//Anritsu TTCN CR R5s210032 MCC

if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@

p_NR_PDCP_State.DRB_Id,

cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@

-,

{cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){

f_NR_ULGrantConfiguration_Stop(nr_Cell1);

} //Anritsu TTCN CR R5s210032 MCC

}

}

else{   //Anritsu TTCN CR R5s210032 MCC

…

}