3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210311

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0654 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 6.3 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Rohde & Schwarz, and Keysight |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-20 |  |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |
| Reason for change: |  | To add NR TC 6.3 Re-Registration Scenarios / 5GS to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 6.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.NR5GC |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS34.229-5 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |  |
| Other comments: |  | Prose CR associated |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	7

2.4 Change 4	10

Additional changes from RnS:	12

2.5 Change 5	12

2.6 Change 6	14

3	Execution Log Files	17

3.1	Qualcomm SM8350 + SDX60	17

3.2	Samsung SM-G986U	17

4	References	17

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Hans Rohnert

Hans.Rohnert@rohde-schwarz.com

Mohit Kanchan

mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	6.3

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Rohde & Schwarz 5G Protocol Conformance Test platform, and Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8350 + SDX60, Samsung NeusERD S5188

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Test case name | 6.3 |
| --- | --- |
| Reason for change | Current guard timer may not be enough only to cover the actual running time even in case there are no delays in test execution as certain steps require larger delays (e.g. step 22 requires 1000s delay). Therefore, it is proposed to double the current value as guard timer. |
| Summary of change | Guard timer doubled. |
| TTCN module | IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment |  |

Before change:

testcase TC_6_3_NR5GC() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   Re-Registration Scenarios / 5GS

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(1200);

v_IMS1 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_6_3_NR5GC_IMS1());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

After change:

testcase TC_6_3_NR5GC() runs on MTC_IMS system SYSTEM_IMS {

// @purpose

//   Re-Registration Scenarios / 5GS

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(2400);

v_IMS1 := IMS_PTC.create alive;

f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2);

v_IMS1.start(f_TC_6_3_NR5GC_IMS1());

t_GuardTimer.start;

f_MTC_MainLoop_IMS(t_GuardTimer);

}

## 2.2 Change 2

| Function name | f_TC_6_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - SIP registration in step 12 is no more initial one, as that happen already in step 1. Therefore ServiceRequest procedure should be expected instead of Registration procedure. |
| Summary of change | - fl_TC_6_3_InitialRegistration is called with “IPCAN_MO_IMS_Signalling” instead of with “IPCAN_InitialRegistration” parameter. |
| TTCN module | NR5GC_6\IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC

{

var SecurityClientParams_Type v_SecurityClientParams;

var charstring v_Expires;

var float v_TimerVal;

var integer v_NonceCount;

var integer v_StatusCode;

timer t_TRegExpireMin;

timer t_TRegExpireMax;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

v_Expires := "120";

v_TimerVal := 60.0;

fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

// @siclog "Step 10 - 11" siclog@

v_NonceCount := 2;

v_StatusCode := 500;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

f_IMS_GlobalRelease();

// @siclog "Step 12 - 19" siclog@

v_Expires := "360";

v_TimerVal := 180.0;

fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

// @siclog "Step 20 - 21" siclog@

v_NonceCount := 2;

v_StatusCode := 200;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");

// @siclog "Step 22 - 27" siclog@

v_NonceCount := 3;

v_StatusCode := 423;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

// @siclog "Step 28 - 31" siclog@

v_NonceCount := 5;

v_StatusCode := 401;

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling);

}

After change:

function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC

{

var SecurityClientParams_Type v_SecurityClientParams;

var charstring v_Expires;

var float v_TimerVal;

var integer v_NonceCount;

var integer v_StatusCode;

timer t_TRegExpireMin;

timer t_TRegExpireMax;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

v_Expires := "120";

v_TimerVal := 60.0;

fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

// @siclog "Step 10 - 11" siclog@

v_NonceCount := 2;

v_StatusCode := 500;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

f_IMS_GlobalRelease();

// @siclog "Step 12 - 19" siclog@

v_Expires := "360";

v_TimerVal := 180.0;

//

fl_TC_6_3_InitialRegistration(IPCAN_MO_IMS_Signalling, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

// @siclog "Step 20 - 21" siclog@

v_NonceCount := 2;

v_StatusCode := 200;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");

// @siclog "Step 22 - 27" siclog@

v_NonceCount := 3;

v_StatusCode := 423;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

// @siclog "Step 28 - 31" siclog@

v_NonceCount := 5;

v_StatusCode := 401;

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling);

}

## 2.3 Change 3

| Function name | fl_TC_6_3_ReRegistration |
| --- | --- |
| Reason for change | - When SIP REGISTER is received it is checked whether it contains the correct value for Expires header which by default is expected to be tsc_IMS_ExpiresDef (=600000). However for step 24 that would be incorrect because UE received 800000 after step 23 in Contact header. Therefore explicit value for this step needs to be signalled in f_IMS_PTC_ReRegistration_WaitForRegistration which declaration is extended in Change 4. - Similarly, the problem with Expires header is also in step 28 and 30 so explicit expiry value needs to be signalled in f_IMS_REGISTER_SubsequentRequest function. Prose CR is associated with this change. |
| Summary of change | - When SIP REGISTER is being processed for step 24 an explicit value for matching Expiry header is sent. - When SIP REGISTER is being processed for step 28 an explicit value for matching Expiry header is sent. |
| TTCN module |  |
| MCC160 Comment |  |

Before change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_NonceCount,

integer p_StatusCode,

template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var REGISTER_Request v_RegisterReq;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var charstring v_Expires;

var float v_TimerVal;

var SipUrl v_ContactUrl;

var template (value) RegInfo_RegistrationList_Type v_RegistrationList;

var charstring v_PublicUserId;

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

v_IMS_REGISTER_REQ := f_IMS_PTC_ReRegistration_WaitForRegistration(p_TRegExpireMin, p_TRegExpireMax, p_NonceCount);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

if (isvalue(p_PrevSecurityClientParams)) {

f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams));

}

select (p_StatusCode) {

case (500) {

// @siclog "Step 11" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader))));

}

case (200) {

// @siclog "Step 21" siclog@

v_Expires := "1600";

v_TimerVal := 1000.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

}

case (423) {

// @siclog "Step 23" siclog@

v_Expires := "800000";

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires))));

// @siclog "Step 24" siclog@

v_IMS_REGISTER_REQ :=  f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires);

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 25" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

// @siclog "Step 26 - 27" siclog@

v_Expires := "60";

v_TimerVal := 30.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader);

v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId);

v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) };

f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList));

}

case (401) {

// @siclog "Step 29" siclog@

f_IMS_Register_SecurityInit(v_RegisterReq, not4b tsc_IMS_AuthRAND);

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq))));

// @siclog "Step 30" siclog@

v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest();               // nonce-count starts from 1 again

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 31" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

}

f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling);

}

After change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_NonceCount,

integer p_StatusCode,

template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var REGISTER_Request v_RegisterReq;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var charstring v_Expires, v_ExplicitExpiry;

var float v_TimerVal;

var SipUrl v_ContactUrl;

var template (value) RegInfo_RegistrationList_Type v_RegistrationList;

var charstring v_PublicUserId;

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

if (p_StatusCode == 401) {

v_ExplicitExpiry := "800000"

}

else {

v_ExplicitExpiry := omit

};

v_IMS_REGISTER_REQ := f_IMS_PTC_ReRegistration_WaitForRegistration(p_TRegExpireMin, p_TRegExpireMax, p_NonceCount, v_ExplicitExpiry);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

if (isvalue(p_PrevSecurityClientParams)) {

f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams));

}

select (p_StatusCode) {

case (500) {

// @siclog "Step 11" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader))));

}

case (200) {

// @siclog "Step 21" siclog@

v_Expires := "1600";

v_TimerVal := 1000.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

}

case (423) {

// @siclog "Step 23" siclog@

v_Expires := "800000";

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires))));

// @siclog "Step 24" siclog@

v_IMS_REGISTER_REQ :=  f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires);

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 25" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

// @siclog "Step 26 - 27" siclog@

v_Expires := "60";

v_TimerVal := 30.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader);

v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId);

v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) };

f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList));

}

case (401) {

// @siclog "Step 29" siclog@

f_IMS_Register_SecurityInit(v_RegisterReq, not4b tsc_IMS_AuthRAND);

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq))));

// @siclog "Step 30" siclog@

//

v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-,-,-,-,v_ExplicitExpiry);               // nonce-count starts from 1 again

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 31" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

}

f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling);

}

## 2.4 Change 4

| Function name | f_IMS_PTC_ReRegistration_WaitForRegistration |
| --- | --- |
| Reason for change | Needed for Change 3.2 and 3.3 |
| Summary of change | Extended declaration of f_IMS_PTC_ReRegistration_WaitForRegistration |
| TTCN module | IMS_Procedures_Registration.ttcn |
| MCC160 Comment |  |

Before change:

function f_IMS_PTC_ReRegistration_WaitForRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_Nc)

runs on IMS_PTC return IMS_DATA_REQ

{ /* @sic R5s130266 change 1.2 - MCC160 Implementation: upper, lower bound for TRegExpire sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef;

var template (present) SipUrl v_SipUrl := cr_SipUri_HostPort(f_IMS_PTC_ImsInfo_GetHomeDomainName());

alt { // @sic R5s130266 change 1.2 - MCC160 Implementation sic@

[] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef)

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "re-registration before timeout of expiry timer");

}

[] p_TRegExpireMin.timeout {}

}

alt {

[] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request(v_SipUrl)), v_IMS_DATA_REQ_ByRef)   // receive any REGISTER

{

p_TRegExpireMax.stop;

f_IMS_REGISTER_SubsequentRequest(v_IMS_DATA_REQ_ByRef, -, -, p_Nc);

}

[] p_TRegExpireMax.timeout

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "timeout while waiting for re-registration");

}

}

return v_IMS_DATA_REQ_ByRef;

}

After change:

function f_IMS_PTC_ReRegistration_WaitForRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_Nc,

template (omit) charstring p_ExplicitExpiry := omit)

runs on IMS_PTC return IMS_DATA_REQ

{ /* @sic R5s130266 change 1.2 - MCC160 Implementation: upper, lower bound for TRegExpire sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef;

var template (present) SipUrl v_SipUrl := cr_SipUri_HostPort(f_IMS_PTC_ImsInfo_GetHomeDomainName());

alt { // @sic R5s130266 change 1.2 - MCC160 Implementation sic@

[] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request), v_IMS_DATA_REQ_ByRef)

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "re-registration before timeout of expiry timer");

}

[] p_TRegExpireMin.timeout {}

}

alt {

[] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request(v_SipUrl)), v_IMS_DATA_REQ_ByRef)   // receive any REGISTER

{

p_TRegExpireMax.stop;

f_IMS_REGISTER_SubsequentRequest(v_IMS_DATA_REQ_ByRef, -, -, p_Nc, p_ExplicitExpiry);

}

[] p_TRegExpireMax.timeout

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "timeout while waiting for re-registration");

}

}

return v_IMS_DATA_REQ_ByRef;

}

## Additional changes from RnS:

## 2.5 Change 5

| Function name | f_TC_6_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | 1. After step 10, f_IMS_GlobalRelease() function call shall release the IMS registration and resetting of IMS information which is not required because when UE sends the step 12 Initial Registration to TTCN than ttcn shall not be handled the drb data at IMS PTC. 2.In f_IMS_CC_Postamble() p_TestProcedure should not be pass as a parameter because f_IMS_CC_ReleaseConnection() already release the IPCAN_MO_IMS_Signalling conncetion in previous function fl_TC_6_3_InitialRegistration |
| Summary of change | 1..Removed the f_IMS_GlobalRelease() call - Omit the TestProcedure in f_IMS_CC_Postamble() |
| TTCN module | NR5GC_6\IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC

{

var SecurityClientParams_Type v_SecurityClientParams;

var charstring v_Expires;

var float v_TimerVal;

var integer v_NonceCount;

var integer v_StatusCode;

timer t_TRegExpireMin;

timer t_TRegExpireMax;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

v_Expires := "120";

v_TimerVal := 60.0;

fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

// @siclog "Step 10 - 11" siclog@

v_NonceCount := 2;

v_StatusCode := 500;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

f_IMS_GlobalRelease();

…...

// @siclog "Step 22 - 27" siclog@

v_NonceCount := 3;

v_StatusCode := 423;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

// @siclog "Step 28 - 31" siclog@

v_NonceCount := 5;

v_StatusCode := 401;

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling);

}

After change:

function f_TC_6_3_NR5GC_IMS1() runs on IMS_PTC

{

var SecurityClientParams_Type v_SecurityClientParams;

var charstring v_Expires;

var float v_TimerVal;

var integer v_NonceCount;

var integer v_StatusCode;

timer t_TRegExpireMin;

timer t_TRegExpireMax;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

v_Expires := "120";

v_TimerVal := 60.0;

fl_TC_6_3_InitialRegistration(IPCAN_InitialRegistration, t_TRegExpireMin, t_TRegExpireMax, v_Expires, v_TimerVal);

// @siclog "Step 10 - 11" siclog@

v_NonceCount := 2;

v_StatusCode := 500;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// REMOVED : f_IMS_GlobalRelease();

…...

// @siclog "Step 22 - 27" siclog@

v_NonceCount := 3;

v_StatusCode := 423;

v_SecurityClientParams := f_IMS_PTC_ReRegistration_SecurityClientParamsGet();

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode, v_SecurityClientParams);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

// @siclog "Step 28 - 31" siclog@

v_NonceCount := 5;

v_StatusCode := 401;

fl_TC_6_3_ReRegistration(t_TRegExpireMin, t_TRegExpireMax, v_NonceCount, v_StatusCode);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 28");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble( );

}

## 2.6 Change 6

| Function name | fl_TC_6_3_ReRegistration |
| --- | --- |
| Reason for change | Previous security should be released before generating a new RAND while initialise security during registration procedure becaue TTCN does not support more than two security contexts at the same time |
| Summary of change | Release the previous security |
| TTCN module |  |
| MCC160 Comment |  |

Before change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_NonceCount,

integer p_StatusCode,

template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var REGISTER_Request v_RegisterReq;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var charstring v_Expires;

var float v_TimerVal;

var SipUrl v_ContactUrl;

var template (value) RegInfo_RegistrationList_Type v_RegistrationList;

var charstring v_PublicUserId;

….

if (isvalue(p_PrevSecurityClientParams)) {

f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams));

}

select (p_StatusCode) {

case (500) {

// @siclog "Step 11" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader))));

}

case (200) {

// @siclog "Step 21" siclog@

v_Expires := "1600";

v_TimerVal := 1000.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

}

case (423) {

// @siclog "Step 23" siclog@

v_Expires := "800000";

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires))));

// @siclog "Step 24" siclog@

v_IMS_REGISTER_REQ :=  f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires);

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 25" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

// @siclog "Step 26 - 27" siclog@

v_Expires := "60";

v_TimerVal := 30.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader);

v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId);

v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) };

f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList));

}

case (401) {

….

// @siclog "Step 31" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

}

f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling);

}

After change:

function fl_TC_6_3_ReRegistration(timer p_TRegExpireMin,

timer p_TRegExpireMax,

integer p_NonceCount,

integer p_StatusCode,

template (omit) SecurityClientParams_Type p_PrevSecurityClientParams := omit) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var REGISTER_Request v_RegisterReq;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var charstring v_Expires, v_ExplicitExpiry;

var float v_TimerVal;

var SipUrl v_ContactUrl;

var template (value) RegInfo_RegistrationList_Type v_RegistrationList;

var charstring v_PublicUserId;

…..

if (isvalue(p_PrevSecurityClientParams)) {

f_IMS_PTC_ReRegistration_SecurityClientParamsCheck(v_RegisterReq, valueof(p_PrevSecurityClientParams));

}

select (p_StatusCode) {

case (500) {

// @siclog "Step 11" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine500, f_IMS_CommonResponse_500_MessageHeaderTX(v_RegisterReq.msgHeader))));

}

case (200) {

// @siclog "Step 21" siclog@

v_Expires := "1600";

v_TimerVal := 1000.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

}

case (423) {

// @siclog "Step 23" siclog@

v_Expires := "800000";

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_Expires))));

// @siclog "Step 24" siclog@

v_IMS_REGISTER_REQ :=  f_IMS_REGISTER_SubsequentRequest(-, -, -, p_NonceCount+1, v_Expires);

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

// @siclog "Step 25" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_Expires))));

// @siclog "Step 26 - 27" siclog@

v_Expires := "60";

v_TimerVal := 30.0;

p_TRegExpireMin.start(f_IMS_SetTimerToleranceMin(v_TimerVal));

p_TRegExpireMax.start(f_IMS_SetTimerToleranceMax(v_TimerVal));

v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_RegisterReq.msgHeader);

v_PublicUserId := f_IMS_PTC_ImsInfo_GetUriString(firstId);

v_RegistrationList := { f_XML_RegistrationInfo(v_RegisterReq, v_PublicUserId, "a100", "980", -, shortened, str2int(v_Expires)) };

f_IMS_Register_Notify(v_ContactUrl, f_XML_RegInfo(partial, 1, v_RegistrationList));

}

case (401) {

f_IMS_Register_SecurityRelease(); //releasing the old security context

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RoutingInfoDL.Security := protectedContext1;

……

// @siclog "Step 31" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

}

f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling);

}

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR, Rohde & Schwarz 5G Protocol Conformance Test platform, and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run using band n41[1]:

- Test case execution log file:

Anritsu/TC_6_3_LOG.xml

RohdenSchwarz/ttcn_tli.log

Keysight/ Keysight\Qualcomm_n41/ TC_6_3_20201109_050503_n41_PASS_v64.0.html

- PICS/PIXIT parameter file:

Anritsu/TC_6_3_PIXIT.txt

RohdenSchwarz/ttcn_tli.log

Keysight/ Keysight\Qualcomm_n41/ TC_6_3_20201109_050503_n41_PASS_v64.0.xml

## •	Samsung NeusERD S5188

The Samsung SM-G986U UE passed this test case on Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run using band n78[1]:

- Test case execution log file:

Keysight\Samsung_n78\ TC_6_3_20201110_175700_n78_v64.0_PASS.html

- PICS/PIXIT parameter file:

Keysight\Samsung_n78\ TC_6_3_20201110_175700_n78_v64.0_PASS.xml

# References

| [1] | R5s201612: Supporting information for agreement of TC 6.3 |
| --- | --- |