3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210322

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0674 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.1 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

4.1	Qualcomm SM8350 + SDX60	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.1

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes of R5s210074, R5s210075 and changes 5-7 of  R5s210330 are required and applied.

## 2.1 Change 1

| Function name | f_TC_7_1_NR5GC_IMS1() |
| --- | --- |
| Reason for change | RetryAfter time is modified by agreed prose CR R5-210882 |
| Summary of change | Modify according to R5-210882 |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_NR5GC_IMS1() runs on IMS_PTC { var template (present) SipUrl v_CalleeUri := f_IMS_INVITE_CalleeUriRX(); // as used per default in the INVITE and in this test case for the ACK as the UE has not gotten any callee's contact URI var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader503; var template (present) MessageHeader v_MessageHeaderACK; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var integer v_RetryAfter := 60; var float v_RetryAfterMin := f_IMS_SetTimerToleranceMin(int2float(v_RetryAfter)); f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(v_CalleeUri); v_InviteRequest := v_InviteRequestWithSdp.Invite; // @siclog "Step 9" siclog@ 503 Service Unavailable v_MessageHeader503 := f_IMS_CommonResponse_503_MessageHeaderTX(v_InviteRequest.msgHeader, cs_RetryAfter(v_RetryAfter)); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine503, v_MessageHeader503))); // @siclog "Step 10" siclog@ ACK v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine503); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeUri, v_MessageHeaderACK))); f_IMS_PTC_ImsInfo_DialogRelease(); // @siclog "Step 11 - 13" siclog@ Check: Does the UE (not) transmit INVITE request message f_Delay(v_RetryAfterMin); // @siclog "Step 14" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(v_CalleeUri); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 15 - 25" siclog@ Steps 2 to 12 of subclause A.4.1 fl_IMS_MOCallSetup_NR5GC_A_4_1_Step2_12(v_InviteRequestWithSdp); f_IMS_TestBody_Set(false); // Call release and postamble: fl_NR5GC_CallControl_Postamble(IPCAN_MO_SpeechCall); }

After Change:

function f_TC_7_1_NR5GC_IMS1() runs on IMS_PTC { var template (present) SipUrl v_CalleeUri := f_IMS_INVITE_CalleeUriRX(); // as used per default in the INVITE and in this test case for the ACK as the UE has not gotten any callee's contact URI var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var template (value) MessageHeader v_MessageHeader503; var template (present) MessageHeader v_MessageHeaderACK; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var integer v_RetryAfter := 20; var float v_RetryAfterMin := f_IMS_SetTimerToleranceMin(int2float(v_RetryAfter)); f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(v_CalleeUri); v_InviteRequest := v_InviteRequestWithSdp.Invite; // @siclog "Step 9" siclog@ 503 Service Unavailable v_MessageHeader503 := f_IMS_CommonResponse_503_MessageHeaderTX(v_InviteRequest.msgHeader, cs_RetryAfter(v_RetryAfter)); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine503, v_MessageHeader503))); // @siclog "Step 10" siclog@ ACK v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine503); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeUri, v_MessageHeaderACK))); f_IMS_PTC_ImsInfo_DialogRelease(); // @siclog "Step 11 - 13" siclog@ Check: Does the UE (not) transmit INVITE request message f_Delay(v_RetryAfterMin); // @siclog "Step 14" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(v_CalleeUri); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 14"); // @siclog "Step 15 - 25" siclog@ Steps 2 to 12 of subclause A.4.1 fl_IMS_MOCallSetup_NR5GC_A_4_1_Step2_12(v_InviteRequestWithSdp); f_IMS_TestBody_Set(false); // Call release and postamble: fl_NR5GC_CallControl_Postamble(IPCAN_MO_SpeechCall); }

# Branches Executed

The test case was executed on NR bands n3 and n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_1_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_1_PIXIT.xml

The Qualcomm SM8350 + SDX60 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_1_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_7_1_PIXIT.txt

# References

| [1] | R5s210323 : Supporting information for addition of IMS 5GS call control test case 7.1 |
| --- | --- |