3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210353

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1628 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR IRAT test case 6.2.3.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC IRAT test case 6.2.3.10 to the 5G/NR5GC_IRAT ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC IRAT test case 6.2.3.10 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	5

4.1	Samsung SM-G996B	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.2.3.10 which is part of the 5G NR5GC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	6.2.3.10

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_6_2_3_10and11_TestBody |
| --- | --- |
| Reason for change | Maximum timer tolerance should be used at step 2 instead of minimum, otherwise a UE which reselects to NR cell 1 within 34s for FR1 or 130s for FR2 may still fail the test case. |
| Summary of change | Used function f_NR_SetTimerToleranceMax(), and rename the timer accordingly. |
| TTCN module | Idle_CellReSelection_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_2_3_10and11_TestBody() runs on NR5GC_PTC { var boolean v_IsFR1; var float v_Timer; timer t_Tmin; v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); //@siclog "Step 1" siclog@ //The SS adjusts the E-UTRAN and NR Cell power levels according to row "T1" in the respective table f_NR_SetCellPower (nr_Cell1, -80, -78); //@siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 1 within 34s for FR1 or 130s for FR2? (Note 1) if (v_IsFR1) { v_Timer := 34.0; } else { v_Timer := 130.0 } t_Tmin.start(f_NR_SetTimerToleranceMin(nr_Cell1, nonProtocolTimer, v_Timer)); alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmin.stop; } [] t_Tmin.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 2"); } } f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }

After Change:

function fl_TC_6_2_3_10and11_TestBody() runs on NR5GC_PTC { var boolean v_IsFR1; var float v_Timer; timer t_Tmax; v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); //@siclog "Step 1" siclog@ //The SS adjusts the E-UTRAN and NR Cell power levels according to row "T1" in the respective table f_NR_SetCellPower (nr_Cell1, -80, -78); //@siclog "Step 2" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.9.2.2-1 indicate that the UE is camped on NR Cell 1 within 34s for FR1 or 130s for FR2? (Note 1) if (v_IsFR1) { v_Timer := 34.0; } else { v_Timer := 130.0 } t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell1, nonProtocolTimer, v_Timer)); alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_Tmax.stop; } [] t_Tmax.timeout { f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Step 2"); } } f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }

# Branches executed

The test case was executed on NR band n78

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_2_3_10_LOG.html

- PICS/PIXIT parameter file:

TC_6_2_3_10_PIXIT.html

# References

| [1] | R5s210354 : NR5GC FR1 : Supporting information for addition of NR5GC IRAT test case 6.2.3.10 |
| --- | --- |