3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210359

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1631 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_NR5GC_IRAT_S1ToN1_SessionModification(), the TTCN determines the expected number of PDU SESSION MODIFICATION REQUESTS based on v_GMM_MobilityInfo.SessionStatus, which was set from PDUSessionStatus IE in REGISTRATION REQUEST. For 9.3.1.2, the UE has no established security context while initiate Registration Request at step 1. So the (non-cleartext) IE pdusessionStatus is signalled in the piggybacked REGISTRATION REQUEST in SECURITY MODE COMPLETE. The current implementation does not update v_GMM_MobilityInfo.SessionStatus after receiving SECURITY MODECOMPLETE, which leads to a run-time error in f_NR5GC_IRAT_S1ToN1_SessionModification(). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extract and save pdusessionStatus after receiving Security Mode Complete |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_3_1_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | In the function f_NR5GC_IRAT_S1ToN1_SessionModification(), the TTCN determines the expected number of PDU SESSION MODIFICATION REQUESTS based on v_GMM_MobilityInfo.SessionStatus, which was set from PDUSessionStatus IE in REGISTRATION REQUEST. For 9.3.1.2, the UE has no established security context while initiate Registration Request at step 1. So the (non-cleartext) IE pdusessionStatus is signalled in the piggybacked REGISTRATION REQUEST in SECURITY MODE COMPLETE. The current implementation does not update v_GMM_MobilityInfo.SessionStatus after receiving SECURITY MODECOMPLETE, which leads to a run-time error in f_NR5GC_IRAT_S1ToN1_SessionModification(). |
| Summary of change | Extract and save pdusessionStatus after receiving Security Mode Complete |
| TTCN module | Mobility_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_Result; var NasCount_Type v_NasCountUL; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); ……….. // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity); v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL); f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def); // @sic R5-205142 sic@ Now finish the procedure v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @sic R5s201005 sic@ return v_SRB; }

After Change

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_Result; var NasCount_Type v_NasCountUL; var template (value) NG_NAS_UL_Message_Type v_NasMsg; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); ……… // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity); v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg)) { v_NasMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; if (ispresent(v_NasMsg.registration_Request.pduSessionStatus)) { v_GMM_MobilityInfo.SessionStatus := v_NasMsg.registration_Request.pduSessionStatus; } else { v_GMM_MobilityInfo.SessionStatus := omit; } } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL); f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def); // @sic R5-205142 sic@ Now finish the procedure v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @sic R5s201005 sic@ return v_SRB; }