3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210383

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1642 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current implementation of function f_TC_10_1_4_1_NR5GCf_NR5GC_Preamblef_NR5GC_RRC_IdleState1N_Deff_NR5GC_RRC_Idle_Steps5_20f_NR5GC_RRC_Idle_Steps16_20f_NR5GC_RRC_Idle_Extensionf_UT_RequestPDUSession is called in preamble which establishes a PDU Session. However, this AT command does not include the parameter for S-NSSAI, and his results in the UE not including a S-NSSAI in the UL NAS Transport message during PDU Session establishment procedure. Later in function fl_TC_10_1_4_1_Body, variable assignment v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); is done. This results in variable v_RequestedNSSAI being set to OMIT. This causes runtime errors when at Step 16 the following assignment I done: v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI); Moreover, the Prose is trying to test that the UE includes the Same DNN and S-NSSAI in the attempts of PDU Session Establishment. Thus it is important that in preamble the PDU Session Establishment takes place with a S-NSSAI. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an additional parameter p_Include_S_NSSAI in following functions : f_NR5GC_Preamble f_NR5GC_RRC_IdleState1N_Def f_NR5GC_RRC_Idle_Steps5_20 f_NR5GC_RRC_Idle_Steps16_20 f_NR5GC_RRC_Idle_Extension Based on this parameter, in function f_NR5GC_RRC_Idle_Extension decided whether the AT comamnd trigger function should include a S-NSSAI or not. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case ends with a runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to f_TC_10_1_4_1_NR5GC	4

1.2	Correction to f_NR5GC_Preamble	6

1.3	Correction to f_NR5GC_RRC_IdleState1N_Def	8

1.4	Correction to f_NR5GC_RRC_Idle_Steps5_20	10

1.5	Correction to f_NR5GC_RRC_Idle_Steps16_20	12

1.6	Correction to f_NR5GC_RRC_Idle_Extension	15

# Corrections required

## Correction to f_TC_10_1_4_1_NR5GC

| Module name | f_TC_10_1_4_1_NR5GC |
| --- | --- |
| Reason for change | In current implementation of function f_TC_10_1_4_1_NR5GCf_NR5GC_Preamblef_NR5GC_RRC_IdleState1N_Deff_NR5GC_RRC_Idle_Steps5_20f_NR5GC_RRC_Idle_Steps16_20f_NR5GC_RRC_Idle_Extensionf_UT_RequestPDUSession is called in preamble which establishes a PDU Session. However, this AT command does not include the parameter for S-NSSAI, and his results in the UE not including a S-NSSAI in the UL NAS Transport message during PDU Session establishment procedure. Later in function fl_TC_10_1_4_1_Body, variable assignment v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); is done. This results in variable v_RequestedNSSAI being set to OMIT. This causes runtime errors when at Step 16 the following assignment I done: v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI); Moreover, the Prose is trying to test that the UE includes the Same DNN and S-NSSAI in the attempts of PDU Session Establishment. Thus it is important that in preamble the PDU Session Establishment takes place with a S-NSSAI. This needs to be addressed. |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in following functions : f_NR5GC_Preamble f_NR5GC_RRC_IdleState1N_Def f_NR5GC_RRC_Idle_Steps5_20 f_NR5GC_RRC_Idle_Steps16_20 f_NR5GC_RRC_Idle_Extension Based on this parameter, in function f_NR5GC_RRC_Idle_Extension decided whether the AT comamnd trigger function should include a S-NSSAI or not. Please see below. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A); if (f_NR5GC_MobileInfo_GetNoOfSessions() == 0){ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, 1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (v_NGC_NASCellA, STATE_CONNECTED_3A, -,-,-,-,-, true /*WA#10_1_4_1*/); if (f_NR5GC_MobileInfo_GetNoOfSessions() == 0){ f_NR5GC_TriggerSinglePDUSession(v_NGC_NASCellA, 1, -, tsc_RandomAPN, cs_S_NSSAI_SST1eMBB); } <<SKIPPED CODE>>

## Correction to f_NR5GC_Preamble

| Module name | f_NR5GC_Preamble |
| --- | --- |
| Reason for change | Please see the change 1.1 above |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in f_NR5GC_Preamble, and passed this value to f_NR5GC_RRC_IdleState1N_Def Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

After Change:

function f_NR5GC_Preamble(NR_CellId_Type p_NR_CellId, NG_STATE_Type p_State, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, boolean p_ActivateSecurityInactiveState := true, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false /*WA#10_1_4_1*/) runs on NR5GC_PTC { //@sic R5s190505: added p_ActivateSecurityInactiveState sic@ //@sic R5-199036: added p_IpPduDelayTime sic@ var RRCConnectionReleaseRequired_Type v_ReleaseConnection := p_ReleaseConnection; // @sic R5-197077 sic@ if (p_State == STATE_INACTIVE_2A) { v_ReleaseConnection := noRrcConnectionRelease; } // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); if (p_State != STATE_OFF_0A ) { // Turn on, Register and establish 1 PDU session // Switch on UE f_NR5GC_SwitchOnAndResetIMS(); // Go to RRC_IDLE f_NR5GC_RRC_IdleState1N_Def (p_NR_CellId, p_TestMode, v_ReleaseConnection, Initial_NoSecurity, p_IpPduDelayTime, p_ServiceType, p_Include_S_NSSAI /*WA#10_1_4_1*/); // @sic R5s210189 sic@ } select (p_State) { case (STATE_OFF_0B) { // Now turn off again f_NR_UE_DeRegisterOnSwitchOff (p_NR_CellId, STATE_IDLE_1A); } case (STATE_INACTIVE_2A) { f_NR_RRC_InactiveDef(p_NR_CellId, -, p_ActivateSecurityInactiveState); } case (STATE_CONNECTED_3A) { // Go to RRC_CONNECTED f_NR5GC_RRC_ConnectedState3N_Def (p_NR_CellId, p_TestMode); } } }

## Correction to f_NR5GC_RRC_IdleState1N_Def

| Module name | f_NR5GC_RRC_IdleState1N_Def |
| --- | --- |
| Reason for change | Please see change 1.1 |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in f_NR5GC_RRC_IdleState1N_Def , and passed this value to f_NR5GC_RRC_Idle_Steps5_20 Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime, -, p_ServiceType); // @sic R5s210189 sic@ }

After Change:

function f_NR5GC_RRC_IdleState1N_Def(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false /*WA#10_1_4_1*/) runs on NR5GC_PTC { //@sic R5-199036: added p_IpPduDelayTime sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, p_RegType)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Steps 5 - 20 f_NR5GC_RRC_Idle_Steps5_20 (v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, p_TestMode, p_ReleaseConnection, p_RegType, -, p_IpPduDelayTime, -, p_ServiceType, p_Include_S_NSSAI /*WA#10_1_4_1*/); // @sic R5s210189 sic@ }

## Correction to f_NR5GC_RRC_Idle_Steps5_20

| Module name | f_NR5GC_RRC_Idle_Steps5_20 |
| --- | --- |
| Reason for change | Please see change 1.1 |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in f_NR5GC_RRC_Idle_Steps5_20, and passed this value to f_NR5GC_RRC_Idle_Steps16_20 Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, // @sic R5s200751 sic@ -, -, -, -, -, -, -, -, -, p_ServiceType); // @sic R5s210189 sic@ }

After Change:

function f_NR5GC_RRC_Idle_Steps5_20(GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, TypeOfRegistration_Type p_RegType := Initial_Secure, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false /*WA#10_1_4_1*/) runs on NR5GC_PTC { var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); // Steps 5-15 f_NR5GC_RRC_Idle_Steps5_15(p_GMM_MobilityInfo_Type, p_NR_CellId, p_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, -, -, p_IwkN26); // @sic R5-202564 sic@ //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId, -, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, // @sic R5s200751 sic@ -, -, -, -, -, -, -, -, -, p_ServiceType, p_Include_S_NSSAI /*WA#10_1_4_1*/); // @sic R5s210189 sic@ }

## Correction to f_NR5GC_RRC_Idle_Steps16_20

| Module name | f_NR5GC_RRC_Idle_Steps16_20 |
| --- | --- |
| Reason for change | Please see change 1.1 |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in f_NR5GC_RRC_Idle_Steps16_20, and passed this value to f_NR5GC_RRC_Idle_Extension Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, -, -, -, -, -, -, -, -, -, p_ServiceType); // @sic R5s210189 sic@ } }

After Change:

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false /*WA#10_1_4_1*/) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 20a1 if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@ // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // @sic R5s200489 sic@ // Step 19Aa1 - 19Aa2 select (p_TestMode) { // @sic R5-194797 sic@ case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@ // @sic R5-199075 sic@ f_NR5GC_RRC_Idle_Extension(p_NR_CellId, p_TestMode, p_ReleaseConnection, p_UE_TestLoopModeA_NR_LB_Setup, p_IpPduDelayTime, -, -, -, -, -, -, -, -, -, p_ServiceType, p_Include_S_NSSAI /*WA#10_1_4_1*/); // @sic R5s210189 sic@ } }

## Correction to f_NR5GC_RRC_Idle_Extension

| Module name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | Please see change 1.1 |
| Summary of change | Added an additional parameter p_Include_S_NSSAI in f_NR5GC_RRC_Idle_Extension, and based on this parameter decided whether the AT comamnd trigger function should include a S-NSSAI or not. Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ var PDU_PDN_DNN_Type v_PDN_Type; var charstring v_DNN := ""; timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } else { // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return; } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, p_ServiceType)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }

After Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B, boolean p_Include_S_NSSAI := false /*WA#10_1_4_1*/) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ var PDU_PDN_DNN_Type v_PDN_Type; var charstring v_DNN := ""; timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } if(p_Include_S_NSSAI){ //WA#10_1_4_1 f_UT_RequestPDUSession(UT, 1, v_DNN, -, -, f_ConvertS_NSSAI_ForATCommand (p_S_NSSAI));//WA#10_1_4_1 }else { f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } } else { // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return; } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, p_ServiceType)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, // @sic R5-206291 sic@ p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }