3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210404

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1652 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR system information modification functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. During calculation of v_TimeOfSysinfoModification in function f_NR_ModifySysinfo(), v_ProcessingDelay is added twice 2. In most cases, calling of f_NR_ModifySysinfo () is followed by f_NR_WaitModificationPeriods() to wait 2.1 modifciation periods to allow the UE to read the updated system information. However the current implementation will start the timer immediately on return from f_NR_ModifySysinfo(), which can be more than 1 modification period before the short message is sent from the SS, which may not leave the UE enough time to read the updated SI. 3. In function f_NR_ModifySysinfo_WaitActivation (), delay time is calculated incorrectly because the paramters in call to f_SubFrameTiming_Duration() are in the wrong order (T1 should be v_Tnow, and T2 should be v_SysInfoActivation) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Remove addition v_ProcessingDelay in calculation of v_StartNextModPeriod_SFN (already incorporated into v_Timing_SFN) 2. Add a delay to ensure short message is sent at approximately the time the function returns (i.e. the time v_TimeOfShortMessage is reached) 3. Reverse the arguments T1 and T2 order when calling f_SubFrameTiming_Duration() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail some NR5GC test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	8

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_ModifySysinfo() |
| --- | --- |
| Reason for change | - During calculation of v_TimeOfSysinfoModification, v_ProcessingDelay is added twice - In most cases, calling of f_NR_ModifySysinfo () is followed by f_NR_WaitModificationPeriods() to wait 2.1 modifciation periods to allow the UE to read the updated system information. However the current implementation will start the timer immediately on return from f_NR_ModifySysinfo(), which can be more than 1 modification period before the short message is sent from the SS, which may not leave the UE enough time to read the updated SI. |
| Summary of change | - Remove addition v_ProcessingDelay in calculation of v_StartNextModPeriod_SFN (already incorporated into v_Timing_SFN) - Add a delay to ensure short message is sent at approximately the time the function returns (i.e. the time v_TimeOfShortMessage is reached) |
| TTCN module | NR5GC_SysinfoPaging |
| MCC160 Comment |  |

Before Change

function f_NR_ModifySysinfo(NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE, boolean p_BroadcastAllSI := false ) runs on NR5GC_PTC return SubFrameTiming_Type { //Note: in INACTIVE state UE may have a different DRX cycle - that is not needed so far var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_TimeOfShortMessage; var SubFrameTiming_Type v_TimeOfSysinfoModification; var integer v_Timing_SFN; var SubFrameTiming_Type v_StartNextModPeriod; var integer v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (p_NR_CellId); var integer v_ProcessingDelay := tsc_NR_SfnDelay; //For IDLE and INACTIVE state var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId,p_BroadcastAllSI); //@sic R5s200703 sic@ var integer v_StartNextModPeriod_SFN; //When UE is in connected mode, short message will be sent during a complete modification period //Add a longer processing delay to current SFN if (p_UE_State == RRC_CONNECTED and not(pc_ETWS_NR or pc_CMAS_NR)) {v_ProcessingDelay := tsc_NR_SfnDelayConnected} //Get the current SFN and subframe v_Timing := f_NR_GetCurrentTiming(p_NR_CellId); //Add processing delay to current SFN v_Timing_SFN := v_Timing.SFN.Number + v_ProcessingDelay; // Calculate the SFN of the next Modification Period //@sic R5-211417 sic@ v_StartNextModPeriod_SFN := v_Timing_SFN + v_ProcessingDelay + (v_ModificationPeriodFrames -(v_Timing_SFN mod v_ModificationPeriodFrames)); v_StartNextModPeriod.HSFN.Number := (v_Timing.HSFN.Number + (v_StartNextModPeriod_SFN/1024) ) mod 1024; v_StartNextModPeriod.SFN.Number := v_StartNextModPeriod_SFN mod 1024; //Set Subframe and slot to 0, i.e. start of the modification period v_StartNextModPeriod.Subframe.Number := 0; v_StartNextModPeriod := f_NR_SubFrameTiming_SetSlot(v_StartNextModPeriod, 0, v_SCS); if (not p_ShortMessageFlag) { //Short Msg Indication not to be sent v_TimeOfSysinfoModification := v_StartNextModPeriod; } else { //@sic R5-211417 sic@ v_TimeOfShortMessage := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; v_TimeOfSysinfoModification := v_StartNextModPeriod; f_NR_ShortMessageIndication_SysinfoMod(p_NR_CellId, v_TimeOfShortMessage, p_UE_State); } f_NR_SS_ConfigureSysinfo (p_NR_CellId, cs_TimingInfo_SFNOnly(v_TimeOfSysinfoModification.SFN.Number), v_NR_BcchToPdschConfig); return v_TimeOfSysinfoModification; }

After Change

function f_NR_ModifySysinfo(NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE, boolean p_BroadcastAllSI := false ) runs on NR5GC_PTC return SubFrameTiming_Type { //Note: in INACTIVE state UE may have a different DRX cycle - that is not needed so far var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_TimeOfShortMessage; var SubFrameTiming_Type v_TimeOfSysinfoModification; var integer v_Timing_SFN; var SubFrameTiming_Type v_StartNextModPeriod; var integer v_ModificationPeriodFrames := f_NR_CalculateModificationPeriod (p_NR_CellId); var integer v_ProcessingDelay := tsc_NR_SfnDelay; //For IDLE and INACTIVE state var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var template(omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId,p_BroadcastAllSI); //@sic R5s200703 sic@ var integer v_StartNextModPeriod_SFN; //When UE is in connected mode, short message will be sent during a complete modification period //Add a longer processing delay to current SFN if (p_UE_State == RRC_CONNECTED and not(pc_ETWS_NR or pc_CMAS_NR)) {v_ProcessingDelay := tsc_NR_SfnDelayConnected} //Get the current SFN and subframe v_Timing := f_NR_GetCurrentTiming(p_NR_CellId); //Add processing delay to current SFN v_Timing_SFN := v_Timing.SFN.Number + v_ProcessingDelay; // Calculate the SFN of the next Modification Period //@sic R5-211417 sic@ v_StartNextModPeriod_SFN := v_Timing_SFN + (v_ModificationPeriodFrames -(v_Timing_SFN mod v_ModificationPeriodFrames)); v_StartNextModPeriod.HSFN.Number := (v_Timing.HSFN.Number + (v_StartNextModPeriod_SFN/1024) ) mod 1024; v_StartNextModPeriod.SFN.Number := v_StartNextModPeriod_SFN mod 1024; //Set Subframe and slot to 0, i.e. start of the modification period v_StartNextModPeriod.Subframe.Number := 0; v_StartNextModPeriod := f_NR_SubFrameTiming_SetSlot(v_StartNextModPeriod, 0, v_SCS); if (not p_ShortMessageFlag) { //Short Msg Indication not to be sent v_TimeOfSysinfoModification := v_StartNextModPeriod; } else { //@sic R5-211417 sic@ v_TimeOfShortMessage := v_StartNextModPeriod; v_StartNextModPeriod.HSFN.Number := (v_StartNextModPeriod.HSFN.Number + (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames)/1024) mod 1024; v_StartNextModPeriod.SFN.Number := (v_StartNextModPeriod.SFN.Number + v_ModificationPeriodFrames) mod 1024; v_TimeOfSysinfoModification := v_StartNextModPeriod; f_NR_ShortMessageIndication_SysinfoMod(p_NR_CellId, v_TimeOfShortMessage, p_UE_State); } f_NR_SS_ConfigureSysinfo (p_NR_CellId, cs_TimingInfo_SFNOnly(v_TimeOfSysinfoModification.SFN.Number), v_NR_BcchToPdschConfig); if (p_ShortMessageFlag){ // wait until the short message is to be sent f_Delay(int2float(f_SubFrameTiming_Duration(v_Timing, v_TimeOfShortMessage) - 100 )/ 1000.0); } return v_TimeOfSysinfoModification; }

## Change 2

| Function name | f_NR_ModifySysinfo_WaitActivation () |
| --- | --- |
| Reason for change | Delay time is calculated incorrectly because the paramters in call to f_SubFrameTiming_Duration() are in the wrong order (T1 should be v_Tnow, and T2 should be v_SysInfoActivation) |
| Summary of change | Reverse the arguments T1 and T2 order when calling f_SubFrameTiming_Duration() |
| TTCN module | NR5GC_SysinfoPaging |
| MCC160 Comment |  |

Before Change

function f_NR_ModifySysinfo_WaitActivation( NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE ) runs on NR5GC_PTC { var SubFrameTiming_Type v_SysinfoActivation; var SubFrameTiming_Type v_Tnow; // Modify Sysinfo and wait until it is activated v_SysinfoActivation := f_NR_ModifySysinfo(p_NR_CellId, p_ShortMessageFlag, p_UE_State); v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); //Wait for 1 second after the start of the modification period in which the modified system information is broadcasted to ensure UE has received Sysinfo f_Delay(int2float(f_SubFrameTiming_Duration(v_SysinfoActivation, v_Tnow) + 1000) / 1000.0); // KS wk12 CRxxxx //@sic R5s200678 channge 2 sic@ }

After Change

function f_NR_ModifySysinfo_WaitActivation( NR_CellId_Type p_NR_CellId, boolean p_ShortMessageFlag := true, NR_RRC_STATE_Type p_UE_State := RRC_IDLE ) runs on NR5GC_PTC { var SubFrameTiming_Type v_SysinfoActivation; var SubFrameTiming_Type v_Tnow; // Modify Sysinfo and wait until it is activated v_SysinfoActivation := f_NR_ModifySysinfo(p_NR_CellId, p_ShortMessageFlag, p_UE_State); v_Tnow := f_NR_GetCurrentTiming(p_NR_CellId); //Wait for 1 second after the start of the modification period in which the modified system information is broadcasted to ensure UE has received Sysinfo f_Delay(int2float(f_SubFrameTiming_Duration(v_Tnow, v_SysinfoActivation) + 1000) / 1000.0); // KS wk12 CRxxxx //@sic R5s200678 channge 2 sic@ }