3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210408

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1656 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC IRAT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Function f_EUTRA38_GetNextDRBIds(), in the case of f_EUTRA38_MobileInfo_GetNoOfDRBs() returning 0, a run-time error can happen at the statement “ while (i < lengthof(v_ExistingDRBs)” because v_ExistingDRBs is not initialised - Function f_Check_NG_PDUSessionModificationReq (), the element p_GSM_MobilityInfo.NSSAI is not initialised, this can lead to errors later in the tets case due to “isvalue” check on this structure failing. - Function f_NR5GC_IRAT_S1ToN1_SessionModification(), v_GSM_MobilityInfo as returned from f_Check_NG_PDUSessionModificationReq() will not have EPS bearer or PDN index fields populated. This can lead to errors on the next N1->S1 transition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Initialise v_ExistingDRBs to empty list - Initialise p_GSM_MobilityInfo.NSSAI in f_Check_NG_PDUSessionModificationReq() - Populate EPS_Bearer and PdnIndex from p_PduSessionsFromLTE before storing PDU session info. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail some NR5GC IRAT test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

2.3	Change 3	10

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_GetNextDRBIds() |
| --- | --- |
| Reason for change | In the case of f_EUTRA38_MobileInfo_GetNoOfDRBs() returning 0, a run-time error can happen at the statement “ while (i < lengthof(v_ExistingDRBs)” because v_ExistingDRBs is not initialised |
| Summary of change | Initialise v_ExistingDRBs to empty list |
| TTCN module | EUTRA38_NR5GC_InitialRegistration |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_GetNextDRBIds (PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB, integer p_NoOfDRBs := 1) runs on EUTRA_Multi5GS_PTC return IntegerList_Type { var integer v_NoOfExistingDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var integer v_DRBId := 0; var IntegerList_Type v_DRBList; var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var integer v_Index; var boolean v_AlreadyIMS_DRB := false; var integer i := 0; var integer j := 0; if (v_NoOfExistingDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, tsc_EUTRA38_IMS_DRB); if (v_Index >= 0) { // IMS is already allocated v_AlreadyIMS_DRB := true; } } if ((p_PDN_Type == IMS_DNN) and p_DefaultDRB) { v_DRBList[j] := tsc_EUTRA38_IMS_DRB; j := j+1; } while (j < p_NoOfDRBs) { if (v_AlreadyIMS_DRB) { v_DRBId := v_NoOfExistingDRBs + (2-1); // allocate the next one, from 2 onwards } else { // IMS is not yet allocated v_DRBId := v_NoOfExistingDRBs + 2; } i := 0; while (i < lengthof(v_ExistingDRBs)) { // @sic R5s210305 sic@ // Already allocated this DRB (due to bearers transferred from NR, so increment and try again ) if (match (v_ExistingDRBs[i].DRB_Id, v_DRBId)) { v_DRBId := v_DRBId + 1; i := -1; // as it's about to be incremented } i := i + 1; } v_DRBList[j] := v_DRBId; j := j+1; // now update number of DRBs whilst in the loop v_NoOfExistingDRBs := v_NoOfExistingDRBs +1; } return v_DRBList; }

After Change

function f_EUTRA38_GetNextDRBIds (PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB, integer p_NoOfDRBs := 1) runs on EUTRA_Multi5GS_PTC return IntegerList_Type { var integer v_NoOfExistingDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var integer v_DRBId := 0; var IntegerList_Type v_DRBList; var EUTRA38_DRBInfoList_Type v_ExistingDRBs :={}; var integer v_Index; var boolean v_AlreadyIMS_DRB := false; var integer i := 0; var integer j := 0; if (v_NoOfExistingDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, tsc_EUTRA38_IMS_DRB); if (v_Index >= 0) { // IMS is already allocated v_AlreadyIMS_DRB := true; } } if ((p_PDN_Type == IMS_DNN) and p_DefaultDRB) { v_DRBList[j] := tsc_EUTRA38_IMS_DRB; j := j+1; } while (j < p_NoOfDRBs) { if (v_AlreadyIMS_DRB) { v_DRBId := v_NoOfExistingDRBs + (2-1); // allocate the next one, from 2 onwards } else { // IMS is not yet allocated v_DRBId := v_NoOfExistingDRBs + 2; } i := 0; while (i < lengthof(v_ExistingDRBs)) { // @sic R5s210305 sic@ // Already allocated this DRB (due to bearers transferred from NR, so increment and try again ) if (match (v_ExistingDRBs[i].DRB_Id, v_DRBId)) { v_DRBId := v_DRBId + 1; i := -1; // as it's about to be incremented } i := i + 1; } v_DRBList[j] := v_DRBId; j := j+1; // now update number of DRBs whilst in the loop v_NoOfExistingDRBs := v_NoOfExistingDRBs +1; } return v_DRBList; }

## Change 2

| Function name | f_Check_NG_PDUSessionModificationReq() |
| --- | --- |
| Reason for change | The element p_GSM_MobilityInfo.NSSAI is not initialised, this can lead to errors later in the tets case due to “isvalue” check on this structure failing. |
| Summary of change | Initialise p_GSM_MobilityInfo.NSSAI in f_Check_NG_PDUSessionModificationReq() |
| TTCN module | NG_NASTemplateFunctions |
| MCC160 Comment |  |

Before Change

function f_Check_NG_PDUSessionModificationReq( out GSM_MobilityInfo_Type p_GSM_MobilityInfo, PDUSessionInfoList_Type p_PDUSessionGlobalInfo, NG_NAS_UL_Pdu_Type p_ReceivedMsgs, template (present) O1_Type p_PDU_SessionId := ?, template (present) ProcedureTransactionIdentifier p_PTI := ?, template NG_UE_SM_Cap p_NG_UE_SM_Cap := *, template GMM_GSM_Cause p_Cause := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := *, template QoS_Rules p_QoS_Rules := *, template QoSFlowDescr p_QoSFlowDescr := *, template MappedEPSBearerContexts p_MappedEPSBearerContexts := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := *, template S_NSSAI_Type p_S_NSSAI := *, template DNN p_DNN := *, template AdditionalInformation p_AdditionalInfo := *, template MA_PDUSessionInfo p_MA_PDUSessionInfo := *, // @sic R5s201387 Baseline Moving sic@ template ReleaseAssistanceInd p_ReleaseAssistanceInd := *, template PortManagementInfoContainer p_PortManagementInfoContainer := *, template IPHeaderCompressionConfig p_IPHeaderCompConfig := *, template EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := *) return boolean { …………… // Store info for this modification p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Modification_Request.pduSessionId.sessionId; // @sic R5s200564 sic@ p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Modification_Request.procedureTransactionIdentifier; p_GSM_MobilityInfo.SessionType := omit; p_GSM_MobilityInfo.EPS_Bearer := omit; // @sic R5s201533 sic@ p_GSM_MobilityInfo.PdnIndex := omit; // @sic R5s201533 sic@ if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) { p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn; } else { p_GSM_MobilityInfo.DNN := omit; } p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN); // @sic R5-206296 sic@ if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.alwaysOnPDUSessionReq)) { p_GSM_MobilityInfo.AlwaysOn := v_GSMMsg.pdu_Session_Modification_Request.alwaysOnPDUSessionReq; } else { p_GSM_MobilityInfo.AlwaysOn := omit; } if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions)) { p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; } else { p_GSM_MobilityInfo.PCO := omit; } return v_Result; }

After Change

function f_Check_NG_PDUSessionModificationReq( out GSM_MobilityInfo_Type p_GSM_MobilityInfo, PDUSessionInfoList_Type p_PDUSessionGlobalInfo, NG_NAS_UL_Pdu_Type p_ReceivedMsgs, template (present) O1_Type p_PDU_SessionId := ?, template (present) ProcedureTransactionIdentifier p_PTI := ?, template NG_UE_SM_Cap p_NG_UE_SM_Cap := *, template GMM_GSM_Cause p_Cause := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := *, template QoS_Rules p_QoS_Rules := *, template QoSFlowDescr p_QoSFlowDescr := *, template MappedEPSBearerContexts p_MappedEPSBearerContexts := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := *, template S_NSSAI_Type p_S_NSSAI := *, template DNN p_DNN := *, template AdditionalInformation p_AdditionalInfo := *, template MA_PDUSessionInfo p_MA_PDUSessionInfo := *, // @sic R5s201387 Baseline Moving sic@ template ReleaseAssistanceInd p_ReleaseAssistanceInd := *, template PortManagementInfoContainer p_PortManagementInfoContainer := *, template IPHeaderCompressionConfig p_IPHeaderCompConfig := *, template EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := *) return boolean { ………….. // Store info for this modification p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Modification_Request.pduSessionId.sessionId; // @sic R5s200564 sic@ p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Modification_Request.procedureTransactionIdentifier; p_GSM_MobilityInfo.SessionType := omit; p_GSM_MobilityInfo.EPS_Bearer := omit; // @sic R5s201533 sic@ p_GSM_MobilityInfo.PdnIndex := omit; // @sic R5s201533 sic@ p_GSM_MobilityInfo.NSSAI := omit; if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) { p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn; } else { p_GSM_MobilityInfo.DNN := omit; } p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN); // @sic R5-206296 sic@ if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.alwaysOnPDUSessionReq)) { p_GSM_MobilityInfo.AlwaysOn := v_GSMMsg.pdu_Session_Modification_Request.alwaysOnPDUSessionReq; } else { p_GSM_MobilityInfo.AlwaysOn := omit; } if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions)) { p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; } else { p_GSM_MobilityInfo.PCO := omit; } return v_Result; }

## Change 3

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification() |
| --- | --- |
| Reason for change | v_GSM_MobilityInfo as returned from f_Check_NG_PDUSessionModificationReq() will not have EPS bearer or PDN index fields populated. This can lead to errors on the next N1->S1 transition. |
| Summary of change | Populate EPS_Bearer and PdnIndex from p_PduSessionsFromLTE before storijg PDU session info. |
| TTCN module | NR5GC_IRAT_CommonFunctions |
| MCC160 Comment |  |

Before Change

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) { // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST"); } // ok, this PDU is to be modified, so can now store it in our list v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@ if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) { v_PDU_SessionIdList[K] := v_PDU_SessionId; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already"); } f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); // @sic R5s201413 sic@ ……. }

After Change

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_NR_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) { // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST"); } // ok, this PDU is to be modified, so can now store it in our list v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@ if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) { v_PDU_SessionIdList[K] := v_PDU_SessionId; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already"); } for (i := 0; i < lengthof(p_PduSessionsFromLTE); i := i + 1) { if (p_PduSessionsFromLTE[i].PDU_SessionId == v_PDU_SessionId) { v_GSM_MobilityInfo.EPS_Bearer := valueof(p_PduSessionsFromLTE[i].EPS_Bearer); v_GSM_MobilityInfo.PdnIndex := valueof(p_PduSessionsFromLTE[i].PdnIndex); break; } } f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); // @sic R5s201413 sic@ ….. }