3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210419

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0683 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR TC 7.6 MTSI MT Voice Call with preconditions at both originating UE and terminating UE / 5GS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 7.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	10

2.5	Change 5	11

2.6	Change 6	13

2.7	Change 7	15

3	Branches Executed	16

4	Execution Log Files	16

4.1	Qualcomm SM8350 + SDX60	16

5	References	17

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.6 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun, Harry

shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.6

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

In addition to the below changes, TTCN CR R5s210074 is required and applied.

## Change 1

| Function name | f_IMS_BuildSDP_UpdatedOfferAudio_TX |
| --- | --- |
| Reason for change | R5-210056 updates the FMTP attribute in SIP UPDATE. |
| Summary of change | FMTP attribute changed in SIP UPDATE |
| TTCN module | IMS_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_BuildSDP_UpdatedOfferAudio_TX(SDP_Message p_SDP_Message183, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s141244: use of f_IMS_BuildSDP_TX sic@ */ /* @sic R5-153610: "sendrecv" media attribute removed from SDP message for C.11 step 7 sic@ */ /* @sic R5-153798: p_SDP_Message183 replacing p_SDP_Attribute_curr_qos_remote and p_Step; p_IsC45 sic@ */ /* @sic R5-153798, R5s150841: split of f_IMS_BuildSDP_AnnexC11_Step1or7 sic@ */ /* @sic R5-168057, R5-168142: p_AudioCodec := AMR_16000 sic@ */ var charstring v_Fmt := f_SDP_AudioCodec_GetFmt(p_AudioCodec); /* @sic R5-197050: fix mapping of fmt values sic@ */ var template (value) SDP_bandwidth_list v_MediaBandwidth_List; var template (omit) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes; var template (value) SemicolonParam_List v_EVS_FmtpParams; var charstring v_DirectionRemote; var charstring v_StrengthRemote := c_mandatory; var template (omit) SDP_attribute v_FmtpAttributeEVS; var charstring v_ReceivedBrVal; var charstring v_ReceivedBwVal; select (p_AudioCodec) { case (AMR_8000) { v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); v_SDP_MediaAttributes := f_SDP_MediaAttributes_AudioDef(v_Fmt); } case (AMR_16000) { /* @sic R5-168057 => C.11 sic@ */ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); /* @sic R5-130751; R5-134270: RR=2500; R5-144700: AS=37; R5-172126: RR=2000 sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AMR_WB_AudioDef(v_Fmt); /* @sic R5-144693: new function f_SDP_MediaAttributes_AudioDef; R5-168057: change for AMR_16000 sic@ */ } case (EVS_16000) { /* => C.45 @sic R5-153798 sic@*/ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(65, 0, 2000); if (f_IMS_PTC_RanTypeIsNR5GC()) { /* 34.229-5 A.5.1/2 UPDATE @sic R5-206382: implementation of TC_7_6 sic@ */ v_EVS_FmtpParams := { cs_AMR_ModeChangeCapability, cs_AMR_MaxRed }; /* !!!! PROSE ISSUE: looks weird !!!! */ } else { v_FmtpAttributeEVS := f_SDP_AttributeList_GetAttribute(p_SDP_Message183.media_list[0].attributes, cr_SDP_Attribute_fmtp(v_Fmt)); v_ReceivedBrVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeEVS.fmtp.params.paramList, "br"); /* @sic R5-195318: "br" instead of "br-recv", "br-send" sic@ */ v_ReceivedBwVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeEVS.fmtp.params.paramList, "bw"); /* @sic R5-195318: "bw" instead of "bw-recv", "bw-send" sic@ */ v_EVS_FmtpParams := cs_EVS_Fmtp_AudioParams(v_ReceivedBrVal, v_ReceivedBwVal); /* @sic R5-195318 sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(v_Fmt, v_EVS_FmtpParams); /* !!!! SHOULD BE f_SDP_MediaAttributes_EVS_AudioCommon(v_Fmt, v_EVS_FmtpParams, cs_SDP_MediaAttributes_ptime_maxptime & {c_sendrecv}); ACC. C.44 !!!! */ v_StrengthRemote := c_optional; // @sic R5-170671 sic@ } } } v_SDP_MediaAttributes := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes, cs_SDP_Attribute_sendrecv); // get curr:qos remote for SDP message in step 7 if (isvalue(p_SDP_Message183.media_list[0]) and f_SDP_AttributeList_CheckAttribute(p_SDP_Message183.media_list[0].attributes, cr_SDP_Attribute_curr_qos(c_local, c_none))) { // see C.11 step 7 Note 1 v_DirectionRemote := c_none; } else { v_DirectionRemote := c_sendrecv; } v_SDP_PrecondionAttributes := cs_SDP_PrecondionAttributes(c_sendrecv, v_DirectionRemote, c_mandatory, v_StrengthRemote); /* @sic R5-153798: usage of cs_SDP_PrecondionAttributes sic@ @sic R5s170078: remote strength v_StrengthRemote := mandatory sic@ */ return f_IMS_BuildSDP_TX(cs_SDP_Media_Audio(v_Fmt), v_MediaBandwidth_List, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes); }

After Change:

function f_IMS_BuildSDP_UpdatedOfferAudio_TX(SDP_Message p_SDP_Message183, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s141244: use of f_IMS_BuildSDP_TX sic@ */ /* @sic R5-153610: "sendrecv" media attribute removed from SDP message for C.11 step 7 sic@ */ /* @sic R5-153798: p_SDP_Message183 replacing p_SDP_Attribute_curr_qos_remote and p_Step; p_IsC45 sic@ */ /* @sic R5-153798, R5s150841: split of f_IMS_BuildSDP_AnnexC11_Step1or7 sic@ */ /* @sic R5-168057, R5-168142: p_AudioCodec := AMR_16000 sic@ */ var charstring v_Fmt := f_SDP_AudioCodec_GetFmt(p_AudioCodec); /* @sic R5-197050: fix mapping of fmt values sic@ */ var template (value) SDP_bandwidth_list v_MediaBandwidth_List; var template (omit) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes; var template (value) SemicolonParam_List v_EVS_FmtpParams; var charstring v_DirectionRemote; var charstring v_StrengthRemote := c_mandatory; var template (omit) SDP_attribute v_FmtpAttributeEVS; var charstring v_ReceivedBrVal; var charstring v_ReceivedBwVal; select (p_AudioCodec) { case (AMR_8000) { v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); v_SDP_MediaAttributes := f_SDP_MediaAttributes_AudioDef(v_Fmt); } case (AMR_16000) { /* @sic R5-168057 => C.11 sic@ */ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); /* @sic R5-130751; R5-134270: RR=2500; R5-144700: AS=37; R5-172126: RR=2000 sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AMR_WB_AudioDef(v_Fmt); /* @sic R5-144693: new function f_SDP_MediaAttributes_AudioDef; R5-168057: change for AMR_16000 sic@ */ } case (EVS_16000) { /* => C.45 @sic R5-153798 sic@*/ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(65, 0, 2000); //TC 7.6 R5-210056 v_FmtpAttributeEVS := f_SDP_AttributeList_GetAttribute(p_SDP_Message183.media_list[0].attributes, cr_SDP_Attribute_fmtp(v_Fmt)); v_ReceivedBrVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeEVS.fmtp.params.paramList, "br"); /* @sic R5-195318: "br" instead of "br-recv", "br-send" sic@ */ v_ReceivedBwVal := f_SIP_SemicolonParamList_GetParamValue(v_FmtpAttributeEVS.fmtp.params.paramList, "bw"); /* @sic R5-195318: "bw" instead of "bw-recv", "bw-send" sic@ */ v_EVS_FmtpParams := cs_EVS_Fmtp_AudioParams(v_ReceivedBrVal, v_ReceivedBwVal); /* @sic R5-195318 sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(v_Fmt, v_EVS_FmtpParams); //After R5-210056 this is the only difference for UPDATE body between 34.229-1 and -5 if (not(f_IMS_PTC_RanTypeIsNR5GC())) {v_StrengthRemote := c_optional;} // @sic R5-170671 sic@ } } v_SDP_MediaAttributes := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes, cs_SDP_Attribute_sendrecv); // get curr:qos remote for SDP message in step 7 if (isvalue(p_SDP_Message183.media_list[0]) and f_SDP_AttributeList_CheckAttribute(p_SDP_Message183.media_list[0].attributes, cr_SDP_Attribute_curr_qos(c_local, c_none))) { // see C.11 step 7 Note 1 v_DirectionRemote := c_none; } else { v_DirectionRemote := c_sendrecv; } v_SDP_PrecondionAttributes := cs_SDP_PrecondionAttributes(c_sendrecv, v_DirectionRemote, c_mandatory, v_StrengthRemote); /* @sic R5-153798: usage of cs_SDP_PrecondionAttributes sic@ @sic R5s170078: remote strength v_StrengthRemote := mandatory sic@ */ return f_IMS_BuildSDP_TX(cs_SDP_Media_Audio(v_Fmt), v_MediaBandwidth_List, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes); }

## Change 2

| Function name | f_NR5GC_IPCAN_MT_SpeechCall() |
| --- | --- |
| Reason for change | Coordination messages expected at IMS PTC are missing |
| Summary of change | Added sending of two coordination triggers (1st one is received in f_IMS_CC_StartCall() and 2nd in f_IMS_CC_TriggerDedicatedBearerActivation()) |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); //WA#784729 _7.x_Service_type handling, R5s210037, Change2 v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

After Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); //WA#784729 _7.x_Service_type handling, R5s210037, Change2 f_IMS_IPCAN_SendCoOrdMsg(p_Port); v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(p_Port); } return v_Result; }

## Change 3

| Function name | f_IMS_BuildSDP_AnswerAudio_RX |
| --- | --- |
| Reason for change | R5-210056 updates the ‘max red’ attribute value. |
| Summary of change | Allowed ‘max red’ attribute value. |
| TTCN module | IMS_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_BuildSDP_AnswerAudio_RX(SDP_KindOfAnswer_Type p_KindOfAnswer, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, boolean p_PreconditionsRequired := true) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-123077 change 3: 'inactive' removed from 'Attributes for media' for step 4 sic@ */ /* @sic R5s150704, R5s150751 - Additional changes: f_IMS_BuildSDP_RX used instead of cr_SDP_Message_Def sic@ */ /* @sic R5-153798: p_IsC45 sic@ */ /* @sic R5-168057: new default for p_AudioCodec sic@ */ /* @sic R5s170946: p_PreconditionsRequired sic@ */ var template (present) SDP_media_field v_SDP_Media_Field := cr_SDP_Media_Audio; var template (present) SDP_attribute_rtpmap_codec v_RtpmapCodec := f_SDP_AttributeRtpmapVoiceRX(p_AudioCodec); /* @sic R5-153798 sic@*/ var template (present) SDP_attribute_fmtp_format_specific_params v_FormatSpecificParameters := ?; var template (present) SemicolonParam_List v_ParamList; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes; var template (present) SDP_attribute_list v_SDP_MediaAttributes; select (p_KindOfAnswer) { case (InitialAnswer) { v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes((c_none, c_sendrecv), c_none, c_mandatory, c_mandatory); /* @sic R5s120530 change 3.15 sic@ NOTE: preconditions only if required */ if (f_IMS_PTC_RanTypeIsNR5GC()) { /* 34.229-5 A.5.1/2 183 Session Progress @sic R5-206382: implementation of TC_7_6 sic@ */ v_ParamList := { cr_GenericParam("br", "13.2"), cr_GenericParam("bw", "swb"), cr_AMR_MaxRed("220"), * }; v_FormatSpecificParameters := cr_Fmtp_ParamList(v_ParamList); } else { v_SDP_PrecondionAttributes := f_SDP_Attributes_Add_RX(v_SDP_PrecondionAttributes, cr_SDP_Attribute_conf_qos); /* @sic R5s140204 sic@ */ if (p_AudioCodec == EVS_16000) { // C.45 @sic R5-153798 sic@ v_ParamList := { /* @sic R5-195318: "br" and "bw" instead of "br-recv", "br-send" and "bw-recv", "bw-send" sic@ */ cr_GenericParam("br", ?), cr_GenericParam("bw", ?), * }; v_FormatSpecificParameters := cr_Fmtp_ParamList(v_ParamList); } } } case (FinalAnswer) { /* @sic R5-150709: cr_SDP_Attribute_sendrecv removed sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); /* NOTE: preconditions only if required */ } case else { FatalError(__FILE__, __LINE__, ""); } } v_SDP_MediaAttributes := { cr_SDP_Attribute_rtpmap(cr_SDP_AnyMediaFormatDescr, v_RtpmapCodec), // @sic R5s130681 change 2 sic@ cr_SDP_Attribute_fmtp(cr_SDP_AnyMediaFormatDescr, v_FormatSpecificParameters) }; if (not p_PreconditionsRequired) { v_SDP_PrecondionAttributes := {}; // @sic R5s170946 sic@ } return f_IMS_BuildSDP_RX(-, v_SDP_Media_Field, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes); }

After Change:

function f_IMS_BuildSDP_AnswerAudio_RX(SDP_KindOfAnswer_Type p_KindOfAnswer, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, boolean p_PreconditionsRequired := true) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-123077 change 3: 'inactive' removed from 'Attributes for media' for step 4 sic@ */ /* @sic R5s150704, R5s150751 - Additional changes: f_IMS_BuildSDP_RX used instead of cr_SDP_Message_Def sic@ */ /* @sic R5-153798: p_IsC45 sic@ */ /* @sic R5-168057: new default for p_AudioCodec sic@ */ /* @sic R5s170946: p_PreconditionsRequired sic@ */ var template (present) SDP_media_field v_SDP_Media_Field := cr_SDP_Media_Audio; var template (present) SDP_attribute_rtpmap_codec v_RtpmapCodec := f_SDP_AttributeRtpmapVoiceRX(p_AudioCodec); /* @sic R5-153798 sic@*/ var template (present) SDP_attribute_fmtp_format_specific_params v_FormatSpecificParameters := ?; var template (present) SemicolonParam_List v_ParamList; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes; var template (present) SDP_attribute_list v_SDP_MediaAttributes; select (p_KindOfAnswer) { case (InitialAnswer) { v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes((c_none, c_sendrecv), c_none, c_mandatory, c_mandatory); /* @sic R5s120530 change 3.15 sic@ NOTE: preconditions only if required */ if (f_IMS_PTC_RanTypeIsNR5GC()) { /* 34.229-5 A.5.1/2 183 Session Progress @sic R5-206382: implementation of TC_7_6 sic@ */ v_ParamList := { cr_GenericParam("br", "13.2"), cr_GenericParam("bw", "swb"), cr_AMR_MaxRed(pattern "0..220"), //R5-210056 * }; v_FormatSpecificParameters := cr_Fmtp_ParamList(v_ParamList); } else { v_SDP_PrecondionAttributes := f_SDP_Attributes_Add_RX(v_SDP_PrecondionAttributes, cr_SDP_Attribute_conf_qos); /* @sic R5s140204 sic@ */ if (p_AudioCodec == EVS_16000) { // C.45 @sic R5-153798 sic@ v_ParamList := { /* @sic R5-195318: "br" and "bw" instead of "br-recv", "br-send" and "bw-recv", "bw-send" sic@ */ cr_GenericParam("br", ?), cr_GenericParam("bw", ?), * }; v_FormatSpecificParameters := cr_Fmtp_ParamList(v_ParamList); } } } case (FinalAnswer) { /* @sic R5-150709: cr_SDP_Attribute_sendrecv removed sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); /* NOTE: preconditions only if required */ } case else { FatalError(__FILE__, __LINE__, ""); } } v_SDP_MediaAttributes := { cr_SDP_Attribute_rtpmap(cr_SDP_AnyMediaFormatDescr, v_RtpmapCodec), // @sic R5s130681 change 2 sic@ cr_SDP_Attribute_fmtp(cr_SDP_AnyMediaFormatDescr, v_FormatSpecificParameters) }; if (not p_PreconditionsRequired) { v_SDP_PrecondionAttributes := {}; // @sic R5s170946 sic@ } return f_IMS_BuildSDP_RX(-, v_SDP_Media_Field, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes); }

## Change 4

| Function name | f_NR5GC_IPCAN_MT_SpeechCall |
| --- | --- |
| Reason for change | For the MT calls there is currently missing trigger from NR5GC to IMS component after the ServiceRequest procedure is finished to signal that SIP Invite should be triggered. |
| Summary of change | Added coordination trigger for IMS component inside f_NR5GC_IPCAN_MT_SpeechCall. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); //Anritsu TTCN CR R5s210037 v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

After Change:

function f_NR5GC_IPCAN_MT_SpeechCall(IMS_IPCAN_CO_ORD_PORT p_Port, NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type { var TriggerResult_Type v_Result := NORMAL; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR_Paging_Def(p_NR_CellId, RRC_IDLE); v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B); //Anritsu TTCN CR R5s210037 f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port); // wait until IMS PTC has sent "100 Trying" and "183 Session Progress" (steps 3 and 4 of C.21) if (v_Result == NORMAL) { // => procedure shall not be aborted f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); } return v_Result; }

## Change 5

| Function name | f_NR5GC_ModifyPDUSession_AddVoice |
| --- | --- |
| Reason for change | SDAP Configuration is not provided for DRB configuration on SS side |
| Summary of change | Updated SDAP Configuration |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before change:

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var boolean v_InterworkWithEPS := false;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var integer v_NewDRBId;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId);

if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) {  // This is only set if the UE supports EPS

v_InterworkWithEPS := true;

}

// build components of Reconfiguration message

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId,

omit,

-, -, -, -,

f_BuildDefaultQoSRules("7_Voice", '7A'O), //R5s210073

cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)}));

v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false);

// FFS: 38.508-1 4.8.1-3 specifies cn association of EPS Bearer

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,

{cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5

v_NewDRBId,

-, -,

cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM

v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId)}; //FFS: 38.508-1 4.8.1-3 specifies cn association of EPS Bearer

v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false);

v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId,

cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)),

cs_38508_LogicalChannelConfig_DRB)};

// then send RRC Reconfiguration message

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SS_Config,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) });

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed");

} else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment");

}

}

After change:

function f_NR5GC_ModifyPDUSession_AddVoice(NR_CellId_Type p_NR_CellId, GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var boolean v_InterworkWithEPS := false;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var integer v_NewDRBId;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId);

if (isvalue(p_GSM_MobilityInfo.EPS_Bearer)) {  // This is only set if the UE supports EPS

v_InterworkWithEPS := true;

}

// build components of Reconfiguration message

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(p_GSM_MobilityInfo.SessionId,

omit,

-, -, -, -,

f_BuildDefaultQoSRules("7_Voice", '7A'O), //R5s210073

cs_QoSFlowDescr({cs_QoS_Flow5(v_InterworkWithEPS)}));

v_NewDRBId := f_NR_GetNextDRBId(p_GSM_MobilityInfo, false);

// FFS: 38.508-1 4.8.1-3 specifies cn association of EPS Bearer

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,

{cs_NR_DRB_ToAddMod(cs_NR_DRB_SDAP(cs_SDAP_Config(oct2int(p_GSM_MobilityInfo.SessionId), false, {7})), // QFI of 7 in flow 5

v_NewDRBId,

-, -,

cs_38508_PDCP_Config (-, -, -, -, omit))}); // UM

v_SS_Config := {f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_NewDRBId, -, -, cs_SDAP_Configuration(oct2int(p_GSM_MobilityInfo.SessionId), -, -, {7}))}; //FFS: 38.508-1 4.8.1-3 specifies cn association of EPS Bearer

v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(p_NR_CellId, {v_NewDRBId}, false);

v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId,

cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)),

cs_38508_LogicalChannelConfig_DRB)};

// then send RRC Reconfiguration message

f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId,

v_RadioBearerConfig,

v_SS_Config,

v_CellGroupConfig,

{ cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) });

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_GSM_MobilityInfo.SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed");

} else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Speech Call Establishment");

}

}

## Change 6

| Function name | f_BuildDefaultQosFlow |
| --- | --- |
| Reason for change | - For QFI 7, Table 4.8.2.3-5: Reference QoS flow #5, 5QI 1 is used which is a GBR flow. For GBR flow, there are four mandatory parameters as per TS 24.501 cl. 6.3.2.4, i.e. GFBR uplink, GFBR downlink, MFBR uplink, and MFBR downlink. Without these mandatory parameters, it will lead to syntactical error: When the flow description operation is "Create new QoS flow description" or "Modify existing QoS flow description", and the UE determines that there is a QoS flow description of a GBR QoS flow (as described in 3GPP TS 23.501 [8] table 5.7.4-1) which lacks at least one of the mandatory parameters (i.e., GFBR uplink, GFBR downlink, MFBR uplink and MFBR downlink). - Encoding of EPS Bearer ID 6 in Qos flow descriptions should be ‘0110 0000’B instead of ‘0000 0110’B. #draft proseCR shall be raised for RAN5#91-e meeting |
| Summary of change | - In Table 4.8.2.3-5: Reference QoS flow #5, add GFBR uplink, GFBR downlink, MFBR uplink, and MFBR downlink as they are mandatory parameters. - Change EBI value to ‘6’ as used in other Reference QoS flow tables like Table 4.8.2.3-1: Reference QoS flow #1. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before change:

function f_BuildDefaultQosFlow (charstring p_FlowNumber,

boolean p_InterworkWithEPS := false,

O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow

{

var template (value) QoSParametersList v_QoSParametersList;

var B6_Type v_QFI;

select (p_FlowNumber) {

case ("1") {

v_QFI := '000001'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@

}

}

case ("2") {

v_QFI := '000010'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O);

}

}

case ("2a") {

v_QFI := '000100'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("3") {

v_QFI := '000101'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("4") {

v_QFI := '000110'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("5") {

v_QFI := '000111'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '06'O);

}

}

case ("6") {

v_QFI := '001000'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '07'O);

}

}

}

return cs_QoS_Flow (v_QFI, v_QoSParametersList);

}

After change:

function f_BuildDefaultQosFlow (charstring p_FlowNumber,

boolean p_InterworkWithEPS := false,

O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow

{

var template (value) QoSParametersList v_QoSParametersList;

var B6_Type v_QFI;

select (p_FlowNumber) {

case ("1") {

v_QFI := '000001'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@

}

}

case ("2") {

v_QFI := '000010'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O);

}

}

case ("2a") {

v_QFI := '000100'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("3") {

v_QFI := '000101'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("4") {

v_QFI := '000110'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)};

}

case ("5") {

v_QFI := '000111'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O)};

v_QoSParametersList[1] := cs_QoSParameters('02'O, '03'O, '040002'O);

v_QoSParametersList[2] := cs_QoSParameters('03'O, '03'O, '040002'O);

v_QoSParametersList[3] := cs_QoSParameters('04'O, '03'O, '040005'O);

v_QoSParametersList[4] := cs_QoSParameters('05'O, '03'O, '040005'O);

if (p_InterworkWithEPS) {

v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '60'O);

}

}

case ("6") {

v_QFI := '001000'B;

v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)};

if (p_InterworkWithEPS) {

v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '07'O);

}

}

}

return cs_QoS_Flow (v_QFI, v_QoSParametersList);

}

## Change 7

| Function name | f_BuildDefaultQoSRules |
| --- | --- |
| Reason for change | Qos rule #7 for IMS voice is incorrectly configured |
| Summary of change | Correct QoS rule#7 configuration according to agreed prose CR R5-210627 |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before change:

function f_BuildDefaultQoSRules (charstring p_RuleNum,

template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules

{

var template (value) QoS_Rules v_QoS_Rules;

if (p_RuleNum == "7_Voice") {

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O), cs_QoS_Rule('02'O, int2bit(5, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B)}, p_IEI)

} else if (p_RuleNum == "7_Video") {

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('01'O, int2bit(2, 6), 'FF'O), cs_QoS_Rule('03'O, int2bit(6, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI)

} else {

v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI);

}

return v_QoS_Rules;

}

After change:

function f_BuildDefaultQoSRules (charstring p_RuleNum,

template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules

{

var template (value) QoS_Rules v_QoS_Rules;

if (p_RuleNum == "7_Voice") {

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B, '000C'O)}, p_IEI)

} else if (p_RuleNum == "7_Video") {

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('04'O, int2bit(8, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B, '000C'O)}, p_IEI)

} else {

v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI);

}

return v_QoS_Rules;

}

# Branches Executed

The test case was executed on NR band n3 on Anritus platform while n41 on Keysight platform.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_7_6_LOG.xml

Keysight\TC_7_6_LOG.html

- PICS/PIXIT parameter file:

Anritsu\ TC_7_6_PIXIT.txt
Keysight\ TC_7_6_PIXIT.xml

# References

| [1] | R5s210331 |
| --- | --- |