3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210433

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1672 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR idle mode test case 6.2.3.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SIB1 contents are incorrect with current implementation because the variable v_ChannelBandwidthDependency is used to populate v_SystemInformationBlockType1_v12j0 before it has been initialised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move setting of v_SystemInformationBlockType1_v12j0 until after v_ChannelBandwidthDependency has been initialised. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_3_11_NR5GC_EUTRA |
| --- | --- |
| Reason for change | SIB1 contents are incorrect with current implementation because the variable v_ChannelBandwidthDependency is used to populate v_SystemInformationBlockType1_v12j0 before it has been initialised. |
| Summary of change | Move setting of v_SystemInformationBlockType1_v12j0 until after v_ChannelBandwidthDependency has been initialised. |
| TTCN module | Idle_CellReSelection_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoListExt-r12 var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency; var template(value) SystemInformationBlockType1_v8h0_IEs v_SIB1_v8h0; var template (value) AllSiSchedul_Type v_SiSchedul; var template (value) EUTRA_CellInfo_Type v_CellInfo; var SysinfoType_Type v_SysinfoType; var template(value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530})); //Statement broken into declaration and initialization separately because here v_ChannelBandwidthDependency was uninitialized. Rest of the statement moved to post init of v_ChannelBandwidthDependency f_EUTRA38_IRAT_Init(c31); // Use syinfo combination 31 to get SIB24 initialised automatically //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set SIB24 as per Table 6.2.3.11.3.3-3: SystemInformationBlockType24 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.11.3.2-3) //Update SIB24 first before changing the scheduling f_EUTRA_CellInfo_SetSIB24_CellReselectionPriority_r15(eutra_Cell1, 3); f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15(eutra_Cell1, 10); //Retreive the SI scheduling ASP info with 3 SI: sib2, sib3, sib24 => to be used when reconfiguring SS with 3 SI v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1); v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth)); //Modify SIB1 scheduling as per Table 6.2.3.11.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1: SI[0]=sib2 SI[1]=sib3 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit,cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first

After Change:

function f_TC_6_2_3_11_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE / schedulingInfoListExt-r12 var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency; var template(value) SystemInformationBlockType1_v8h0_IEs v_SIB1_v8h0; var template (value) AllSiSchedul_Type v_SiSchedul; var template (value) EUTRA_CellInfo_Type v_CellInfo; var SysinfoType_Type v_SysinfoType; var template(value) SystemInformationBlockType1_v12j0_IEs v_SystemInformationBlockType1_v12j0; f_EUTRA38_IRAT_Init(c31); // Use syinfo combination 31 to get SIB24 initialised automatically //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Set SIB24 as per Table 6.2.3.11.3.3-3: SystemInformationBlockType24 of E-UTRA Cell 1 (preamble and all steps, Table 6.2.3.11.3.2-3) //Update SIB24 first before changing the scheduling f_EUTRA_CellInfo_SetSIB24_CellReselectionPriority_r15(eutra_Cell1, 3); f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15(eutra_Cell1, 10); //Retreive the SI scheduling ASP info with 3 SI: sib2, sib3, sib24 => to be used when reconfiguring SS with 3 SI v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_SysinfoType := f_EUTRA_CellInfo_GetSysinfoType(eutra_Cell1); v_SiSchedul := f_EUTRA_SS_SetupSchedulingInfo(valueof(v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1), f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell1), v_SysinfoType, valueof(v_CellInfo.Frequency.DL_ChBandwidth)); //Modify SIB1 scheduling as per Table 6.2.3.11.3.3-1: SystemInformationBlockType1 for cell 1 - as per ccombination 1: SI[0]=sib2 SI[1]=sib3 // Acc to Table 6.2.3.10.3.3-1: schedulingInfoList-v12j0 includes the same number of entries, and listed in the same order, as in schedulingInfoList (without suffix) => SIB2+SIB24 => remove SIB3 in schedulingInfoList v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth(eutra_Cell1)); f_EUTRA_CellInfo_SetSysInfo_SchedulingInfoList(eutra_Cell1, cs_SchedulingInfo_2SI(v_ChannelBandwidthDependency.SIB3_periodicity)); v_SystemInformationBlockType1_v12j0 := cs_SystemInformationBlockType1_v12j0(omit, cs_SchedulingInfoListExt_1SI(v_ChannelBandwidthDependency.SIB24_periodicity,{sibType24_v1530}));//KS@6.2.3.11 //set schedulingInfoListExt-r12 in SIB1 with sib24 v_SIB1_v8h0 := cs_SystemInformationBlockType1_v8h0_IEs(omit,cs_SystemInformationBlockType1_v9e0(omit, omit,cs_SystemInformationBlockType1_v10j0(cs_SystemInformationBlockType1_v10l0(cs_SystemInformationBlockType1_v10x0(v_SystemInformationBlockType1_v12j0))))); f_EUTRA_CellInfo_SetSysInfo_SIB1_v8h0_IEs(eutra_Cell1, v_SIB1_v8h0); // Modify sysinfo UE off with the scheduling including the extended SI sib24 // The schuling in SIB1 includes only 2 SI, the extended scheduling include 1 SI - provide to SS all 3 SIs (in v_SiSchedul) f_EUTRA_SS_ConfigureSysinfo(eutra_Cell1, -, -, -, -, v_SiSchedul); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first