3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210440

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1678 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR idle mode test case 6.1.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Iel of NSSAI_SSTs is incorect, because the length of s_nssai is 3 octet - UE can not receive RegistrationAccept because it is send with SRB2 - UE is unhappy with the RegistrationAccept because 5G-GUTI and TAI list are not included. According to 38.508 Table 4.7.1-7, they should be included - According to 38.523-1 Table 6.1.2.8.3.2-3, UE shall camp to Cell2 at step 5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change iel to := '03'O 1 - send RegistrationAccept using SRB1 - include 5G-GUTI and TAI list in RegistrationAccept - Change Cell ID to NR Cell 2 at step 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	3

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | cs_NSSAI_SSTs |
| --- | --- |
| Reason for change | Iel of NSSAI_SSTs is incorect, because the length of s_nssai is 3 octet |
| Summary of change | Change iel to := '03'O |
| TTCN module | NG_NAS_Templates |
| MCC160 Comment |  |

Before Change

template (value) NSSAI cs_NSSAI_SSTs (template (value) IEI8_Type p_IEI) := { /* @status APPROVED (NR5GC) */ iei := p_IEI, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '02'O, '01'O, omit, '01'O)} };

After Change

template (value) NSSAI cs_NSSAI_SSTs (template (value) IEI8_Type p_IEI) := { /* @status APPROVED (NR5GC) */ iei := p_IEI, iel := '03'O, s_nssai := {cs_S_NSSAI(omit, '02'O, '01'O, omit, '01'O)} };

## Change 2

| Function name | f_NR5GC_MobilityRegistrationWithRegistrationAcceptMsg |
| --- | --- |
| Reason for change | 1, UE can not receive RegistrationAccept because it is send with SRB2 2. UE is unhappy with the RegistrationAccept because 5G-GUTI and TAI list are not included. According to 38.508 Table 4.7.1-7, they should be included |
| Summary of change | 1, send RegistrationAccept using SRB1 2, include 5G-GUTI and TAI list in RegistrationAccept |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment |  |

Before Change

function f_NR5GC_MobilityRegistrationWithRegistrationAcceptMsg (NR_CellId_Type p_NR_CellId, template (omit) PLMN_List p_EPLMNs := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease) runs on NR_BASE_PTC { // @sic R5-211330 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Mobility Registration procedure steps 4 to 6 // Step 4: Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, omit, //v_GUTIToSend omit, //v_TAIList p_EPLMNs, //p_EPLMNs cs_NSSAI_SSTs('15'O)); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, -, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(p_NR_CellId); // only SRB1 // Step 6 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } }

After Change

function f_NR5GC_MobilityRegistrationWithRegistrationAcceptMsg (NR_CellId_Type p_NR_CellId, template (omit) PLMN_List p_EPLMNs := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease) runs on NR5GC_PTC { // @sic R5-211330 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Mobility Registration procedure steps 4 to 6 // Step 4: Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, p_EPLMNs, //p_EPLMNs cs_NSSAI_SSTs('15'O)); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//KS r5sxx change_6128 // Step 5 f_NR5GC_Registration_Complete(p_NR_CellId); // only SRB1 // Step 6 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } }

## Change 3

| Function name | f_TC_6_1_2_8_NR5GC_TestBody |
| --- | --- |
| Reason for change | According to 38.523-1 Table 6.1.2.8.3.2-3, UE shall camp to Cell2 at step 5 |
| Summary of change | Change Cell ID to NR Cell 2 at step 5 |
| TTCN module | Idle_CellReSelection_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_6_1_2_8_NR5GC_TestBody() runs on NR5GC_PTC { //Power levels on FR2 are FFS …. // @siclog "Step 5" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.5 indicate that the UE is camped on NR Cell 2? // NOTE: The UE performs a Registration procedure for mobility registration update with PLMN1 and PLMN3 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message. f_NR5GC_MobilityRegistrationWithRegistrationAcceptMsg(nr_Cell4, cs_PLMN_List_2PLMNs(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell4)))); // Table 6.1.2.8.3.3-3 //Re-start the Idle Mode Generic timer for step 7 ….. }

After Change

function f_TC_6_1_2_8_NR5GC_TestBody() runs on NR5GC_PTC { //Power levels on FR2 are FFS …. // @siclog "Step 5" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.5 indicate that the UE is camped on NR Cell 2? // NOTE: The UE performs a Registration procedure for mobility registration update with PLMN1 and PLMN3 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message. f_NR5GC_MobilityRegistrationWithRegistrationAcceptMsg(nr_Cell2, cs_PLMN_List_2PLMNs(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell1)), f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell4)))); // Table 6.1.2.8.3.3-3 //Re-start the Idle Mode Generic timer for step 7 ….. }