3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210441

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0688 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IA5toGsm7Bit functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In function f_CharPacking_IA5toGsm7Bit_WithFiller the 3rd parameter p_NumFillbits is used to to add a filler bit at the end of the header and in front of the actual data According to 23.040 sec 9.2.3.4 If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header In the current TTCN implementation the filler bit is added to the bitstring from the return octet of f_CharPacking_IA5toGsm7Bit, this is incorrect as the filler bit needs to be added at the start of the SMS message so the first character starts at the septets boundary . |  |  |  |  |  |  |  |  |
| Summary of change: |  | p_NumFillbits is passed as a parameter to f_CharPacking_IA5toGsm7Bit and the first character of the SMS starts on the septets boundary. “The” -> 546865 results in 1100101110100010101000 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to f_CharPacking_IA5toGsm7Bit_WithFiller	3

1.2	Correction to f_CharPacking_IA5toGsm7Bit	5

# Corrections required

## Correction to f_CharPacking_IA5toGsm7Bit_WithFiller

| Function name | f_CharPacking_IA5toGsm7Bit_WithFiller |
| --- | --- |
| Reason for change | In function f_CharPacking_IA5toGsm7Bit_WithFiller the 3rd parameter p_NumFillbits is used to to add a filler bit at the end of the header and in front of the actual data According to 23.040 sec 9.2.3.4 If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header In the current TTCN implementation the filler bit is added to the bitstring from the return octet of f_CharPacking_IA5toGsm7Bit. This is incorrect as the filler bit needs to be added at the start of the SMS message so the first character starts at the septets boundary. |
| Summary of change | v_Fillbits is updated and passed into function f_CharPacking_IA5toGsm7Bit. Removed update to v_BitString at the end of the function as its incorrect to update it here. |
| TTCN module | IMS_SMS_Messages |
| MCC160 Comment |  |

Before Change:

function f_CharPacking_IA5toGsm7Bit_WithFiller(charstring p_Text, Gsm7BitPacking_Type p_Packing, integer p_NumFillbits) return octetstring { // @sic R5s201446 sic@ var octetstring v_OctetString; var bitstring v_BitString := ''B; var bitstring v_Fillbits := ''B; var integer v_BitStringLength := 0; // conversion of the text string, padding is added v_OctetString := f_CharPacking_IA5toGsm7Bit(p_Text, p_Packing); // conversion to a bitstring v_BitString := oct2bit(v_OctetString); // note the length of the bitstring v_BitStringLength := lengthof(v_BitString); // calculate the wanted fillbits v_Fillbits := int2bit(0, p_NumFillbits); // 1..6 fill bits, all 0 @sic R5s201329 sic@ // concatenate fillbits and bitstring v_BitString := v_Fillbits & v_BitString; // cut the resulting bitstring to the length of the original bitstring - assumption that the user of this function knows there is enough padding v_BitString := substr(v_BitString, 0, v_BitStringLength); // convert resulting bitstring to octets return(bit2oct(v_BitString)); }

After Change:

function f_CharPacking_IA5toGsm7Bit_WithFiller(charstring p_Text, Gsm7BitPacking_Type p_Packing, integer p_NumFillbits) return octetstring { // @sic R5s201446 sic@ var octetstring v_OctetString; var bitstring v_BitString := ''B; var bitstring v_Fillbits := ''B; var integer v_BitStringLength := 0; v_Fillbits := int2bit(0, p_NumFillbits); // 1..6 fill bits, all 0 @sic R5s201329 sic@ // conversion of the text string, padding is added v_OctetString := f_CharPacking_IA5toGsm7Bit(p_Text, p_Packing, v_Fillbits); // conversion to a bitstring v_BitString := oct2bit(v_OctetString); // note the length of the bitstring v_BitStringLength := lengthof(v_BitString); // calculate the wanted fillbits // concatenate fillbits and bitstring // v_BitString := v_Fillbits & v_BitString; not needed here,as filler bits are done within f_CharPacking_IA5toGsm7Bit // cut the resulting bitstring to the length of the original bitstring - assumption that the user of this function knows there is enough padding v_BitString := substr(v_BitString, 0, v_BitStringLength); // convert resulting bitstring to octets return(bit2oct(v_BitString)); }

## Correction to f_CharPacking_IA5toGsm7Bit

| Function name | f_CharPacking_IA5toGsm7Bit |
| --- | --- |
| Reason for change | In function f_CharPacking_IA5toGsm7Bit_WithFiller the 3rd parameter p_NumFillbits is used to to add a filler bit at the end of the header and in front of the actual data According to 23.040 sec 9.2.3.4 If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header In the current TTCN implementation the filler bit is added to the bitstring from the return octet of f_CharPacking_IA5toGsm7Bit. This is incorrect as the filler bit needs to be added at the start of the SMS message so the first character starts at the septets boundary. |
| Summary of change | Added new parameter to receive filler bits and updated v_BitString and v_BitStringLength |
| TTCN module | CommonDefs |
| MCC160 Comment |  |

Before Change:

function f_CharPacking_IA5toGsm7Bit(charstring p_Text, Gsm7BitPacking_Type p_Packing) return octetstring { /* @sic R5s130103 change 3: consideration of non alpha numeric characters sic@ */ var octetstring v_OctetString := char2oct(p_Text); // "Hallo" -> '48616C6C6F'O var bitstring v_BitString := ''B; var bitstring v_Padding := ''B; var integer v_TextLength; var integer v_BitStringLength := 0; var integer v_IntValue; var integer v_NoOfOctets; var integer v_NoOfPaddingBits; var integer v_Index; var integer i; // convert ascii table to GSM 7 bit table v_OctetString := fl_XlatAcsiiToGsm7Bit(v_OctetString); // @sic R5s130103 change 3 sic@ v_TextLength := lengthof(v_OctetString); if (((p_Packing == SMS_Packing) and (v_TextLength > 160)) or // a maximum of 160 characters for SMS (i.e. 140 octets) ((p_Packing == CBS_Packing) and (v_TextLength > 93)) or // a maximum of 93 characters for CBS (i.e. 82 octets) ((p_Packing == USSD_Packing) and (v_TextLength > 182))) { // a maximum of 182 characters for USSD (i.e. 160 octets) FatalError(__FILE__, __LINE__, "input string is too long"); } for (i:=0; i<v_TextLength; i:=i+1) { v_IntValue := oct2int(v_OctetString[i]); v_BitString := int2bit(v_IntValue, 7) & v_BitString; // concat 7 bits only v_BitStringLength := v_BitStringLength + 7; } // add padding: v_NoOfOctets := (v_BitStringLength + 7) / 8; v_NoOfPaddingBits := (v_NoOfOctets * 8) - v_BitStringLength; if (v_NoOfPaddingBits > 0) { if ((p_Packing == USSD_Packing) and (v_NoOfPaddingBits == 7)) { v_Padding := '0001101'B; // use <CR> instead of the @ character; see TS 23.038 cl. 6.1.2.3.1 } else { v_Padding := int2bit(0, v_NoOfPaddingBits); // 1..7 padding bits, all 0 } } v_BitString := v_Padding & v_BitString; // convert bitstring back to octetstring: the first character is at the end of the bitstring // => we need to take the octets from the end of the bitstring v_OctetString := ''O; for (i:=0; i < v_NoOfOctets; i:=i+1) { v_Index := (v_NoOfOctets - (i+1)) * 8; // get start index of the next octet v_OctetString[i] := bit2oct(substr(v_BitString, v_Index, 8)); } return v_OctetString; }

After Change:

function f_CharPacking_IA5toGsm7Bit(charstring p_Text, Gsm7BitPacking_Type p_Packing, bitstring p_fillBits := ''B) return octetstring //WA# filler bits added. { /* @sic R5s130103 change 3: consideration of non alpha numeric characters sic@ */ var octetstring v_OctetString := char2oct(p_Text); // "Hallo" -> '48616C6C6F'O var bitstring v_BitString := p_fillBits; // filler bits added. var bitstring v_Padding := ''B; var integer v_TextLength; var integer v_BitStringLength := lengthof(p_fillBits); //WA# take the length of the filler bits into consideration. var integer v_IntValue; var integer v_NoOfOctets; var integer v_NoOfPaddingBits; var integer v_Index; var integer i; // convert ascii table to GSM 7 bit table v_OctetString := fl_XlatAcsiiToGsm7Bit(v_OctetString); // @sic R5s130103 change 3 sic@ v_TextLength := lengthof(v_OctetString); if (((p_Packing == SMS_Packing) and (v_TextLength > 160)) or // a maximum of 160 characters for SMS (i.e. 140 octets) ((p_Packing == CBS_Packing) and (v_TextLength > 93)) or // a maximum of 93 characters for CBS (i.e. 82 octets) ((p_Packing == USSD_Packing) and (v_TextLength > 182))) { // a maximum of 182 characters for USSD (i.e. 160 octets) FatalError(__FILE__, __LINE__, "input string is too long"); } for (i:=0; i<v_TextLength; i:=i+1) { v_IntValue := oct2int(v_OctetString[i]); v_BitString := int2bit(v_IntValue, 7) & v_BitString; // concat 7 bits only v_BitStringLength := v_BitStringLength + 7; } // add padding: v_NoOfOctets := (v_BitStringLength + 7) / 8; v_NoOfPaddingBits := (v_NoOfOctets * 8) - v_BitStringLength; if (v_NoOfPaddingBits > 0) { if ((p_Packing == USSD_Packing) and (v_NoOfPaddingBits == 7)) { v_Padding := '0001101'B; // use <CR> instead of the @ character; see TS 23.038 cl. 6.1.2.3.1 } else { v_Padding := int2bit(0, v_NoOfPaddingBits); // 1..7 padding bits, all 0 } } v_BitString := v_Padding & v_BitString; // convert bitstring back to octetstring: the first character is at the end of the bitstring // => we need to take the octets from the end of the bitstring v_OctetString := ''O; for (i:=0; i < v_NoOfOctets; i:=i+1) { v_Index := (v_NoOfOctets - (i+1)) * 8; // get start index of the next octet v_OctetString[i] := bit2oct(substr(v_BitString, v_Index, 8)); } return v_OctetString; }