3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210450

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0689 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.31 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.31 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.31 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.31.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	8

3	Branches executed	8

4	Execution Log Files	8

4.1	Samsung SM-G996B	8

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.31 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.31.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350+SDX60

Verification Status:	PASS

# Corrections required

## 1 Change 1

| Testcase name | TC_7_31_NR5GC |
| --- | --- |
| Reason for change | Guard timer for this test is insufficient due to the delays at steps 21 and 23 |
| Summary of change | Increase guard timer to 3500s |
| TTCN module | IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

testcase TC_7_31_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Session Timer / MT Call / Remote end supports but does not send Session-Expires / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_7_31_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

After Change:

testcase TC_7_31_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Session Timer / MT Call / Remote end supports but does not send Session-Expires / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(3500); v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_7_31_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

## 2.2 Change 2

| Function name | f_TC_7_31_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Step 22 mandates presence of “timer” in the Supported header of 200 OK response for UPDATE. However based on RFC 4028 cl 7.1 the “timer” support should be indicated only in SIP requests. Note : An associated prose CR on 34.229-5 will be raised at RAN5#91. |
| Summary of change | Removed the check for “timer” in the Supported header in step 22 |
| TTCN module | NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_31_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1_Step1_10A(); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(v_InviteRequestWithSdp, cr_Require_timer, -, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Step 21" siclog@ v_MessageHeaderUpdate := f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(v_InviteRequest, v_RoutingInfo_DL, -, v_ContactUrlUE, v_DeltaSec, "uac"); // @siclog "Step 22" siclog@ f_IMS_CallWithSessionTimer_Receive200OK(v_RoutingInfo_DL.Protocol, v_MessageHeaderUpdate, -, cr_Supported_timer, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Steps 23 - 24" siclog@ f_IMS_CallWithSessionTimer_MOCallRelease(v_InviteRequest, v_ContactUrlSS, v_DeltaSec); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 23"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); }

After Change:

function f_TC_7_31_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlSS; var SipUrl v_ContactUrlUE; var IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) MessageHeader v_MessageHeaderUpdate; var integer v_DeltaSec := 1800; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1 - 8" IPCAN siclog@ // @siclog "Step 9 - 18" IPCAN siclog@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_1_Step1_10A(); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_RoutingInfo_DL := v_InviteRequestWithSdp.RoutingInfo; // @siclog "Steps 19 - 20" siclog@ f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(v_InviteRequestWithSdp, cr_Require_timer, -, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19"); // @siclog "Step 21" siclog@ v_MessageHeaderUpdate := f_IMS_CallWithSessionTimer_SendUpdateAfterHalfDeltaSec(v_InviteRequest, v_RoutingInfo_DL, -, v_ContactUrlUE, v_DeltaSec, "uac"); // @siclog "Step 22" siclog@ f_IMS_CallWithSessionTimer_Receive200OK(v_RoutingInfo_DL.Protocol, v_MessageHeaderUpdate, -, -, v_DeltaSec, "uac"); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22"); // @siclog "Steps 23 - 24" siclog@ f_IMS_CallWithSessionTimer_MOCallRelease(v_InviteRequest, v_ContactUrlSS, v_DeltaSec); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 23"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MT_SpeechCall); }

# Branches executed

The test case was executed on NR bands n3 and n41

# Execution Log Files

## Qualcomm SM8350+SDX60

The Qualcomm SM8350+SDX60  UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_31_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_31_PIXIT.xml

The Qualcomm SM8350+SDX60  UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_31_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_7_31_PIXIT.txt

# References

| [1] | R5s210451 : Supporting information for addition of IMS 5GS call control tets case 7.31 |
| --- | --- |