3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210453

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1686 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcitons for EN-DC RRC test case 8.2.4.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Changes made for co-ordinate message at NR and EUTRA side for test case 8.2.4.3.1.1 which cause a mismatch in coordinate message while RB configuration and sending system information of NR cell1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct NR<>EUTRA coordination |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.3.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_4_3_1_x_TestBody() |
| --- | --- |
| Reason for change | EUTRA PTC needs NR PTC to send coordination message for configuring the RB configuration. |
| Summary of change | Wait for coordination message from NR PTC for RB configuration. |
| TTCN module | RRC_Measurement_CA_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_4_3_1_x_TestBody() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; var EUTRA38_SecurityParamsExtd_Type v_Security_Params; //Send a trigger to NR to change the power f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 1-2" siclog@ //SS transmits an RRCConnectionReconfiguration message containing RRCReconfiguration message to configure SCell (NR Cell 3) and SRB3 //The UE transmits an RRCConnectionReconfigurationComplete message containing RRCReconfigurationComplete message v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); // NR_RBConfig received is not used afterwards f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, -, omit); //@sic R5s210160, R5-210399 sic@ …………………….................................................... ………………………………………………………….. }

After Change

function fl_TC_8_2_4_3_1_x_TestBody() runs on EUTRA_5GS_PTC { var template (omit) octetstring v_NR_SCGConfig := omit; var template (omit) octetstring v_NR_RBConfig := omit;// KS#200XXX var EUTRA38_SecurityParamsExtd_Type v_Security_Params; //Send a trigger to NR to change the power f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 1-2" siclog@ //SS transmits an RRCConnectionReconfiguration message containing RRCReconfiguration message to configure SCell (NR Cell 3) and SRB3 //The UE transmits an RRCConnectionReconfigurationComplete message containing RRCReconfigurationComplete message v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");//KS#200XXX f_EUTRA38_508_ENDC_SendReconfiguration(eutra_Cell1, -, -, v_NR_SCGConfig, v_NR_RBConfig,//KS#200XXX -,omit); ………………………………………………………………........... ……………………………………………………………………… }

## Change 2

| Function name | function f_TC_8_2_4_3_1_Common_ENDC_NR() |
| --- | --- |
| Reason for change | EUTRA PTC waits for system information of NR Cell1 to be sent to EUTRA after the NR preamble complete. But current TTCN implementation has the f_NR_SendCoOrdSysInfoToEUTRA called before NR preamble |
| Summary of change | Move the f_NR_SendCoOrdSysInfoToEUTRA call to after NR preamble |
| TTCN module | RRC_Measurement_CA_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_2_4_3_1_Common_ENDC_NR(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { ………………………………………………….. ………………………………………………….. f_NR_CellConfig_SCell(p_NeigbourCellId); // Send SysInfo from NR Cell1 to EUTRA f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); f_NR_TestBody_Set(true); fl_TC_8_2_4_3_1_x_TestBody(p_SCellId, p_NeigbourCellId); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s201309 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); } // End of f_TC_8_2_4_3_1_Common_ENDC_NR()

After Change

function f_TC_8_2_4_3_1_Common_ENDC_NR(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId, NR_CA_Tested_Type p_NR_CA_Tested) runs on ENDC_NR_PTC { ………………………………………………. ………………………….................................. f_NR_CellConfig_SCell(p_NeigbourCellId); f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); // Send SysInfo from NR Cell1 to EUTRA f_NR_SendCoOrdSysInfoToEUTRA({nr_Cell1}); f_NR_TestBody_Set(true); fl_TC_8_2_4_3_1_x_TestBody(p_SCellId, p_NeigbourCellId); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s201309 sic@ f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); } // End of f_TC_8_2_4_3_1_Common_ENDC_NR()