3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210458

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1690 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC EPS fall back test case 11.1.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify NR TC 11.1.1: MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 11.1.1 required for reverification. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case on D21wk12 TTCN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	7

2.4	Change 4	9

2.5	Change 5	10

2.6	Change 6	10

3	Branches Executed	11

4	Execution Log Files	11

4.1	Qualcomm SM8350 + SDX60	11

4.2	MediaTek MT6297	11

5	References	11

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.1 which is part of the NR5GC IRAT test suite.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.1

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60 IMS UE

MediaTek MT6297

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_IMSFallback_PreambleRegistrations |
| --- | --- |
| Reason for change | Same change as Change 2 in R5s210149 |
| Summary of change |  |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function fl_IMSFallback_PreambleRegistrations() runs on IMS_PTC { f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_Registration(OtherIPCAN_E); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function fl_IMSFallback_PreambleRegistrations() runs on IMS_PTC { f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_Registration(OtherIPCAN_E); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); //Must not wait for for IPCAN release as the EUTRA wont send it at this moment f_IMS_IpcanReleaseWithOptionalDeregistration(-,false); f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

## Change 2

| Function name | fl_TC_11_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | Same change as Change 3 bullet 2 in R5s210149 |
| Summary of change |  |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_1_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var NR_RadioBearerId_Type v_SRB; // @siclog "Step 0" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ // return the Sysinfo before the test body really starts f_EUTRA_NR_SendSysinfoCoOrd(EUTRA, f_NR5GC_IRAT_ListOfFrequencies()); //@siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 2-3" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 4" siclog@ if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B, -, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Steps 5-8" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 9-13" siclog@ // done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 14" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); // @sic R5s210149 sic@ f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // @sic R5s210149 sic@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 35" siclog@ v_SRB := f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease); //@siclog "Step 36 -37" siclog@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); return v_SRB; }

After Change

import from CommonIP all; import from IP_PTC_Templates all; import from IP_PTC_CtrlMsgs all; import from IP_ASP_TypeDefs all; function fl_TC_11_1_1_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var NR_RadioBearerId_Type v_SRB; var template (value) QosFlow_Identification_Type v_QosFlowId; //Anritsu TTCN CR R5s210149 var PDN_AddressInfo_Type v_PDN_AddressInfo; var template (value) IP_AddrInfo_Type v_IPv4Addr, v_IPv6Addr; // @siclog "Step 0" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ // return the Sysinfo before the test body really starts f_EUTRA_NR_SendSysinfoCoOrd(EUTRA, f_NR5GC_IRAT_ListOfFrequencies()); //@siclog "Step 1" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 2-3" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 4" siclog@ if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B, -, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //@siclog "Steps 5-8" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 9-13" siclog@ // done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 14" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); // @sic R5s210149 sic@ f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // @sic R5s210149 sic@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); //@siclog "Step 35" siclog@ v_SRB := f_NR5GC_IRAT_S1ToN1_IdleMode(nr_Cell1, noRrcConnectionRelease); //Anritsu TTCN CR R5s210149 v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(tsc_NR_DRB1); //IP handling will go through QFI 2, because this QFI is associated with the default QoS rule @sic R5s200564, R5s201505 sic@ v_PDN_AddressInfo := f_PDN_AddressInfo_Get(PDN_1); v_IPv4Addr := cs_IPv4Addr(v_PDN_AddressInfo.PCSCF_IPAddressIPv4); v_IPv6Addr := cs_IPv6Addr(v_PDN_AddressInfo.PCSCF_IPAddressIPv6); f_IP_AddExplicitRouting_TCP_UDP(IP, 5062, v_IPv4Addr, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId)); f_IP_AddExplicitRouting_TCP_UDP(IP, 5062, v_IPv6Addr, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId)); //@siclog "Step 36 -37" siclog@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); return v_SRB; }

## Change 3

| Function name | f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg |
| --- | --- |
| Reason for change | Same change as Change 5 in R5s210149 |
| Summary of change |  |
| TTCN module |  |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, NasCount_Type p_NasCountUL, UInt_Type p_NoOfAddDRBs_AM := 0, UInt_Type p_NoOfAddDRBs_UM := 0, template (omit) NAS_MSG_RequestList_Type p_NAS_MsgList := omit, boolean p_ConfigureSRB2 := true) runs on EUTRA_Multi5GS_PTC { var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var template (value) RadioBearerList_Type v_DrbConfigList; var template (value) EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); var template (value) EUTRA38_DRBInfoList_Type v_NewDRBs := {}; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) SRB_ToAddModList v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; var integer i; if (not p_ConfigureSRB2) { v_SRB_ToAddModList := omit; } // first add any new DRBs requested v_NewDRBs := fl_EUTRA38_MultiPDN_BuildDedicatedDRBList(p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (valueof(v_ExistingDRBs), valueof(v_NewDRBs)); // now compile the full list for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DrbConfigList[i] := v_ExistingDRBs[i].SS_Config; v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs)); /* step 6-8 of procedure in 36.508 cl. 4.5.3.3 */ v_AuthParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_AuthParams, p_NasCountUL, v_DrbConfigList); f_EUTRA_Security_Set(v_AuthParams); /* Step 8 of procedure in 36.508 cl. 4.5.3.3 */ f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(p_CellId, p_RRC_TI, v_SRB_ToAddModList, v_DRB_ToAddModList, p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM, p_NAS_MsgList); }

After Change

function f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, NasCount_Type p_NasCountUL, UInt_Type p_NoOfAddDRBs_AM := 0, UInt_Type p_NoOfAddDRBs_UM := 0, template (omit) NAS_MSG_RequestList_Type p_NAS_MsgList := omit, boolean p_ConfigureSRB2 := true, boolean p_IMScontextOnly := false) runs on EUTRA_Multi5GS_PTC { var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get(); var template (value) RadioBearerList_Type v_DrbConfigList; var template (value) EUTRA38_DRBInfoList_Type v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); var template (value) EUTRA38_DRBInfoList_Type v_NewDRBs := {}; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) SRB_ToAddModList v_SRB_ToAddModList := {cs_508_SRB2_ToAddMod_DEFAULT}; var integer i, v_Index; if (not p_ConfigureSRB2) { v_SRB_ToAddModList := omit; } // first add any new DRBs requested v_NewDRBs := fl_EUTRA38_MultiPDN_BuildDedicatedDRBList(p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (valueof(v_ExistingDRBs), valueof(v_NewDRBs)); // now compile the full list // in NR in PDUsessionEstAccept only the IMS context was brought as mapped context, therefore in steps 21-23 in 11.1.1 only the IMS context should be brought up if (p_IMScontextOnly) { v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, tsc_EUTRA38_IMS_DRB); v_ExistingDRBs := {v_ExistingDRBs[v_Index]}; } for (i:=0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DrbConfigList[i] := v_ExistingDRBs[i].SS_Config; v_DRB_ToAddModList[i] := v_ExistingDRBs[i].DRB_ToAdd; } f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs)); /* step 6-8 of procedure in 36.508 cl. 4.5.3.3 */ v_AuthParams := f_EUTRA_RRC_ActivateSecurity(p_CellId, v_AuthParams, p_NasCountUL, v_DrbConfigList); f_EUTRA_Security_Set(v_AuthParams); /* Step 8 of procedure in 36.508 cl. 4.5.3.3 */ f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(p_CellId, p_RRC_TI, v_SRB_ToAddModList, v_DRB_ToAddModList, p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM, p_NAS_MsgList); }

## Change 4

| Function name | f_EUTRA38_NR5GC_PreambleAndSwitchOff |
| --- | --- |
| Reason for change | The EUTRA->IMS coordination in f_EUTRA38_NR5GC_PreambleAndSwitchOff has currently no corresponding handling in IMS component and so it should be removed. |
| Summary of change | Removed EUTRA->IMS coordination. |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_NR5GC_PreambleAndSwitchOff(EUTRA_CellId_Type p_CellId) runs on EUTRA_Multi5GS_PTC { // Attach on EUTRA f_EUTRA38_NR5GC_Preamble (p_CellId, TESTMode_OFF, RRC_IDLE, noRrcConnectionRelease); // Detach on Switch off f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe (p_CellId, RRC_CONNECTED); // @sic R5-204399 sic@ // Tell IMS PTC to stop waiting for De-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s210149 sic@ // Reset Routing Table f_IP_Handling_StopPDN(IP); // @sic R5s210149 sic@ // Turn Power Off f_EUTRA38_SetCellPower(p_CellId, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // Tell NR to turn on and do registration f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Ok, UE now registered on both RATs }

After Change

function f_EUTRA38_NR5GC_PreambleAndSwitchOff(EUTRA_CellId_Type p_CellId) runs on EUTRA_Multi5GS_PTC { // Attach on EUTRA f_EUTRA38_NR5GC_Preamble (p_CellId, TESTMode_OFF, RRC_IDLE, noRrcConnectionRelease); // Detach on Switch off f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe (p_CellId, RRC_CONNECTED); // @sic R5-204399 sic@ // Tell IMS PTC to stop waiting for De-register //This coordination is not handled anywhere REMOVED //f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s210149 sic@ // Reset Routing Table f_IP_Handling_StopPDN(IP); // @sic R5s210149 sic@ // Turn Power Off f_EUTRA38_SetCellPower(p_CellId, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // Tell NR to turn on and do registration f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Ok, UE now registered on both RATs }

## Change 5

| Function name | fl_IMSFallback_PreambleRegistrations |
| --- | --- |
| Reason for change | Same change as Change 10 in R5s210149 |
| Summary of change |  |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(boolean p_AllowRetransmission := false) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(IPCAN, t_Dummy, -, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

After Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(boolean p_AllowRetransmission := false, boolean p_WaitForIpcanRelease := true) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(IPCAN, t_Dummy, p_WaitForIpcanRelease, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

## Change 6

| Function name | f_IP_AddExplicitRouting_TCP_UDP |
| --- | --- |
| Reason for change | New function: Same change as Change 11 in R5s210149 |
| Summary of change |  |
| TTCN module | IP_PTC_CtrlMsgs.ttcn |
| MCC160 Comment |  |

After Change

function f_IP_AddExplicitRouting_TCP_UDP(IP_RAT_CTRL_PORT p_Port, PortNumber_Type p_MediaPort, template (omit) IP_AddrInfo_Type p_IpAddrRemoteUE, template (value) IP_DrbInfo_Type p_DrbInfo) { fl_IP_Config(p_Port, cs_IP_ConfigureDiscardDataLoopbackDataReq(tcp, p_MediaPort, p_IpAddrRemoteUE, p_DrbInfo, omit)); fl_IP_Config(p_Port, cs_IP_ConfigureDiscardDataLoopbackDataReq(udp, p_MediaPort, p_IpAddrRemoteUE, p_DrbInfo, omit)); }

# Branches Executed

The test case was executed on NR band n41 on Qualcomm and n78 on MediaTek UE.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Qualcomm\TC_11_1_1_LOGS.xml

- PICS/PIXIT parameter file:

Qualcomm\TC_11_1_1_PIXIT.txt

## MediaTek MT6297

The MediaTek MT6297 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

MediaTek\TC11_1_1_LOGS.xml

- PICS/PIXIT parameter file:

MediaTek\TC11_1_1_PIXIT.txt

# References

| [1] | R5s210459 |
| --- | --- |