3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210462

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0690 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR TC 7.25 MTSI MT Voice Call without SDP offer in INVITE / 5GS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 7.25 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.25.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications 38.523-1 |  |  |  | TS34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	6

3	Branches Executed	8

4	Execution Log Files	8

4.1	Qualcomm SM8350 + SDX60	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.25 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.25

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes from R5s210102 are also required

## 2.1 Change 1

| Function name | f_BuildDefaultQosFlow |
| --- | --- |
| Reason for change | Lengths of QoS parametes that are used in PDU modification command are incorrect and may result in UE invalidating the message. |
| Summary of change | Corrected QoS parameters length for QoSFlow number 5. |
| TTCN module | Common\NG_NAS\NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } case ("2a") { v_QFI := '000100'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("3") { v_QFI := '000101'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("4") { v_QFI := '000110'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("5") { v_QFI := '000111'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(320, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(320, 1))}; // @sic R5s210330 sic@ if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '60'O); // @sic R5s210330 sic@ } } case ("6") { v_QFI := '001000'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)/*, cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(128, 1))*/}; if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '70'O); } } case ("7") { // @sic R5-211499 sic@ v_QFI := '001001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '52'O)}; // 5QI 82 } } return cs_QoS_Flow (v_QFI, v_QoSParametersList); }

After Change:

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } case ("2a") { v_QFI := '000100'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("3") { v_QFI := '000101'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("4") { v_QFI := '000110'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; } case ("5") { v_QFI := '000111'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '01'O), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '03'O, fl_FlowParameterBitRate(320, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '03'O, fl_FlowParameterBitRate(320, 1))}; // @sic R5s210330 sic@ if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '60'O); // @sic R5s210330 sic@ } } case ("6") { v_QFI := '001000'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '02'O)/*, cs_QoSParameters(tsc_QoSFlowParameter_GFBR_UL, '02'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_GFBR_DL, '03'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_UL, '04'O, fl_FlowParameterBitRate(128, 1)), cs_QoSParameters(tsc_QoSFlowParameter_MFBR_DL, '05'O, fl_FlowParameterBitRate(128, 1))*/}; if (p_InterworkWithEPS) { v_QoSParametersList[5] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '06'O, '70'O); } } case ("7") { // @sic R5-211499 sic@ v_QFI := '001001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '52'O)}; // 5QI 82 } } return cs_QoS_Flow (v_QFI, v_QoSParametersList); }

## 2.2 Change 2

| Function name | f_IMS_BuildSDP_InitialOfferAudio_RX |
| --- | --- |
| Reason for change | As per TS24.229 section 6.1.2, the Strength-tag for remote segment can be either “optional” or “mandatory”. |
| Summary of change | Updated Strength-tag for remote segment to consider both “optional” or “mandatory” value instead of just considering “optional”. |
| TTCN module | IMS_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_BuildSDP_InitialOfferAudio_RX(boolean p_PreconditionsRequired := true, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-135020, R5-135021: new PICS pc_IMS_RtcpDuringVoiceSession sic@ */ /* @sic R5w140113, R5w140009: new SDP type definitions */ /* @sic R5s150704, R5s150751 - Additional changes: parameter p_PreconditionsRequired, f_IMS_BuildSDP_RX used instead of cr_SDP_Message_AnySession sic@ */ /* @sic R5-160906: p_RtcpDuringVoiceSession removed sic@ */ /* @sic R5-168053: p_AudioCodecList sic@ */ var template (present) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (present) integer v_BandwidthRS := ?; /* @sic R5-160906 sic@ */ var template (present) integer v_BandwidthRR := (1..infinity); /* @sic R5-160906 sic@ */ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes := {}; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171476: C.21a sic@ */ var boolean v_IsCatM1 := f_IMS_TestcaseIsCatM1(); /* @sic R5-174483: C.21d sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributesAudioCodecListRX(p_AudioCodecList); /* @sic R5-168031, R5-168053, R5-168054, R5-172931 sic@ */ /* @sic R5-150709: cr_SDP_Attribute_inactive removed sic@ */ if (not v_IsWLAN and not v_IsCatM1) { /* Attributes for media security mechanism may be required for C.21 only */ v_SDP_MediaAttributes := f_IMS_SDP_MediaAttributes_AddMediaSecurityAttributesRX(v_SDP_MediaAttributes); /* @sic R5-160293, R5-160915 new TC H.12.3 sic@ */ } if (p_PreconditionsRequired) { /* @sic R5s150704, R5s150751 - Additional changes sic@ */ if (v_IsWLAN) { /* @sic R5-171476 - WLAN Annex C.21a sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); } else { v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_optional); } } return f_IMS_BuildSDP_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes, -, v_BandwidthRS, v_BandwidthRR); }

After Change:

function f_IMS_BuildSDP_InitialOfferAudio_RX(boolean p_PreconditionsRequired := true, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR) runs on IMS_PTC return template (present) SDP_Message { /* @sic R5-135020, R5-135021: new PICS pc_IMS_RtcpDuringVoiceSession sic@ */ /* @sic R5w140113, R5w140009: new SDP type definitions */ /* @sic R5s150704, R5s150751 - Additional changes: parameter p_PreconditionsRequired, f_IMS_BuildSDP_RX used instead of cr_SDP_Message_AnySession sic@ */ /* @sic R5-160906: p_RtcpDuringVoiceSession removed sic@ */ /* @sic R5-168053: p_AudioCodecList sic@ */ var template (present) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (present) integer v_BandwidthRS := ?; /* @sic R5-160906 sic@ */ var template (present) integer v_BandwidthRR := (1..infinity); /* @sic R5-160906 sic@ */ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes := {}; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171476: C.21a sic@ */ var boolean v_IsCatM1 := f_IMS_TestcaseIsCatM1(); /* @sic R5-174483: C.21d sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributesAudioCodecListRX(p_AudioCodecList); /* @sic R5-168031, R5-168053, R5-168054, R5-172931 sic@ */ /* @sic R5-150709: cr_SDP_Attribute_inactive removed sic@ */ if (not v_IsWLAN and not v_IsCatM1) { /* Attributes for media security mechanism may be required for C.21 only */ v_SDP_MediaAttributes := f_IMS_SDP_MediaAttributes_AddMediaSecurityAttributesRX(v_SDP_MediaAttributes); /* @sic R5-160293, R5-160915 new TC H.12.3 sic@ */ } if (p_PreconditionsRequired) { /* @sic R5s150704, R5s150751 - Additional changes sic@ */ if (v_IsWLAN) { /* @sic R5-171476 - WLAN Annex C.21a sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); } else { v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, (c_optional, c_mandatory)); } } return f_IMS_BuildSDP_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes, -, v_BandwidthRS, v_BandwidthRR); }

# Branches Executed

The test case was executed on NR band n3 on Anritus and n41 on Keysight.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_7_25_LOGS.xml

Keysight\TC_7_25_LOGS.xml

- PICS/PIXIT parameter file:

Anritsu\TC_7_25_PIXIT.txt
Keysight\TC_7_25_PIXIT.txt

# References

| [1] | R5s210461: |
| --- | --- |