3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210463

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1692 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation for 7.1.4.1, In step 5 , RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND is sent by SS. In step 5 , cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig) is used, which is having Complete nonCriticalExtension IE set as Omit. [ each element inside nonCriticalExtension is left uninitialized ] While filling dedicatedNAS_MessageList in the altstep a_AspFromTestcaseHandler_NRNG , only dedicatedNAS_MessageList is getting assigned to NAS_DedicatedMsg nonCriticalExtension.dedicatedNAS_MessageList [I] := v_NAS_DedicatedMsg[I]; Because of the above assignment, only nonCriticalExtension.dedicatedNAS_MessageList is set with values where as other below IEs are left uninitilized . v_V1530Ext.fullConfig , v_V1530Ext.masterCellGroup , v_V1530Ext.masterKeyUpdate , v_V1530Ext.dedicatedSIB1_Delivery , v_V1530Ext.dedicatedSystemInformationDelivery , v_V1530Ext.nonCriticalExtension , v_V1530Ext.otherConfig This implementation is creating runtime issue only when reconfiguration is sent with NAS Message Piggybacking. Otherwise nonCriticalExtension altogether is set to Omit ; In case of sending Piggybacked Nas Request , this issue is exposed as dedicatedNAS_MessageList inside nonCriticalExtension is set , and other fields are left uninitialized |  |  |  |  |  |  |  |  |
| Summary of change: |  | Uninitilized IEs in nonCriticalExtension are set to Omit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_7_1_4_1_NR_TestBody	3

# Corrections required

## Correction to function fl_TC_7_1_4_1_NR_TestBody

| Function name | function fl_TC_7_1_4_1_NR_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation for 7.1.4.1, In step 5 , RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND is sent by SS. In step 5 , cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig) is used, which is having Complete nonCriticalExtension IE set as Omit. [ each element of nonCriticalExtension is left uninitialized ] While filling dedicatedNAS_MessageList in the altstep a_AspFromTestcaseHandler_NRNG , only dedicatedNAS_MessageList is getting assigned to NAS_DedicatedMsg nonCriticalExtension.dedicatedNAS_MessageList [I] := v_NAS_DedicatedMsg[I]; Because of the above assignment, only nonCriticalExtension.dedicatedNAS_MessageList is set with values where as other below IEs are left uninitilized . v_V1530Ext.fullConfig , v_V1530Ext.masterCellGroup , v_V1530Ext.masterKeyUpdate , v_V1530Ext.dedicatedSIB1_Delivery , v_V1530Ext.dedicatedSystemInformationDelivery , v_V1530Ext.nonCriticalExtension , v_V1530Ext.otherConfig This implementation is creating issue only when reconfiguration is sent with NAS Message Piggybacking. Otherwise nonCriticalExtension altogether is set to Omit , in case of sending Piggybacked Nas Request , this issue is exposed as dedicatedNAS_MessageList inside nonCriticalExtension is set , and other fields are left uninitialized |
| Summary of change | Uninitilized IEs in nonCriticalExtension are set to Omit |
| TTCN module | ttcn\develop\NR5GC\7_1_4\SDAP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB, integer p_SecondDRB, GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC { var template (present) SDAP_PDU_Type v_ExpectedPdu; var octetstring v_IPDataQFI1 := f_SdapTests_CreateDatagram("2"); // Packet with remote address matching filter #2 var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4 var octetstring v_IPDataQFI4 := f_SdapTests_CreateDatagram("3a"); // Packet with remote address matching filter #3a var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) RadioBearerConfig v_RadioBearerConfig; var integer v_PduSessionId := oct2int(p_GSM_MobilityInfo_Type.SessionId); // @sic R5s191119, R5s200564 sic@ var boolean v_PDU := false; // @sic R5s191119 sic@; boolean to indicate if the SDAP PDU has been received var boolean v_EndMarker := false; // @sic R5s191119 sic@; boolean to indicate if the End-Marker has been received var boolean v_RrcMsg := false; // @sic R5s191119 sic@; boolean to indicate if the RRC message has been received var boolean v_NasMsg := false; // @sic R5s191119 sic@; boolean to indicate if the NAS message has been received var NR_SRB_COMMON_IND v_ReceivedAsp; // @sic R5s191119 sic@ // @siclog "Step 1" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=1. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI1, v_IPDataQFI1); // @siclog "Step 2" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI1),v_IPDataQFI1); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5); // EXCEPTION: In parallel to the event described in step 4 the events specified in Table 7.1.4.1.3.2-2 shall take place. // @siclog "Table 7.1.4.1.3.2-2, Step 1" siclog@ // @siclog "Step 4" siclog@ v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI5),v_IPDataQFI5); // @sic R5s191119 sic@ do { alt { [not v_PDU] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})) { // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=5? v_PDU := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI5)})) { // Check: Does the UE transmits End-Marker Control PDU on DRB1 for QFI=5? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } } } while (not(v_PDU and v_EndMarker)); // @siclog "Step 5" siclog@ // The SS transmits an RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND // Note: As SDAP is in transparent mode, there is no need to reconfigure the SS v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {4}), // @sic R5s191119 sic@ p_SecondDRB, cs_38508_PDCP_Config(ms300))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, v_DRB_ToAddModList, omit, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand (p_GSM_MobilityInfo_Type.SessionId, -,-,-,-,-, cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("4a")}, '7A'O), // @sic R5s191119 sic@ cs_QoSFlowDescr ({cs_QoS_Flow2a})); SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NR_RRC_MSG_RequestDCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig)), {cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)})); // EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place. // @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@ // @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@ // @siclog "Step 6" siclog@ // @sic R5s191119 sic@ v_EndMarker := false; // Reset variable for reusing do { alt { [not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { // The UE transmits an RRCReconfigurationComplete message. v_RrcMsg := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)})) { // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } [not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // The UE Transmits PDU SESSION MODIFICATION COMPLETE if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) { v_NasMsg := true; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received"); } } } } while (not(v_RrcMsg and v_EndMarker and v_NasMsg)); // @siclog "Step 7" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4); // @siclog "Step 8" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }

After Change:

function fl_TC_7_1_4_1_NR_TestBody(integer p_FirstDRB, integer p_SecondDRB, GSM_MobilityInfo_Type p_GSM_MobilityInfo_Type) runs on NR_BASE_PTC { var template (present) SDAP_PDU_Type v_ExpectedPdu; var octetstring v_IPDataQFI1 := f_SdapTests_CreateDatagram("2"); // Packet with remote address matching filter #2 var octetstring v_IPDataQFI5 := f_SdapTests_CreateDatagram("4"); // Packet with remote address matching filter #4 var octetstring v_IPDataQFI4 := f_SdapTests_CreateDatagram("3a"); // Packet with remote address matching filter #3a var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) RadioBearerConfig v_RadioBearerConfig; var integer v_PduSessionId := oct2int(p_GSM_MobilityInfo_Type.SessionId); // @sic R5s191119, R5s200564 sic@ var boolean v_PDU := false; // @sic R5s191119 sic@; boolean to indicate if the SDAP PDU has been received var boolean v_EndMarker := false; // @sic R5s191119 sic@; boolean to indicate if the End-Marker has been received var boolean v_RrcMsg := false; // @sic R5s191119 sic@; boolean to indicate if the RRC message has been received var boolean v_NasMsg := false; // @sic R5s191119 sic@; boolean to indicate if the NAS message has been received var NR_SRB_COMMON_IND v_ReceivedAsp; // @sic R5s191119 sic@ var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; v_V1530Ext.fullConfig := omit; v_V1530Ext.masterCellGroup := omit; v_V1530Ext.masterKeyUpdate := omit; v_V1530Ext.dedicatedSIB1_Delivery := omit; v_V1530Ext.dedicatedSystemInformationDelivery := omit; v_V1530Ext.nonCriticalExtension := omit; v_V1530Ext.otherConfig := omit; v_V1530Ext.dedicatedNAS_MessageList := omit; // @siclog "Step 1" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=1. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI1, v_IPDataQFI1); // @siclog "Step 2" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping flow with QFI=1? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI1),v_IPDataQFI1); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=1, RQI=0, QFI=5. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI1, tsc_RQI0, tsc_QFI5, v_IPDataQFI5); // EXCEPTION: In parallel to the event described in step 4 the events specified in Table 7.1.4.1.3.2-2 shall take place. // @siclog "Table 7.1.4.1.3.2-2, Step 1" siclog@ // @siclog "Step 4" siclog@ v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI5),v_IPDataQFI5); // @sic R5s191119 sic@ do { alt { [not v_PDU] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})) { // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=5? v_PDU := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI5)})) { // Check: Does the UE transmits End-Marker Control PDU on DRB1 for QFI=5? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } } } while (not(v_PDU and v_EndMarker)); // @siclog "Step 5" siclog@ // The SS transmits an RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND // Note: As SDAP is in transparent mode, there is no need to reconfigure the SS v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {4}), // @sic R5s191119 sic@ p_SecondDRB, cs_38508_PDCP_Config(ms300))}; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, v_DRB_ToAddModList, omit, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master)); v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand (p_GSM_MobilityInfo_Type.SessionId, -,-,-,-,-, cs_QoS_Rules ({f_BuildQoSRuleRemoteAccess("4a")}, '7A'O), // @sic R5s191119 sic@ cs_QoSFlowDescr ({cs_QoS_Flow2a})); SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_Now, cs_NR_RRC_MSG_RequestDCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, v_RadioBearerConfig, -, - ,v_V1530Ext)), {cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)})); // EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place. // @siclog "Table 7.1.4.1.3.3-3, Step 1" siclog@ // @siclog "Table 7.1.4.1.3.3-4, Step 1" siclog@ // @siclog "Step 6" siclog@ // @sic R5s191119 sic@ v_EndMarker := false; // Reset variable for reusing do { alt { [not v_RrcMsg] SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) { // The UE transmits an RRCReconfigurationComplete message. v_RrcMsg := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [not v_EndMarker] DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_FirstDRB, -, {cr_SDAP_PDU_UL_EndMarker(tsc_QFI4)})) { // Check: Does the UE transmit End-Marker Control PDU on DRB 1 for QFI=4? v_EndMarker := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "End-Marker received"); } [not v_NasMsg] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // The UE Transmits PDU SESSION MODIFICATION COMPLETE if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg , cr_NG_PDU_SESSION_MODIFICATION_COMPLETE(?,?))) { v_NasMsg := true; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS message received"); } } } } while (not(v_RrcMsg and v_EndMarker and v_NasMsg)); // @siclog "Step 7" siclog@ // The SS sends the SDAP Data PDU with SDAP header on DRB 2 and the following content to the UE: RDI=0. RQI=0, QFI=4. f_SDAP_SendWithHeader(nr_Cell1, p_SecondDRB, tsc_RDI0, tsc_RQI0, tsc_QFI4, v_IPDataQFI4); // @siclog "Step 8" siclog@ // Check: Does the UE re-transmit SDAP Data PDU on DRB 2 with SDAP header as per the stored DRB mapping Flow with QFI=4? v_ExpectedPdu := cr_SDAP_PDU_UL_WithHeader(cr_SDAP_UL_PduHeader_Data(tsc_QFI4),v_IPDataQFI4); DRB.receive(car_NR_DRB_COMMON_IND_SDAP_PDUList(nr_Cell1, p_SecondDRB, -, {v_ExpectedPdu})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }