3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210482

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4603 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_UTRAN_InitializeSIB11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The function fl_UTRAN_InitializeSIB11returns UTRAN_CellInfoList_Type, which is at present a fixed length of tsc_UTRAN_NumberOfCells, which is set to value 8. Within this function variable var template (value) UTRAN_CellInfoList_Type v_CellList := p_CellList; is declared which is updated only for utran cells 5,7,8,and 9. Thus, by the time this variable (v_CellList) is returned, it contains 4 elements, leaving the other 4 elements uninitialized. But the TTCN expects the full list to be returned, that is value of tsc_UTRAN_NumberOfCells. Since 4 elements are un-initialized at the time of returning v_CellList , this causes a runtime error as this variable is being returned with un-initialized values. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of declaring v_CellList tobe of type template (value), declared it to be of type UTRAN_CellInfoList_Type. Also instead of assigninh v_CellList[x] := v_SysInfoType11;, assigned v_CellList[x] := valueof(v_SysInfoType11); Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | LTE-UTRAN testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to fl_UTRAN_InitializeSIB11	4

# Corrections required

## Correction to fl_UTRAN_InitializeSIB11

| Function name | fl_UTRAN_InitializeSIB11 |
| --- | --- |
| Reason for change | The function fl_UTRAN_InitializeSIB11returns UTRAN_CellInfoList_Type, which is at present a fixed length of tsc_UTRAN_NumberOfCells, which is set to value 8. Within this function variable var template (value) UTRAN_CellInfoList_Type v_CellList := p_CellList; is declared which is updated only for utran cells 5,7,8,and 9. Thus, by the time this variable (v_CellList) is returned, it contains 4 elements, leaving the other 4 elements uninitialized. But the TTCN expects the full list to be returned, that is value of tsc_UTRAN_NumberOfCells. Since 4 elements are un-initialized at the time of returning v_CellList , this causes a runtime error as this variable is being returned with un-initialized values. This needs to be corrected. |
| Summary of change | Instead of declaring v_CellList tobe of type template (value), declared it to be of type UTRAN_CellInfoList_Type. Also instead of assigninh v_CellList[x] := v_SysInfoType11;, assigned v_CellList[x] := valueof(v_SysInfoType11); Please see below. |
| TTCN module | UTRAN_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_UTRAN_InitializeSIB11(UTRAN_CellInfoList_Type p_CellList) return UTRAN_CellInfoList_Type { var template (value) SysInfoType11 v_SysInfoType11; var template (value) UTRAN_CellInfoList_Type v_CellList := p_CellList; var integer v_Index_Cell5 := f_UTRAN_CellInfo_GetIndex(utran_Cell5); var integer v_Index_Cell7 := f_UTRAN_CellInfo_GetIndex(utran_Cell7); var integer v_Index_Cell8 := f_UTRAN_CellInfo_GetIndex(utran_Cell8); var integer v_Index_Cell9 := f_UTRAN_CellInfo_GetIndex(utran_Cell9); v_SysInfoType11 := cs_SIB11_Def_FDD(5, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell5].sysInfo.SIB11 := v_SysInfoType11; v_SysInfoType11 := cs_SIB11_Def_FDD(7, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell7].sysInfo.SIB11 := v_SysInfoType11; v_SysInfoType11 := cs_SIB11_Freq2_FDD(8, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].frequencyInfo, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].frequencyInfo, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell8].sysInfo.SIB11 := v_SysInfoType11; v_SysInfoType11 := cs_SIB11_Freq2_FDD(9, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].frequencyInfo, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].frequencyInfo, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell9].sysInfo.SIB11 := v_SysInfoType11; return valueof(v_CellList); }

After Change:

function fl_UTRAN_InitializeSIB11(UTRAN_CellInfoList_Type p_CellList) return UTRAN_CellInfoList_Type { var template (value) SysInfoType11 v_SysInfoType11; var UTRAN_CellInfoList_Type v_CellList := p_CellList; var integer v_Index_Cell5 := f_UTRAN_CellInfo_GetIndex(utran_Cell5); var integer v_Index_Cell7 := f_UTRAN_CellInfo_GetIndex(utran_Cell7); var integer v_Index_Cell8 := f_UTRAN_CellInfo_GetIndex(utran_Cell8); var integer v_Index_Cell9 := f_UTRAN_CellInfo_GetIndex(utran_Cell9); v_SysInfoType11 := cs_SIB11_Def_FDD(5, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell5].sysInfo.SIB11 := valueof(v_SysInfoType11); v_SysInfoType11 := cs_SIB11_Def_FDD(7, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell7].sysInfo.SIB11 := valueof(v_SysInfoType11); v_SysInfoType11 := cs_SIB11_Freq2_FDD(8, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].frequencyInfo, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].frequencyInfo, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 9, p_CellList[v_Index_Cell9].frequencyInfo, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell8].sysInfo.SIB11 := valueof(v_SysInfoType11); v_SysInfoType11 := cs_SIB11_Freq2_FDD(9, p_CellList[v_Index_Cell9].cellInfo_Specific_FDD.priScrmCode, 5, p_CellList[v_Index_Cell5].frequencyInfo, p_CellList[v_Index_Cell5].cellInfo_Specific_FDD.priScrmCode, 7, p_CellList[v_Index_Cell7].frequencyInfo, p_CellList[v_Index_Cell7].cellInfo_Specific_FDD.priScrmCode, 8, p_CellList[v_Index_Cell8].frequencyInfo, p_CellList[v_Index_Cell8].cellInfo_Specific_FDD.priScrmCode); v_CellList[v_Index_Cell9].sysInfo.SIB11 := valueof(v_SysInfoType11); return (v_CellList); }