3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210502

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3661 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2021-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the currecnt TTCN implemenation the variable v_UE_CapabilityContainer_Add_Ext is declared, but not assigned any value. In case the UE does not include IE RRCConnectionSetupComplete_r3_add_ext_IEs in the rrcConnection SetupComplete message, this variable remains un-initialized. This causes runtime error when this variable is passed to function fl_UTRAN34_CheckCapabilityContainerAddExt. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of leaving v_UE_CapabilityContainer_Add_Ext un-initialized, assigned it OMIT. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the TC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1 fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete	3

## 1.1 fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete

| Object name | fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete |
| --- | --- |
| Reason for change | In the currecnt TTCN implemenation the variable v_UE_CapabilityContainer_Add_Ext is declared, but not assigned any value. In case the UE does not include IE RRCConnectionSetupComplete_r3_add_ext_IEs in the rrcConnection SetupComplete message, this variable remains un-initialized. This causes runtime error when this variable is passed to function fl_UTRAN34_CheckCapabilityContainerAddExt. This needs to be corrected. |
| Summary of change | Instead of leaving v_UE_CapabilityContainer_Add_Ext un-initialized, assigned it OMIT. Please see below. |
| Module | UTRAN34_UE_CapabilityInfo.ttcn |
| Label |  |

Before:

function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete(UTRAN_CellId_Type p_CellId, RRCConnectionSetupComplete p_RRCConnectionSetupComplete) runs on UTRAN_PTC { var UE_CapabilityContainer_IEs v_UE_CapabilityContainer; var bitstring v_UeCapabilityContainerBitstring; var bitstring v_UeCapabilityContainerAddExtBitstring; var RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_Ext; // Decode the UE-CapabilityContainer <<SKIPPED CODE>>

After:

function fl_UTRAN34_CheckAllCapabilitiesIn_RrcConnectionSetupComplete(UTRAN_CellId_Type p_CellId, RRCConnectionSetupComplete p_RRCConnectionSetupComplete) runs on UTRAN_PTC { var UE_CapabilityContainer_IEs v_UE_CapabilityContainer; var bitstring v_UeCapabilityContainerBitstring; var bitstring v_UeCapabilityContainerAddExtBitstring; var RRCConnectionSetupComplete_r3_add_ext_IEs v_UE_CapabilityContainer_Add_Ext := omit; // Decode the UE-CapabilityContainer <SKIPPED CODE>