3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210545

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4596 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_CheckUE_SupportedEUTRA_BandsExt |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-03-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current implementation of R5-205088 which is done in function f_CheckUE_SupportedEUTRA_BandsExt for bands 87-88 appears to be incorrect. f_CheckUE_SupportedEUTRA_BandsExt uses input parameters two,three and four to select a substring from the whole band list, indicate the position of the first band to be checked, respectively. In this case 87 and the number of positions to be checked, two, for band 87 and 88. As can be observed in ttcn3 code below: // appearance of bands 87-88 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 22, 1); // band 87 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 2); // @sic R5-205088 sic@ v_SuppEutraBandsExtAccToPICS_Chunk is of size one when later in f_CheckUE_SupportedEUTRA_BandsExt_Chunk as fourth parameter is “2”, position one is going to be accesed causing a runtime error as array is of length one when it should be of length two. An additional correction must be done for Bands 68-74, in this case no runtime error is seen but less fiels than required are checked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected bands to be checked for 68-74 and 87-88 chunks.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function f_CheckUE_SupportedEUTRA_BandsExt	4

# Corrections required

## Correction to function f_CheckUE_SupportedEUTRA_BandsExt

| Function name | f_CheckUE_SupportedEUTRA_BandsExt |
| --- | --- |
| Reason for change | v_SuppEutraBandsExtAccToPICS_Chunk is of size one when later in f_CheckUE_SupportedEUTRA_BandsExt_Chunk as fourth parameter is “2”, position one is going to be accesed causing a runtime error as array is of length one when it should be of length two. An additional correction must be done for Bands 68-74, in this case no runtime error is seen but less fiels than required are checked. |
| Summary of change | Corrected bands to be checked for 68-7 and 87-88 chunks.. |
| TTCN module | Common/IRAT/EUTRA_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> // Check that any value B such that pc_eBandB_Supp is TRUE and different from all eutra-Band[k] where k = 1 to a - 1 // Go through the received supportedBandListEUTRA and match against the supported bands according to the PICS // Check only the bands for which there are PICS; do not check other bands which may or may not be reported. // Done for all sets of continuous PICS. v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 0, 2); // bands 65-66 [0 = 65-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 0, 2); // @sic R5-162652, R5-163090 sic@ // appearance of bands 68-74 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 3, 7); // bands 68-74 [3 = 68-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 3, 5); // @sic R5-168397, R5-176304, R5-181163, R5-181164, R5-176295, R5-180920, R5-196883 sic@ // appearance of band 85 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 20, 1); // band 85 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 1); // @sic R5-193737 sic@ // appearance of bands 87-88 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 22, 1); // band 87 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 2); // @sic R5-205088 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // Check that any value B such that pc_eBandB_Supp is TRUE and different from all eutra-Band[k] where k = 1 to a - 1 // Go through the received supportedBandListEUTRA and match against the supported bands according to the PICS // Check only the bands for which there are PICS; do not check other bands which may or may not be reported. // Done for all sets of continuous PICS. v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 0, 2); // bands 65-66 [0 = 65-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 0, 2); // @sic R5-162652, R5-163090 sic@ // appearance of bands 68-74 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 3, 7); // bands 68-74 [3 = 68-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 3, 7); // @sic R5-168397, R5-176304, R5-181163, R5-181164, R5-176295, R5-180920, R5-196883 sic@ // appearance of band 85 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 20, 1); // band 85 [20 = 85-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 20, 1); // @sic R5-193737 sic@ // appearance of bands 87-88 v_SuppEutraBandsExtAccToPICS_Chunk := substr(v_SuppEutraBandsExtAccToPICS, 22, 2); // band 87 [22 = 87-maxFBI-1] f_CheckUE_SupportedEUTRA_BandsExt_Chunk(v_SupportedBandListEUTRA_v9e0, v_SuppEutraBandsExtAccToPICS_Chunk, 22, 2); // @sic R5-205088 sic@ <<SKIPPED CODE>>