3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210552

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1613 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_RRC_SetupRequest_Common |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per clause 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases, and in the TTCN code, the TBS of UL grant is set to 4992. If the UE has long UE information (> 4992 *2), then the UE needs to send three SRs to ask for enough UL grant. But the duration will be 10.240s * 3 = 30.72s, the Timer T3510 will expire before all UE information is sent out. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase the TBS of UL grant from 4992 to 12808 to make sure all UE information will be sent out. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All layer 2 test cases, which set the value of retxBSR-Timer to default sf10240. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

# Corrections required

| Function name | f_NR_RRC_SetupRequest_Common |
| --- | --- |
| Reason for change | As per clause 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases, and in the TTCN code, the TBS of UL grant is set to 4992. If the UE has long UE information (> 4992 *2), then the UE needs to send three SRs to ask for enough UL grant. But the duration will be 10.240s * 3 = 30.72s, the Timer T3510 will expire before all UE information is sent out. |
| Summary of change | Increase the TBS of UL grant from 4992 to 12808 to make sure all UE information will be sent out. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before change:

…

function f_NR_RRC_SetupRequest_Common(NR_CellId_Type p_NR_CellId,

timer p_Timer,

template  EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ?)

runs on NR_BASE_PTC return boolean

{ /* common procedure to receive the initial RRC setup request for an expected RRC connection establishment procedure;

when RRC setup is not explicitly sent from TTCN but is preconfigured at the SS to be sent together with the contention resolution,

the default UL grant configuration is configured at the SS */

var boolean v_TimerIsRunning := p_Timer.running;

var template (omit)  NR_UplinkTimeAlignment_Synch_Type v_PUCCH_Synch;

if (not f_NR_EarlyContentionResolution()) {

v_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Stop;  /* No automatic UL time alignment:

if the SS would send MAC PDU with TA while the waits for contention resolution, it will consider contention resolutions as being fail

=> in case of automatic handling of the RRCConnectionSetup in Msg4 (RRCConnectionSetup) UL time alignment shall be started not before RRCConnectionSetupComplete has been received */

f_NR_ULGrantConfiguration_Start(p_NR_CellId, -, -, -, v_PUCCH_Synch, -, -, tsc_NoCnfReq ); //@sic R5s200161 sic@

} // else: UL grant configuration is done just before sending of RRCSetup

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(p_EstablishmentCause, p_InitialUE_Identity)))

{

p_Timer.stop;       /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */

return true;

}

[v_TimerIsRunning] p_Timer.timeout

{

}

}

return false;

}

....

After change:

…

function f_NR_RRC_SetupRequest_Common(NR_CellId_Type p_NR_CellId,

timer p_Timer,

template  EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ?)

runs on NR_BASE_PTC return boolean

{ /* common procedure to receive the initial RRC setup request for an expected RRC connection establishment procedure;

when RRC setup is not explicitly sent from TTCN but is preconfigured at the SS to be sent together with the contention resolution,

the default UL grant configuration is configured at the SS */

var boolean v_TimerIsRunning := p_Timer.running;

var template (omit)  NR_UplinkTimeAlignment_Synch_Type v_PUCCH_Synch;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, tsc_NR_BWP_Id);

if (not f_NR_EarlyContentionResolution()) {

v_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Stop;  /* No automatic UL time alignment:

if the SS would send MAC PDU with TA while the waits for contention resolution, it will consider contention resolutions as being fail

=> in case of automatic handling of the RRCConnectionSetup in Msg4 (RRCConnectionSetup) UL time alignment shall be started not before RRCConnectionSetupComplete has been received */

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(12808, v_NR_UplinkBWP);

f_NR_ULGrantConfiguration_Start(p_NR_CellId,

-,

v_NR_ResourceAllocation.Imcs,

cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),

v_PUCCH_Synch,

-,

-,

tsc_NoCnfReq ); //@sic R5s200161 sic@

} // else: UL grant configuration is done just before sending of RRCSetup

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(p_EstablishmentCause, p_InitialUE_Identity)))

{

p_Timer.stop;       /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */

return true;

}

[v_TimerIsRunning] p_Timer.timeout

{

}

}

return false;

} …