3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210555

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0676 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.10 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.10 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

4.1	Qualcomm SM8350 + SDX60	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.10 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.10

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes of R5s210074, R5s210075 and R5s210330 are required and applied.

## 2.1 Change 1

| Function name | f_TC_7_10_NR5GC_IMS1() |
| --- | --- |
| Reason for change | - To start the signalling for MT call sequence, f_IMS_CC_StartCall() should be used instead of f_IMS_CC_StartSignalling() - v_InviteRequestWithSdp is not Initialized with the received INVITE request. - Voice call bearer establishment is missing prior to step 7. |
| Summary of change | - Replaced f_IMS_CC_StartSignalling() with f_IMS_CC_StartCall() to establish MT call. - v_InviteRequestWithSdp is initialized. - Added call to f_IMS_CC_TriggerDedicatedBearerActivation tp trigger IPCAN procedures before Step 7 |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_10_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; var boolean v_PreconditionsRequired := false; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartSignalling(IPCAN_MT_SpeechCall); // @siclog "Step 1" siclog@ Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 // @siclog "Step 2" siclog@ INVITE v_AudioCodecList := {}; // => no SDP f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(v_PreconditionsRequired, v_AudioCodecList); // !!!! PROSE ISSUE: Table 7.10.3.3-1 needs the Content-Type to be specified as not present !!!! // @siclog "Step 3 - 4" siclog@ 100 Trying, 183 Session Progress v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_RX(v_PreconditionsRequired, v_AudioCodecList); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol); f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ PRACK v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_ResponseWithSdp.SdpMessage, v_PreconditionsRequired, EVS_16000, 65); v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_MessageHeader_Prack := f_IMS_PrackRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader, cs_ContentTypeSDP); IMS_Client.send(cas_IMS_Prack_Request(cs_IMS_RoutingInfo(v_Protocol), cs_PRACK_Request(v_ContactUrl, v_MessageHeader_Prack, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 6" siclog@ 200 OK a_IMS_ReceiveResponse200OK(v_MessageHeader_Prack, v_Protocol); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7 - 9" siclog@ 180 Ringing, PRACK (conditional), 200 OK (conditional) f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 10A - 11" siclog@ MMI (Accept Call), 200 OK, ACK f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); f_IMS_TestBody_Set(false); // Call release and postamble: fl_NR5GC_CallControl_Postamble(IPCAN_MT_SpeechCall); }

After Change:

function f_TC_7_10_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; var boolean v_PreconditionsRequired := false; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @siclog "Step 1" siclog@ Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 // @siclog "Step 2" siclog@ INVITE v_AudioCodecList := {}; // => no SDP v_InviteRequestWithSdp:= f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(v_PreconditionsRequired, v_AudioCodecList); // !!!! PROSE ISSUE: Table 7.10.3.3-1 needs the Content-Type to be specified as not present !!!! // @siclog "Step 3 - 4" siclog@ 100 Trying, 183 Session Progress v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_RX(v_PreconditionsRequired, v_AudioCodecList); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol); f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ PRACK v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_ResponseWithSdp.SdpMessage, v_PreconditionsRequired, EVS_16000, 65); v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_MessageHeader_Prack := f_IMS_PrackRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader, cs_ContentTypeSDP); IMS_Client.send(cas_IMS_Prack_Request(cs_IMS_RoutingInfo(v_Protocol), cs_PRACK_Request(v_ContactUrl, v_MessageHeader_Prack, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 6" siclog@ 200 OK a_IMS_ReceiveResponse200OK(v_MessageHeader_Prack, v_Protocol); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); // @siclog "Step 7 - 9" siclog@ 180 Ringing, PRACK (conditional), 200 OK (conditional) f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // @siclog "Step 10A - 11" siclog@ MMI (Accept Call), 200 OK, ACK f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); f_IMS_TestBody_Set(false); // Call release and postamble: fl_NR5GC_CallControl_Postamble(IPCAN_MT_SpeechCall); }

# Branches Executed

The test case was executed on NR bands n3 and n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_10_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_10_PIXIT.xml

The Qualcomm SM8350 + SDX60 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_10_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_7_10_PIXIT.txt

# References

| [1] | R5s210327 : Supporting information for addition of IMS 5GS call control test case 7.10 |
| --- | --- |