3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210570

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1638 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Summary of regression errors in 21wk12 TTCN for NR testcases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following TTCN issues are observed during testing with 21wk12 IWD for NR testcases. Please see details of individual issues below. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented TTCN corrections as described in individual changes needed. Please see below for detailed description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ALL NR testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1	f_EUTRA38_GetNextDRBIds	3

1.2	cads_NR_ULGrantAllocation_REQ	5

1.3	f_NR_RRC_CipherActTime_GetCurrent	7

## f_EUTRA38_GetNextDRBIds

| Function name | f_EUTRA38_GetNextDRBIds |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the variable v_ExistingDRBs is not initialized. If the condition v_NoOfExistingDRBs > 0 is not met, then this variable remains uninitialized and results in a runtime error. This needs to be adddressed. |
| Summary of change | Initialized variable v_ExistingDRBs := {}. Please see below. |
| TTCN module | Eutra38_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_GetNextDRBIds (PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB, integer p_NoOfDRBs := 1) runs on EUTRA_Multi5GS_PTC return IntegerList_Type { var integer v_NoOfExistingDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var integer v_DRBId := 0; var IntegerList_Type v_DRBList; var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var integer v_Index; var boolean v_AlreadyIMS_DRB := false; var integer i := 0; var integer j := 0; if (v_NoOfExistingDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, tsc_EUTRA38_IMS_DRB); if (v_Index >= 0) { // IMS is already allocated v_AlreadyIMS_DRB := true; } } if ((p_PDN_Type == IMS_DNN) and p_DefaultDRB) { v_DRBList[j] := tsc_EUTRA38_IMS_DRB; j := j+1; } while (j < p_NoOfDRBs) { if (v_AlreadyIMS_DRB) { v_DRBId := v_NoOfExistingDRBs + (2-1); // allocate the next one, from 2 onwards } else { // IMS is not yet allocated v_DRBId := v_NoOfExistingDRBs + 2; } i := 0; while (i < lengthof(v_ExistingDRBs)) { // @sic R5s210305 sic@ // Already allocated this DRB (due to bearers transferred from NR, so increment and try again ) if (match (v_ExistingDRBs[i].DRB_Id, v_DRBId)) { v_DRBId := v_DRBId + 1; i := -1; // as it's about to be incremented } i := i + 1; } v_DRBList[j] := v_DRBId; j := j+1; // now update number of DRBs whilst in the loop v_NoOfExistingDRBs := v_NoOfExistingDRBs +1; } return v_DRBList; }

After Change:

function f_EUTRA38_GetNextDRBIds (PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB, integer p_NoOfDRBs := 1) runs on EUTRA_Multi5GS_PTC return IntegerList_Type { var integer v_NoOfExistingDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var integer v_DRBId := 0; var IntegerList_Type v_DRBList; var EUTRA38_DRBInfoList_Type v_ExistingDRBs := {}; //WA#21wk12 var integer v_Index; var boolean v_AlreadyIMS_DRB := false; var integer i := 0; var integer j := 0; if (v_NoOfExistingDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, tsc_EUTRA38_IMS_DRB); if (v_Index >= 0) { // IMS is already allocated v_AlreadyIMS_DRB := true; } } if ((p_PDN_Type == IMS_DNN) and p_DefaultDRB) { v_DRBList[j] := tsc_EUTRA38_IMS_DRB; j := j+1; } while (j < p_NoOfDRBs) { if (v_AlreadyIMS_DRB) { v_DRBId := v_NoOfExistingDRBs + (2-1); // allocate the next one, from 2 onwards } else { // IMS is not yet allocated v_DRBId := v_NoOfExistingDRBs + 2; } i := 0; while (i < lengthof(v_ExistingDRBs)) { // @sic R5s210305 sic@ // Already allocated this DRB (due to bearers transferred from NR, so increment and try again ) if (match (v_ExistingDRBs[i].DRB_Id, v_DRBId)) { v_DRBId := v_DRBId + 1; i := -1; // as it's about to be incremented } i := i + 1; } v_DRBList[j] := v_DRBId; j := j+1; // now update number of DRBs whilst in the loop v_NoOfExistingDRBs := v_NoOfExistingDRBs +1; } return v_DRBList; }

## cads_NR_ULGrantAllocation_REQ

| Function name | cads_NR_ULGrantAllocation_REQ |
| --- | --- |
| Reason for change | In the current TTCN implementation of this template, IEs AssignedBWPs and SearchSpaceType are not initialized. This causes runtime errors. TTCN CR R5s210263 was raised for this earlier. This needs to be adddressed. |
| Summary of change | IEs AssignedBWPs and SearchSpaceType are initialized and set to OMIT. Please see below. |
| TTCN module | NR_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_ULGrantAllocation_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_DciUlInfo_Type p_DciUlInfo := omit, template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := omit, template (omit) UL_GrantConfig_Type p_UL_GrantConfig := omit, template (omit) NR_ServingCellConfig_Type p_NR_ServingCellConfig := omit, template (omit) boolean p_CnfFlag := omit ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5s210263 added omit IEs DL, DrxCtrl and MeasGapCtrl sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag), //@sic R5s200161 sic@ Request := { Cell := { AddOrReconfigure := { DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { DciInfo := p_DciUlInfo }, PUCCH_Synch := p_PUCCH_Synch, GrantConfig := p_UL_GrantConfig }, DrxCtrl := omit, MeasGapCtrl := omit }, ServingCellConfig := p_NR_ServingCellConfig } } } };

After Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_ULGrantAllocation_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (omit) NR_DciUlInfo_Type p_DciUlInfo := omit, template (omit) NR_UplinkTimeAlignment_Synch_Type p_PUCCH_Synch := omit, template (omit) UL_GrantConfig_Type p_UL_GrantConfig := omit, template (omit) NR_ServingCellConfig_Type p_NR_ServingCellConfig := omit, template (omit) boolean p_CnfFlag := omit ) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //@sic R5s210263 added omit IEs DL, DrxCtrl and MeasGapCtrl sic@ Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, p_CnfFlag), //@sic R5s200161 sic@ Request := { Cell := { AddOrReconfigure := { DcchDtchConfig := { DL := omit, UL := { SearchSpaceAndDci := { AssignedBWPs := omit, // WA#21wk12 fields are not initialised and results in runtime error (See R5s210263) SearchSpaceType := omit, // WA#21wk12 fields are not initialised and results in runtime error (See R5s210263) DciInfo := p_DciUlInfo }, PUCCH_Synch := p_PUCCH_Synch, GrantConfig := p_UL_GrantConfig }, DrxCtrl := omit, MeasGapCtrl := omit }, ServingCellConfig := p_NR_ServingCellConfig } } } };

## f_NR_RRC_CipherActTime_GetCurrent

| Function name | f_NR_RRC_CipherActTime_GetCurrent |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, the variable v_SecurityActTimeList is not initialized. If the condition I < lengthof(v_PdcpCountInfoList) is not met, then this variable remains uninitialized and results in a runtime error. This needs to be adddressed. |
| Summary of change | Initialized variable v_SecurityActTimeList := {}. Please see below. |
| TTCN module | NR_SecuritySteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RRC_CipherActTime_GetCurrent(NR_CellId_Type p_NR_CellId, boolean p_IncrSRB1, boolean p_IncrSRB1_DL := false ) runs on NR_BASE_PTC return NR_SecurityActTimeList_Type { var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; var NR_PdcpCountInfo_Type v_PdcpCountInfo; var NR_PDCP_ActTime_Type v_PDCP_ActTimeUL; var NR_PDCP_ActTime_Type v_PDCP_ActTimeDL; var NR_RadioBearerId_Type v_RadioBearerId; var NR_SecurityActTime_Type v_SecurityActTime; var integer I; var NR_SecurityActTimeList_Type v_SecurityActTimeList; v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_NR_CellId); for (I:=0; I < lengthof(v_PdcpCountInfoList); I:=I+1) { v_PdcpCountInfo := v_PdcpCountInfoList[I]; if ((not ispresent(v_PdcpCountInfo.UL)) or (not ispresent(v_PdcpCountInfo.DL))) { FatalError(__FILE__, __LINE__, "missing PdcpCountInfo for UL or DL"); } v_RadioBearerId := v_PdcpCountInfo.RadioBearerId; // Convert PdcpCount to integer value (UL an DL) v_PDCP_ActTimeUL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.UL); v_PDCP_ActTimeDL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.DL); // increment SQN for SRB1 (UL and DL) if (p_IncrSRB1 and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { // @sic R5s190214 sic@ v_PDCP_ActTimeUL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeUL.SQN); v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // When transmitting RRCReestablishment to UE on DL SRB1 ciphering is not applied // When receiving RRCReestablishemntComplete on UL SRB1 ciphering is applied by UE. // So the new ciphering keys needs to be applied at SN+1 in DL and at SN in UL => increment DL SN only // @sic R5s201221 sic@ if ((p_IncrSRB1_DL) and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // set activation time v_SecurityActTime.RadioBearerId := v_RadioBearerId; v_SecurityActTime.UL := v_PDCP_ActTimeUL; v_SecurityActTime.DL := v_PDCP_ActTimeDL; v_SecurityActTimeList[I] := v_SecurityActTime; } return v_SecurityActTimeList; }

After Change:

function f_NR_RRC_CipherActTime_GetCurrent(NR_CellId_Type p_NR_CellId, boolean p_IncrSRB1, boolean p_IncrSRB1_DL := false ) runs on NR_BASE_PTC return NR_SecurityActTimeList_Type { var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; var NR_PdcpCountInfo_Type v_PdcpCountInfo; var NR_PDCP_ActTime_Type v_PDCP_ActTimeUL; var NR_PDCP_ActTime_Type v_PDCP_ActTimeDL; var NR_RadioBearerId_Type v_RadioBearerId; var NR_SecurityActTime_Type v_SecurityActTime; var integer I; var NR_SecurityActTimeList_Type v_SecurityActTimeList := {}; //WA_21wk12 v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_NR_CellId); for (I:=0; I < lengthof(v_PdcpCountInfoList); I:=I+1) { v_PdcpCountInfo := v_PdcpCountInfoList[I]; if ((not ispresent(v_PdcpCountInfo.UL)) or (not ispresent(v_PdcpCountInfo.DL))) { FatalError(__FILE__, __LINE__, "missing PdcpCountInfo for UL or DL"); } v_RadioBearerId := v_PdcpCountInfo.RadioBearerId; // Convert PdcpCount to integer value (UL an DL) v_PDCP_ActTimeUL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.UL); v_PDCP_ActTimeDL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.DL); // increment SQN for SRB1 (UL and DL) if (p_IncrSRB1 and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { // @sic R5s190214 sic@ v_PDCP_ActTimeUL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeUL.SQN); v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // When transmitting RRCReestablishment to UE on DL SRB1 ciphering is not applied // When receiving RRCReestablishemntComplete on UL SRB1 ciphering is applied by UE. // So the new ciphering keys needs to be applied at SN+1 in DL and at SN in UL => increment DL SN only // @sic R5s201221 sic@ if ((p_IncrSRB1_DL) and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // set activation time v_SecurityActTime.RadioBearerId := v_RadioBearerId; v_SecurityActTime.UL := v_PDCP_ActTimeUL; v_SecurityActTime.DL := v_PDCP_ActTimeDL; v_SecurityActTimeList[I] := v_SecurityActTime; } return v_SecurityActTimeList; }