3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210574

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1724 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.4.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the step 5, PCell is changed via reconfigurationWithSync and a same SCell is added. But according to TS 38.331, Cl 5.3.5.5.9 the sCellIndex shall be not part of current UE configuration for SCell addition. Otherwise, it shall be released first, so the sCellIndex needs to be present in the sCellToReleaseList. The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “sCellToReleaseList” to the reconfiguration message if source and target SCell are the same. Note: an associated prose CR on 38.523-1 will be raised at RAN5#91-e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell() |
| --- | --- |
| Reason for change | In the step 5, PCell is changed via reconfigurationWithSync and a same SCell is added. But according to TS 38.331, Cl 5.3.5.5.9 the sCellIndex shall be not part of current UE configuration for SCell addition. Otherwise, it shall be released first, so the sCellIndex needs to be present in the sCellToReleaseList. The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; |
| Summary of change | Add “sCellToReleaseList” to the reconfiguration message if source and target SCell are the same. Note : An associated prose CR on 38.523-1 will br raised at RAN#91 |
| TTCN module | NR5GC_Handover |
| MCC160 Comment |  |

Before Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetPCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; var template (omit) CellGroupConfig.sCellToAddModList v_SCellToAddModList := omit; var template (omit) CellGroupConfig.sCellToReleaseList v_SCellToReleaseList := omit; var template (omit) CellGroupConfig v_SCGConfig_SCell := omit;//@sic R5s201249 sic@ if (isvalue(p_MasterKeyUpdate)){ v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount); } else { v_NCC := tsc_NR_38508_NextHopChainingCount; } if (isvalue(p_TargetSCellId)) { //Set sCellToAddModList with p_TargetSCellId info v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(valueof(p_TargetSCellId),tsc_SCellIndex_1);//@sic R5s201249 sic@ v_SCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; if (p_SourceSCellId == p_TargetPCellId) { //PCell becomes SCell v_SCellToReleaseList := {tsc_SCellIndex_1}; //@sic R5s210079 sic@ } } else { // Release the p_SourceSCellId v_SCellToReleaseList := {tsc_SCellIndex_1}; } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetPCellId, p_RNTI_Value, p_RBConfig_KeyChange, -, -, -, p_MasterKeyUpdate, -, -, v_SCellToAddModList, v_SCellToReleaseList ); //Steps 1 to 18.1 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(p_SourcePCellId, p_SourceSCellId, p_TargetPCellId, p_TargetSCellId, v_SCGConfig_SCell, p_RNTI_Value, p_RBConfig_KeyChange, v_RRCReconfiguration, v_RachProcedureConfig, v_DRBInfoList, -, -, -, v_NCC, - ); }

After Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell(NR_CellId_Type p_SourcePCellId, NR_CellId_Type p_SourceSCellId, NR_CellId_Type p_TargetPCellId, template(omit) NR_CellId_Type p_TargetSCellId, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, boolean p_RBConfig_KeyChange := true, template (omit) MasterKeyUpdate p_MasterKeyUpdate := omit ) runs on NR5GC_PTC { var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetPCellId); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var NextHopChainingCount v_NCC; var template (omit) CellGroupConfig.sCellToAddModList v_SCellToAddModList := omit; var template (omit) CellGroupConfig.sCellToReleaseList v_SCellToReleaseList := omit; var template (omit) CellGroupConfig v_SCGConfig_SCell := omit;//@sic R5s201249 sic@ if (isvalue(p_MasterKeyUpdate)){ v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount); } else { v_NCC := tsc_NR_38508_NextHopChainingCount; } if (isvalue(p_TargetSCellId)) { //Set sCellToAddModList with p_TargetSCellId info v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(valueof(p_TargetSCellId),tsc_SCellIndex_1);//@sic R5s201249 sic@ v_SCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; if (p_SourceSCellId == p_TargetPCellId or p_SourceSCellId == p_TargetSCellId) { //PCell becomes SCell v_SCellToReleaseList := {tsc_SCellIndex_1}; //@sic R5s210079 sic@ } } else { // Release the p_SourceSCellId v_SCellToReleaseList := {tsc_SCellIndex_1}; } v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetPCellId, p_RNTI_Value, p_RBConfig_KeyChange, -, -, -, p_MasterKeyUpdate, -, -, v_SCellToAddModList, v_SCellToReleaseList ); //Steps 1 to 18.1 f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common(p_SourcePCellId, p_SourceSCellId, p_TargetPCellId, p_TargetSCellId, v_SCGConfig_SCell, p_RNTI_Value, p_RBConfig_KeyChange, v_RRCReconfiguration, v_RachProcedureConfig, v_DRBInfoList, -, -, -, v_NCC, - ); }