3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210578

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1729 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR_SendRRCResume_Def |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_NR_SendRRCResume_Def() 1) Resume peer message is sent in the middle of local end procedures , and there are occasions where the UE responds back with Resume Complete Peer message, even before the other local configuration procedure are completed and leads to the test case failure due to unexpected SRB message received error 2) The Rach configuration provided in the function is redundant ,and can be removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Resume Peer message transmission is moved at end of the function f_NR_SendRRCResume_Def() 2) Redundant Rach configuration is removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1.1.NR5GC, 8.1.1.4.2.NR5GC, 8.1.3.1.23.NR5GC, 8.1.5.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_NR_SendRRCResume_Def()	3

# Corrections required

## Correction to function f_NR_SendRRCResume_Def()

| Function name | function f_NR_SendRRCResume_Def() |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_NR_SendRRCResume_Def() 1) Resume peer message is sent in the middle of local end procedures , and there are occasions where the UE responds back with Resume Complete Peer message, even before the other local configuration procedure completed and leads to the test case failure due to unexpected SRB message received error 2) The Rach configuration provided in the function is redundant ,and can be removed |
| Summary of change | 1) Resume Peer message transmission is moved at end of the function f_NR_SendRRCResume_Def() 2) Redundant Rach configuration is removed |
| TTCN module | Common\NR_5GC\ NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template(omit) CellGroupConfig p_CellGroupConfig := omit, template(omit) RadioBearerConfig p_RadioBearerConfig := omit, template(omit) MeasConfig p_MeasConfig := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template(value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_NR_CellId); var template(omit) octetstring v_EncodedSCG_Config := omit; var template(value) CellGroupConfig v_CellGroupConfig; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; // switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters // @sic R5s190505 change 1 sic@ if (isvalue (p_CellGroupConfig)) { v_EncodedSCG_Config := bit2oct(encvalue(p_CellGroupConfig)); } else { v_CellGroupConfig := f_NR_GetSCGConfigResume(p_NR_CellId); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); } if ((isvalue (p_RadioBearerConfig))) { v_RadioBearerConfig := p_RadioBearerConfig; } else { //Set RadioBearerConfig as per 38.508-1 Table 4.6.3-132: RadioBearerConfig with condition RESUME //@sic R5-204386 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); } SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, cs_38508_RRCResume(p_RRC_TI, v_EncodedSCG_Config, v_RadioBearerConfig, p_MeasConfig))); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedureConfig); }

After Change:

function f_NR_SendRRCResume_Def( NR_CellId_Type p_NR_CellId, template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template(omit) CellGroupConfig p_CellGroupConfig := omit, template(omit) RadioBearerConfig p_RadioBearerConfig := omit, template(omit) MeasConfig p_MeasConfig := omit ) runs on NR5GC_PTC { //@sic R5-201646: added p_RadioBearerConfig sic@ //@sic R5-204441: added p_MeasConfig sic@ var NR_DRBInfoList_Type v_ExistingDRBs := f_NR5GC_MobileInfo_GetDRBInfoList(); var template(value) NR_RadioBearerList_Type v_SS_Config := f_NR5GC_GetRadioBearerList(v_ExistingDRBs); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(p_NR_CellId); var template(omit) octetstring v_EncodedSCG_Config := omit; var template(value) CellGroupConfig v_CellGroupConfig; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(omit) DRB_ToAddModList v_DRB_ToAddModList := omit; var integer i; // switch on UL grant assignments f_NR_ULGrantConfiguration_Start(p_NR_CellId); // refresh security keys and activate security => moved to the function bringing UE to Inactive state to give time to SS to configure security parameters // @sic R5s190505 change 1 sic@ if (isvalue (p_CellGroupConfig)) { v_EncodedSCG_Config := bit2oct(encvalue(p_CellGroupConfig)); } else { v_CellGroupConfig := f_NR_GetSCGConfigResume(p_NR_CellId); v_EncodedSCG_Config := bit2oct(encvalue(v_CellGroupConfig)); } if ((isvalue (p_RadioBearerConfig))) { v_RadioBearerConfig := p_RadioBearerConfig; } else { //Set RadioBearerConfig as per 38.508-1 Table 4.6.3-132: RadioBearerConfig with condition RESUME //@sic R5-204386 sic@ for (i:= 0; i < lengthof(v_ExistingDRBs); i :=i+1) { v_DRB_ToAddModList[i] := cs_NR_DRB_ToAddMod(omit, v_ExistingDRBs[i].DRB_Id, true_, -, omit); } v_RadioBearerConfig := cs_38508_RadioBearerConfigDef( {cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}, v_DRB_ToAddModList); } //SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, cs_38508_RRCResume(p_RRC_TI, v_EncodedSCG_Config, v_RadioBearerConfig, p_MeasConfig))); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ //Reconfigure DRB in SS - existing DRBs are not reconfigured during release f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Config, p_TimingInfo); //Activate security on newly configured DRB @sic R5s200102 sic@ f_NR_SS_AS_ActivateSecurity_NoRefresh(p_NR_CellId, p_TimingInfo); //f_NR_SS_RachProcedureConfig(p_NR_CellId, v_NR_RachProcedureConfig); //Duplicate Configuration SRB.send(cas_NR_SRB1_RrcPdu_REQ(p_NR_CellId, p_TimingInfo, cs_38508_RRCResume(p_RRC_TI, v_EncodedSCG_Config, v_RadioBearerConfig, p_MeasConfig))); //@sic R5-204441, R5s191051: added p_CellGroupConfig as a temporary solution sic@ }