3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210581

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1732 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_6_4_2_1_NR5GC_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In preamble, the security context activation for this case is not required, as during test case body the resume procedure is not activated (steps 3 /4). The UE is checked to see if it responds to paging; therefore there is no need to configure security and later release it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed security activation in preamble and also removed redundant security release and reset of DRB/SRBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Redundant SS reconfiguration procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required to test case 6.4.2.1	3

1.1	Correction to function f_TC_6_4_2_1_NR5GC	3

1.2	function f_TC_6_4_2_1_NR5GC_TestBody	4

# Corrections required to test case 6.4.2.1

## Correction to function f_TC_6_4_2_1_NR5GC

| Function name | f_TC_6_4_2_1_NR5GC |
| --- | --- |
| Reason for change | In preamble, the security context activation for this case is not required, as during test case body the resume procedure is not activated (steps 3 /4). The UE is checked to see if it responds to paging; therefore there is no need to configure security and later release it. |
| Summary of change | Set security activation as false. |
| TTCN module | NR5GC\6_4_2\Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_2_1_NR5GC() runs on NR5GC_PTC {// Cell Selection/Qrxlevmin & Cell Reselection (Intra NR in RRC_INACTIVE state) // @sic R5s200377 sic@ f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. // Table 6.4.2.1.3.3-1: SIB3 of NR Cell 1 f_NR5GC_CellInfo_SetSIB3(nr_Cell1, cs_38508_SIB3_Def({cs_IntraFreqNeighCellInfoDef(f_NR_CellInfo_GetPhysicalCellId(nr_Cell2), dB0)})); // @sic R5s200537 sic@ //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -78, -78); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2 f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A); f_NR_TestBody_Set(true); f_TC_6_4_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell2, STATE_INACTIVE_2A); // @sic R5s201000 sic@ } //end of f_TC_6_4_2_1_NR5GC

After Change:

function f_TC_6_4_2_1_NR5GC() runs on NR5GC_PTC {// Cell Selection/Qrxlevmin & Cell Reselection (Intra NR in RRC_INACTIVE state) // @sic R5s200377 sic@ f_NR5GC_Init(NR_3); // System information combination NR-3 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. // Table 6.4.2.1.3.3-1: SIB3 of NR Cell 1 f_NR5GC_CellInfo_SetSIB3(nr_Cell1, cs_38508_SIB3_Def({cs_IntraFreqNeighCellInfoDef(f_NR_CellInfo_GetPhysicalCellId(nr_Cell2), dB0)})); // @sic R5s200537 sic@ //Set maximum cell power level for the Cells to be used f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -78, -78); // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2 f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false); f_NR_TestBody_Set(true); f_TC_6_4_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell2, STATE_INACTIVE_2A); // @sic R5s201000 sic@ } //end of f_TC_6_4_2_1_NR5GC <<SKIPPED CODE>>

## function f_TC_6_4_2_1_NR5GC_TestBody

| Function name | f_TC_6_4_2_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | Based on Change 1 the security context will not be established and the reset of RB is already handled in the preamble. Therefore the configuration for releasing the security context and resetting the DRBs can be removed. |
| Summary of change | REMOVE f_NR_SS_RRC_ReleaseSecurity REMOVE f_NR_SS_SRBs_DRBs_Release REMOVE f_NR_SS_SRBs_DRBs_Config |
| TTCN module | NR5GC\6_4_2\Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 sic@ //FR2 Power levels are FFS ….. // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); f_NR_SetCellPowerList(v_CellPowerList_AtT0); <<SKIPPED CODE>>

After Change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 sic@ //FR2 Power levels are FFS ….. // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. //REMOVE f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now); //REMOVE f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //REMOVE f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); f_NR_SetCellPowerList(v_CellPowerList_AtT0); <<SKIPPED CODE>>