3GPP TSG-RAN5 Meeting #2020-TTCN email	R5s210585

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4612 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_1_6_2_EUTRA for Cat-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently Test case 7.1.6.2 fails for a cat-M UE due to conflict in DCI transmission of step 3 and PDSCH transmission of step 1. For ex: Step 1 DCI transmission is at 449/7/1 for which PDSCH transmission is at 449/9/1 which is conflicting with Step 3 DCI transmission which is also at 449/9/1.This causes delay in PDSCH transmission for step 3 PDU(at 450/2/1) resulting in delay of HARQ feedback from UE later at step 4 and test case fails. Same is the case with step 7 PDSCH transmission and Step 9 DCI transmission. Reference collison handling R1-157885. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the value of tsc_SubFrameY=2 to 3 and tsc_SubFrameZ=3 to 4 where Y and Z determine the timing of step 3 and step 7 MAC PDU transmission to avoid overlapping M-PDCCH DCI transmission and PDSCH transmission conflict. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant cat-M UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_6_2_EUTRA |
| --- | --- |
| Reason for change | Currently Test case 7.1.6.2 fails for a cat-M UE due to conflict in DCI transmission of step 3 and PDSCH transmission of step 1. For ex: Step 1 DCI transmission is at 449/7/1 for which PDSCH transmission is at 449/9/1 which is conflicting with Step 3 DCI transmission which is also at 449/9/1.This causes delay in PDSCH transmission for step 3 PDU(at 450/2/1) resulting in delay of HARQ feedback from UE later at step 4 and test case fails. Same is the case with step 7 PDSCH transmission and Step 9 DCI transmission. Reference collison handling R1-157885. |
| Summary of change | Changed the value of tsc_SubFrameY=2 to 3 and tsc_SubFrameZ=3 to 4 where Y and Z determine the timing of step 3 and step 7 MAC PDU transmission to avoid overlapping M-PDCCH DCI transmission and PDSCH transmission conflict |
| TTCN module | LTE\source\LTE\7_1\MAC_716.ttcn |
| MCC160 Comment |  |

Before change:

…

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC

{ /* L2/MAC

* 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception

*/

//@sic R5s110173 R5-113679 sic@

const integer tsc_LongDRXCycle := 640;

const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@

const integer tsc_OnDurationTimer := 20;

const integer tsc_DRXRetranTimer := 6;

const integer tsc_SubFrameX := 6;

const integer tsc_SubFrameY := 2;

const integer tsc_SubFrameZ := 3;

....

//@siclog "Step 3" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, tsc_SubFrameY, v_EUTRA_FDD_TDD_Mode);

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(2)));

//@siclog "Step 5" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing2, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Erroneous),

cs_TimingInfo_EUTRA_NB(v_Timing));

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (1, 10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B))))); */

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing1, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

else

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode);

}

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing3, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing3),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(3)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing3.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 7" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD)      //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing4),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(4)));

//@siclog "Step 9" siclog@

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (2,10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */

if (v_EUTRA_FDD_TDD_Mode == FDD)    //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF);

}

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing5),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(5) ));

....

After change:

…

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC

{ /* L2/MAC

* 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception

*/

//@sic R5s110173 R5-113679 sic@

const integer tsc_LongDRXCycle := 640;

const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@

const integer tsc_OnDurationTimer := 20;

const integer tsc_DRXRetranTimer := 6;

const integer tsc_SubFrameX := 6;

const integer tsc_SubFrameY := 3; //Anritsu TTCN fix for cat-m 7.1.6.2

const integer tsc_SubFrameZ := 4; //Anritsu TTCN fix for cat-m 7.1.6.2

....

//@siclog "Step 3" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, tsc_SubFrameY, v_EUTRA_FDD_TDD_Mode);

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(2)));

//@siclog "Step 5" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing2, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Erroneous),

cs_TimingInfo_EUTRA_NB(v_Timing));

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (1, 10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B))))); */

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing1, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

else

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode);

}

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing3, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing3),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(3)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing3.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 7" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD)      //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing4),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(4)));

//@siclog "Step 9" siclog@

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (2,10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */

if (v_EUTRA_FDD_TDD_Mode == FDD)    //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF);

}

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing5),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(5) ));

....