3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210587

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4614 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE measurement TC 8.3.1.16 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the case of CATM1 UEs, during the execution of TC 8.3.1.16 ,while sending RRC connection reconfiguration message in step 1-2 and step 14-15, where MeasGapConfig IE is included, need to derive the value of MeasGapConfig using the function f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE().Otherwise the TC for CATM1 UEs will fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE() is used to derive MeasGapConfig IE in case of CATM1 UEs for 8.3.1.16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant CATM1 UE may fail during the RRC Connection reconfiguration process which is included with MeasGapConfig IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3- B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_8_3_1_16_EUTRA |
| --- | --- |
| Reason for change | In the case of CATM1 UEs, during the execution of TC 8.3.1.16 ,while sending RRC connection reconfiguration message in Step 1-2 and Step 14-15, where MeasGapConfig IE is included, need to derive the value of MeasGapConfig using the function f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE().Otherwise the TC for CATM1 UEs will fail. |
| Summary of change | f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE() is used to derive MeasGapConfig IE in case of CATM1 UEs for 8.3.1.16. |
| TTCN module | \LTE\8_3\RRC_Measurement.ttcn |
| MCC160 Comment |  |

Before change:

<<SKIPPED CODE>>

//@siclog "Step 1-2" siclog@

//The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency and inter-band measurements on Cell 1.

//The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of intra-frequency and inter-band measurements on Cell 1.

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1,

cs_RRCConnectionReconfiguration_8_3_1_16_Step1(v_RRC_TI,

v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5s170224 sic@

v_ChBandDependency_f1.AllowedMeasBandwidth,

v_Frequency_IE_f5.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5s170224 sic@

v_ChBandDependency_f5.AllowedMeasBandwidth ));

<<SKIPPED CODE>>

//@siclog "Step 14 and Step 15" siclog@

//The SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 2.

//The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 2.

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell2, cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI)); //@sic R5s140492 sic@

<<SKIPPED CODE>>

After change:

<<SKIPPED CODE>>

//@siclog "Step 1-2" siclog@

//The SS transmits an RRCConnectionReconfiguration message to setup intra-frequency and inter-band measurements on Cell 1.

//The UE transmits an RRCConnectionReconfigurationComplete message to confirm the setup of intra-frequency and inter-band measurements on Cell 1.

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1,

cs_RRCConnectionReconfiguration_8_3_1_16_Step1(v_RRC_TI,

v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5s170224 sic@

v_ChBandDependency_f1.AllowedMeasBandwidth,

v_Frequency_IE_f5.UL_DL_Earfcn.dl_CarrierFreq_r9, //@sic R5s170224 sic@

v_ChBandDependency_f5.AllowedMeasBandwidth,

f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE()));

<<SKIPPED CODE>>

//@siclog "Step 14 and Step 15" siclog@

//The SS transmits an RRCConnectionReconfiguration message to activate the measurement gaps on Cell 2.

//The UE transmits an RRCConnectionReconfigurationComplete message to confirm the activation of the measurement gaps on Cell 2.

f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell2, cs_RRCConnectionReconfiguration_ResumeInterFreqMeas(v_RRC_TI,f_EUTRA_SS_MeasGapConfigIntraFreqOverrideCE())); //@sic R5s140492 sic@

<<SKIPPED CODE>>

## 2.2 Change 2

| Function name | template cs_RRCConnectionReconfiguration_8_3_1_16_Step1 |
| --- | --- |
| Reason for change | In the case of CATM1 UEs, during the execution of TC 8.3.1.16 ,while sending RRC connection reconfiguration message in step 1-2, where MeasGapConfig IE is included, need to include template (omit) MeasGapConfig p_MeasGapConfig inside the template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_3_1_16_Step1 to get the MeasGapConfig IE.Otherwise the TC for CATM1 UEs will fail. |
| Summary of change | template (omit) MeasGapConfig p_MeasGapConfig is included to derive MeasGapConfig IE in case of CATM1 UEs for 8.3.1.16. |
| TTCN module | \LTE\8_3\EUTRA_Measurement_Specific_Templates.ttcn |
| MCC160 Comment |  |

Before change:

template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_3_1_16_Step1(RRC_TransactionIdentifier p_RRC_TI,

ARFCN_ValueEUTRA_r9  p_EUTRA_DL_CarrierFreq_1, //@sic R5s170224 sic@

AllowedMeasBandwidth p_EUTRA_MeasurementBandwidth_1,

ARFCN_ValueEUTRA_r9  p_EUTRA_DL_CarrierFreq_5, //@sic R5s170224 sic@

AllowedMeasBandwidth p_EUTRA_MeasurementBandwidth_5) :=

/* @status    APPROVED (LTE) */

cs_508_RRCConnectionReconfiguration_ConditionMEAS(p_RRC_TI,

cs_MeasConfig(omit,

cs_MeasObjectToAddModList_2Entries(tsc_IdMeasObject_f1,

cs_508_MeasObjectEUTRA_GENERIC(f_EUTRA_ARFCN_Value(p_EUTRA_DL_CarrierFreq_1), p_EUTRA_MeasurementBandwidth_1), //@sic R5s170224 sic@

tsc_IdMeasObject_f5,

cs_508_MeasObjectEUTRA_GENERIC(f_EUTRA_ARFCN_Value(p_EUTRA_DL_CarrierFreq_5), p_EUTRA_MeasurementBandwidth_5)), //@sic R5s170224 sic@

omit,

cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3,

cs_ReportConfig_reportConfigEUTRA_eventA3(0, false, 0, ms0, rsrp, both, 1, ms1024, r1)), //@sic R5s120860 sic@

omit,

cs_MeasIdToAddModList_2Entries(1, tsc_IdMeasObject_f1, tsc_IdReportConfig_A3,

2, tsc_IdMeasObject_f5, tsc_IdReportConfig_A3),

-, //@sic R5s140184, R5-134645 sic@

cs_508_MeasGapConfig_GP1,

-,-,-,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(p_EUTRA_DL_CarrierFreq_1, p_EUTRA_DL_CarrierFreq_5))); //@sic R5s170224 sic@));

After change:

template (value) DL_DCCH_Message cs_RRCConnectionReconfiguration_8_3_1_16_Step1(RRC_TransactionIdentifier p_RRC_TI,

ARFCN_ValueEUTRA_r9  p_EUTRA_DL_CarrierFreq_1, //@sic R5s170224 sic@

AllowedMeasBandwidth p_EUTRA_MeasurementBandwidth_1,

ARFCN_ValueEUTRA_r9  p_EUTRA_DL_CarrierFreq_5, //@sic R5s170224 sic@

AllowedMeasBandwidth p_EUTRA_MeasurementBandwidth_5,

template (omit) MeasGapConfig p_MeasGapConfig) :=

/* @status    APPROVED (LTE) */

cs_508_RRCConnectionReconfiguration_ConditionMEAS(p_RRC_TI,

cs_MeasConfig(omit,

cs_MeasObjectToAddModList_2Entries(tsc_IdMeasObject_f1,

cs_508_MeasObjectEUTRA_GENERIC(f_EUTRA_ARFCN_Value(p_EUTRA_DL_CarrierFreq_1), p_EUTRA_MeasurementBandwidth_1), //@sic R5s170224 sic@

tsc_IdMeasObject_f5,

cs_508_MeasObjectEUTRA_GENERIC(f_EUTRA_ARFCN_Value(p_EUTRA_DL_CarrierFreq_5), p_EUTRA_MeasurementBandwidth_5)), //@sic R5s170224 sic@

omit,

cs_ReportConfigToAddModList_1Entry(tsc_IdReportConfig_A3,

cs_ReportConfig_reportConfigEUTRA_eventA3(0, false, 0, ms0, rsrp, both, 1, ms1024, r1)), //@sic R5s120860 sic@

omit,

cs_MeasIdToAddModList_2Entries(1, tsc_IdMeasObject_f1, tsc_IdReportConfig_A3,

2, tsc_IdMeasObject_f5, tsc_IdReportConfig_A3),

-, //@sic R5s140184, R5-134645 sic@

p_MeasGapConfig,

-,-,-,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(p_EUTRA_DL_CarrierFreq_1, p_EUTRA_DL_CarrierFreq_5))); //@sic R5s170224 sic@));