3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210588

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1735 | rev | - | Current version: | 16.1.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.4.1.9.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5GC test case 8.1.4.1.9.1 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.4.1.9.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

3 Branches executed	5

4 Execution Log Files	5

4.1 HiSilicon Balong 5000	5

5 References	5

- Overview

This document lists all the changes needed to correct issues in the ATS NR5GC_IWD_21wk12  related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

- Corrections requireds

  - Change 1

| Function name | f_TC_8_1_4_1_9_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | According to 38.331 cl. 5.5.4.1. when measurement report is triggered by A3 event on SCC, SCell shall also be considered as a neigibour cell as well. 4> if the eventA3 or eventA5 is configured in the corresponding reportConfig: 5> if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;. On the other hand, UE is also required to include measurement results of every serving cell in measResultServingCell field according to 38.331 cl.5.5.5.1: 1> for each serving cell configured with servingCellMO: 2> if the reportConfig associated with the measId that triggered the measurement reporting includesrsType: 3> if the serving cell measurements based on the rsTypeincluded in the reportConfigthat triggered the measurement report are available: 4> set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfigthat triggered the measurement report;. As a result, UE shall include measurement results of Cell 3 in both measResultNeighCells field and measResultServingCell field. A related prose cr R5-212419 has been rised by Huawei, HiSilicon in RAN5#91-e. |
| Summary of change | Measurement results of Cell 3 is added in measResultNeighCells |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell,

NR_CellId_Type p_SCell) runs on NR5GC_PTC

{

…

//@siclog "Step 6" siclog@

//The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10.

v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell);

v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell);

f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}));

//Config target cell for no RACH response

f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse);

…

//@siclog "Step 17" siclog@

//The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10.

v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell);

v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell);

f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}));

…

}

After change

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell,

NR_CellId_Type p_SCell) runs on NR5GC_PTC

{

…

//@siclog "Step 6" siclog@

//The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10.

v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell);

v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell);

f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}),-,v_PhysCellIdNeighbour);

//Config target cell for no RACH response

f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse);

…

//@siclog "Step 17" siclog@

//The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10.

v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell);

v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell);

f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}),-,v_PhysCellIdNeighbour);

…

}

- Branches executed

The test case was executed on NR band CA_n41C.

- Execution Log Files

  - HiSilicon Balong 5000

The HiSilicon Balong 5000passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_1_4_1_9_1_NR5GC_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

- References

| [1] | R5s210589: Supporting information for agreement of NR5GC test case 8.1.4.1.9.1 in FR1. |
| --- | --- |