3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210595

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1739 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RRC test case 8.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In test case 8.1.1.2.4.NR5GC, the Test Purpose is to verify a Rel-15 UE by sending an extended field in system information which this UE does not comprehend. For this purpose, SIB11-r16 is sent along with SIB2 as per Table 8.1.1.2.4.3.3-2: System Information in 38.523-1. But to achieve this, Scheduling information of SIB11-r16 should be present in SIB1 otherwise the UE may ignore the extended SIB due to incorrect scheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New system information combination NR-12 is introduced which contains scheduling of System information SIB2 and SIB11. See also related Prose CRs R5-213097 and R5-213098. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TP1 will not be fulfilled and UE may ignore extended SIBs for the wrong reason. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 2246 + TS 38.508-1 CR 1915 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to module RRC_ConnEst_NR5GC	2

1.2	Correction to function fl_TC_8_1_1_2_4_TestBody	3

1.3	Correction to enumeration NR_SysinfoCombination_Type	5

1.4	Correction to function fl_NR_InitSI_SchedulingInfo	5

1.5	Introduction of new function f_NR_SetSysinfoCombinationToNR_12	7

# Corrections required

## Correction to module RRC_ConnEst_NR5GC

| Module name | RRC_ConnEst_NR5GC |
| --- | --- |
| Reason for change | In test case 8.1.1.2.4.NR5GC, the Test Purpose is to verify a Rel-15 UE by sending an extended field in system information which this UE does not comprehend. For this purpose, SIB11-r16 is sent along with SIB2 as per Table 8.1.1.2.4.3.3-2: System Information in 38.523-1. But to achieve this, Scheduling information of SIB11-r16 should be present in SIB1 otherwise the UE may ignore the extended SIB due to incorrect scheduling. |
| Summary of change | Added “NR CellInfoInit” to the “import from” list at the beginning of TTCN module RRC_ConnEst_NR5GC.ttcn” in order to support change 1.2. |
| TTCN module | \NR5GC\8_1_1\RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

module RRC_ConnEst_NR5GC { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; <<SKIPPED CODE>> import from UpperTesterFunctions all;

After Change:

module RRC_ConnEst_NR5GC { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; <<SKIPPED CODE>> import from UpperTesterFunctions all; import from NR_CellInfoInit all;

## Correction to function fl_TC_8_1_1_2_4_TestBody

| Function name | fl_TC_8_1_1_2_4_TestBody |
| --- | --- |
| Reason for change | See change 1.1 above. |
| Summary of change | NR-12 system information combination is set so that SIB1 contains the scheduling information for SIB11. |
| TTCN module | NR5GC\8_1_1\RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC { //@sic R5-211312 R5s210243 Ch. 2 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ //System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B; // Table 8.1.1.2.4.3.3-1: MIB //Add SIB11 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1) v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[1].sib11_v1610 := valueof(cs_SIB11_r16); << SKIPPED CODE >> if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-211412 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); // Condition for Rel-15 UE only checked at the very end of the test case due to test flow as UE is only registered at the end and preamble is done with STATE_OFF_0A } }

After Change:

function fl_TC_8_1_1_2_4_TestBody() runs on NR5GC_PTC { //@sic R5-211312 R5s210243 Ch. 2 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var UE_NR_Capability v_NrCapability; var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //@siclog "Step 1" siclog@ f_NR_SetSysinfoCombinationToNR_12(nr_Cell1); v_NR_CellInfo := f_NR_CellInfo_Get(nr_Cell1); //System information that includes extended and spare fields that the UE does not comprehend is broadcasted on BCCH. v_NR_CellInfo.Sysinfo.BcchInfo.MIB.message_.mib.spare := '1'B; // Table 8.1.1.2.4.3.3-1: MIB //Add SIB11 with SIB2 in SI[0] as per Table 8.1.1.2.4.3.3-2: SystemInformation (step 1, Table 8.1.1.2.4.3.2-1) v_NR_CellInfo.Sysinfo.BcchInfo.SIs[0].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[1].sib11_v1610 := valueof(cs_SIB11_r16); << SKIPPED CODE >> if (not match (v_NrCapability.accessStratumRelease, rel15)) { //@sic R5-211412 sic@ f_SetVerdictInconc(__FILE__, __LINE__, "Test Case applicable to Rel-15 UE only"); // Condition for Rel-15 UE only checked at the very end of the test case due to test flow as UE is only registered at the end and preamble is done with STATE_OFF_0A } }

## Correction to enumeration NR_SysinfoCombination_Type

| Enum name | NR_SysinfoCombination_Type |
| --- | --- |
| Reason for change | See change 1.1 above. |
| Summary of change | Addition of new System information combination NR_12. |
| TTCN module | Common\NR\NR_Component_Base.ttcn |
| MCC160 Comment |  |

Before Change:

type enumerated NR_SysinfoCombination_Type { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //Acc to 38.508-1 Table 4.4.3.1.2-1 MibOnly, NR_1, NR_2, NR_3, NR_4, NR_5, NR_6, NR_7, NR_8, NR_9, NR_10, NR_11 };

After Change:

type enumerated NR_SysinfoCombination_Type { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ //Acc to 38.508-1 Table 4.4.3.1.2-1 MibOnly, NR_1, NR_2, NR_3, NR_4, NR_5, NR_6, NR_7, NR_8, NR_9, NR_10, NR_11, NR_12 };

## Correction to function fl_NR_InitSI_SchedulingInfo

| Function name | fl_NR_InitSI_SchedulingInfo |
| --- | --- |
| Reason for change | See change 1.1 above. |
| Summary of change | Introduced NR_12 System information combination to have SIB2 and SIB11 scheduling information. |
| TTCN module | Common\NR\NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { var template (omit) SI_SchedulingInfo v_SI_SchedulingInfo := omit; var template (value) SI_SchedulingInfo.schedulingInfoList v_SchedulingInfoList; var SI_SchedulingInfo.si_WindowLength v_SI_WindowLength; << SKIPPED CODE >> case(NR_10) { //SIB6 // @sic R5-191450 R5-202525 sic@ v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB6_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType6)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } case(NR_11) { //SIB7 // @sic R5-191450 R5-202525 sic@ v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB7_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType7)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } case else { FatalError(__FILE__, __LINE__, "Invalid NR SysinfoCombination"); } } return v_SI_SchedulingInfo; }

After Change:

function fl_NR_InitSI_SchedulingInfo(NR_SysinfoCombination_Type p_NR_SysinfoCombination, FreqBandIndicatorNR p_FreqBandIndicatorNR) return template (omit) SI_SchedulingInfo { var template (omit) SI_SchedulingInfo v_SI_SchedulingInfo := omit; var template (value) SI_SchedulingInfo.schedulingInfoList v_SchedulingInfoList; var SI_SchedulingInfo.si_WindowLength v_SI_WindowLength; << SKIPPED CODE >> case(NR_10) { //SIB6 // @sic R5-191450 R5-202525 sic@ v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB6_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType6)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } case(NR_11) { //SIB7 // @sic R5-191450 R5-202525 sic@ v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB7_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType7)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } case(NR_12) { //SIB11 // @sic R5-191450 R5-202525 sic@ v_SchedulingInfoList := cs_NR_SI_SchedulingInfo_List1(cs_NR_SchedulingInfoDef(tsc_SIB2_Periodicity, {cs_NR_SIB_TypeInfoDef(sibType2),cs_NR_SIB_TypeInfoDef(sibType11_v1610)})); v_SI_SchedulingInfo := cs_NR_SI_SchedulingInfoDef(v_SchedulingInfoList, v_SI_WindowLength); } case else { FatalError(__FILE__, __LINE__, "Invalid NR SysinfoCombination"); } } return v_SI_SchedulingInfo; }

## Introduction of new function f_NR_SetSysinfoCombinationToNR_12

| Function name | f_NR_SetSysinfoCombinationToNR_12 |
| --- | --- |
| Reason for change | See change 1.1 above. |
| Summary of change | Introduced a new function to take care of setting the scheduling information list of SIB1 to contain SIB2 and SIB11 and also to set the system information combination as NR_12. |
| TTCN module | Common\NR\ NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

New function Implementation:

function f_NR_SetSysinfoCombinationToNR_12(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC { //@sic R5-202525 sic@ var FreqBandIndicatorNR v_FreqBandIndicatorNR := f_NR_CellInfo_GetBandIndicator(p_NR_CellId); var template SI_SchedulingInfo v_SI_SchedulingInfo := fl_NR_InitSI_SchedulingInfo(NR_12, v_FreqBandIndicatorNR); f_NR_CellInfo_SetSIB1_SiSchedulingInfo(p_NR_CellId, valueof(v_SI_SchedulingInfo)); f_NR_CellInfo_SetSysinfoCombination(p_NR_CellId, NR_12); }