3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210599

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1743 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 8.1.1.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation , at step 7 of the test case , UE transmits RRC Resume Request and in response SS does not respond with Resume peer message . Expectation is that UE to trigger RRC setup request on the expiry of T319 . At step 7 , on the reception of RRC Resume Request, and further with Msg4 CR transmission , Active BWP is switched in the SS side . For SS to accept any further Prach , it is required to bring the SS back to Initial BWP state by sending the rach configuration again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rach configuration is sent in the test case after the expiry of T319 function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_8_1_1_4_2_TestBody	2

# Corrections required

## Correction to function fl_TC_8_1_1_4_2_TestBody

| Function name | fl_TC_8_1_1_4_2_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation , at step 7 of the test case , UE transmits RRC Resume Request and in response SS does not respond with Resume peer message . Expectation is that UE to trigger RRC setup request on the expiry of T319 . At step 7 , on the reception of RRC Resume Request, and further with Msg4 CR transmission , Active BWP is switched in the SS side . For SS to accept any further Prach , it is required to bring the SS back to Initial BWP state by sending the rach configuration again. |
| Summary of change | Rach configuration is sent in the test case after the expiry of T319 function |
| TTCN module | ttcn-3\ttcn\develop\NR5GC\8_1_1\RRC_Resume_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_1_4_2_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams; var NG_5G_S_TMSI v_NG_5G_S_TMSI; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var integer v_WaitTime := 1;//T319 is ms1000 by default var float v_T319Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); //@sic R5s191194 sic@ <<< SKIPPED CODE >>> //@siclog "Step 8" siclog@ //The SS waits for T319 expiry. f_Delay(v_T319Min); //@sic R5s191194 sic@ //@siclog "Step 9" siclog@ //Check: Does the test result of test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? f_NR5GC_MobilityRegistration(nr_Cell1); //@sic R5-199029 sic@ }

After Change:

function fl_TC_8_1_1_4_2_TestBody() runs on NR5GC_PTC { var NG_NAS_GutiParameters_Type v_GutiParams; var NG_5G_S_TMSI v_NG_5G_S_TMSI; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var integer v_WaitTime := 1;//T319 is ms1000 by default var float v_T319Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_WaitTime)); //@sic R5s191194 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedureConfig := f_NR_InitialiseRachProcedure(nr_Cell1); << SKIPPED CODE >> //@siclog "Step 8" siclog@ //The SS waits for T319 expiry. f_Delay(v_T319Min); //@sic R5s191194 sic@ f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedureConfig); //@siclog "Step 9" siclog@ //Check: Does the test result of test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? f_NR5GC_MobilityRegistration(nr_Cell1); //@sic R5-199029 sic@ }