3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210615

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0706 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improved handling of preconditions in 200 OK's Require header field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IMS over NR5GC call control test cases are with and without preconditions, what in general is reflected in the 200 OK's Require header field containing or not containing a "preconditions" option-tag. This can easily be expressed by a boolean but in TTCN the enumerated IMS_PreconditionIndication_Type is used which should be used only in cases where a "preconditions" option-tag may be either in the Require header field on in the Supported header field or not present at all. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMS_PreconditionIndication_Type replaced by boolean as parameter for f_IMS_OtherResponse_200_MessageHeaderTX, f_IMS_OtherResponse_200_MessageHeaderRX and f_IMS_MTCallSetup_Receive200OK_SendACK |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation remains confusing; additional complexity for implementation of new test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_IMS_OtherResponse_200_MessageHeaderTX, f_IMS_OtherResponse_200_MessageHeaderRX, f_IMS_MTCallSetup_Receive200OK_SendACK |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

function f_IMS_OtherResponse_200_MessageHeaderTX(MessageHeader p_MessageHeader_Request, template (omit) ContentType p_ContentType := omit, template (omit) charstring p_EmergencyTelURI := omit, charstring p_ContactUri := px_IMS_CalleeContactUri, boolean p_PreconditionsRequired := false, IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return template (value) MessageHeader { /* @sic R5-130683: common handling for INVITE and UPDATE sic@ */ /* p_EmergencyTelURI .. shall be set e.g. to tsc_IMS_Emergency_TelUri for condition A6 in A.3.1 */ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5s150237: parameter p_AdditionalOptionTagsForRequire sic@ */ /* @sic R5s150692 change 17: parameter p_CallType sic@ */ /* @sic R5s150851: no consistency check for p_CallType and p_EmergencyTelURI sic@ */ /* @sic R5-160794: p_AdditionalOptionTagsForRequire replaced by p_PreconditionIndication sic@ */ /* @sic R5-174544: SessionId removed sic@ */ var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(p_MessageHeader_Request); /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */ var charstring v_Method := p_MessageHeader_Request.cSeq.method; var boolean v_A1A3 := false; var boolean v_A18 := pc_IMS_Video_FeatureTag; var boolean v_A19 := false; var template (value) RouteBody_List v_RouteBodyList; var template (omit) Require v_Require := omit; var template (omit) SemicolonParam_List v_ContactParams := omit; if (p_PreconditionsRequired and ispresent(p_ContentType)) { /* @sic R5-160794 sic@ NOTE: responses use Require header even for session modifications */ v_Require := cs_Require(tsc_OptionTagList_precondition); } select (v_Method) { case ("INVITE") { v_A1A3 := true; v_A19 := true; } case ("UPDATE") { v_A1A3 := true; } } v_MessageHeader_Response.require := v_Require; /* @sic R5-160794 sic@ */ v_MessageHeader_Response.contentType := p_ContentType; if (v_A1A3) { v_RouteBodyList := f_IMS_RouteSet_MO_Call_TX(p_CallType); /* @sic R5s150692 change 17 sic@ */ v_MessageHeader_Response.recordRoute := cs_RecordRoute(v_RouteBodyList); v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("audio")); /* @sic R5-165943 sic@ */ if (v_A18 and v_A19) { v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("video")); /* @sic R5-191117 sic@ */ } v_MessageHeader_Response.contact := cs_Contact(f_SIP_BuildSipUri_TX(p_ContactUri), v_ContactParams); } if (ispresent(p_EmergencyTelURI)) { // Condition A6 @sic R5s150851: in case of A6 p_EmergencyTelURI shall be handed over from the calling side; p_CallType is not a criteria sic@ v_MessageHeader_Response.pAssertedID := cs_PAssertedId(f_SIP_BuildSipUri_lr_TX(valueof(p_EmergencyTelURI))); } return v_MessageHeader_Response; }

function f_IMS_OtherResponse_200_MessageHeaderRX(template (value) MessageHeader p_MessageHeader_Request, template ContentType p_ContentType := omit, boolean p_PreconditionRequiredForSDP := true ) runs on IMS_PTC return template (present) MessageHeader { /* NOTE: Message is sent by the UE => conditions A2, A4 and A5 are applicable; * For A4 (GIBA) pAccessNetworkInfo is optional */ /* @sic R5-130683 sic@ */ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5-160790, R5-160903, R5-160793: p_PreconditionIndication sic@ */ /* @sic R5-162930: p_PreconditionIndication := PreconditionRequired => per default there is a Require header if the message contains an SDP sic@ */ /* @sic R5-174544: SessionId removed sic@ */ var template (present) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderRX(p_MessageHeader_Request); /* @sic R5s200000: f_IMS_BaseResponse_MessageHeaderRX sic@ */ var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); /* acc. to A.3.1 condition A5: access network technology and, if applicable, the cell ID */ var boolean v_A2A4 := false; /* true => response sent by UE for INVITE/UPDATE */ var boolean v_A8 := true; /* true => any response but not CANCEL */ var boolean v_A20 := false; /* true => response sent by UE for INVITE */ var boolean v_A18 := pc_IMS_Video_FeatureTag; var charstring v_Method := valueof(p_MessageHeader_Request.cSeq.method); var integer v_Port_us := f_IMS_PTC_GetPort_us(); /* @sic R5s130985 Change 2 sic@ */ var template (present) SemicolonParam_List v_MandatoryContactParams := {}; var template (present) SipUrl v_ContactSipUrl := f_Contact_SipUri_HostPortRX(?, v_Port_us); /* host address needs to be checked after receiving (f_IMS_MessageHeader_CheckContactAddr) @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ select (v_Method) { case ("UPDATE") { v_A2A4 := true; } case ("INVITE") { v_A2A4 := true; v_A20 := true; } case ("CANCEL") { v_A8 := false; } } if (p_PreconditionRequiredForSDP and f_ContentType_IsSDP(p_ContentType)) { /* @sic R5-160794 sic@ NOTE 1: in general a SIP response carries an SDP answer (i.e. no offer) and therefore uses a Require but no Supported header NOTE 2: when the SDP is optional, ContentType is optional too (-> "ifpresent") and f_ContentType_IsSDP returns false */ v_MessageHeader_Response.require := cr_Require(tsc_OptionTagList_precondition); /* @sic R5-160790, R5-160903, R5-160793 sic@ */ } if (not f_IMS_PTC_SecurityScheme_IsGiba() and v_A8) { /* @sic R5-130683: pAccessNetworkInfo is optional for GIBA; R5s140092 change 12: CANCEL sic@ @sic R5-142223: new condition A8: Any response sent by the UE, except for CANCEL requests sic@ */ v_MessageHeader_Response.pAccessNetworkInfo := v_PAccessNetworkInfo; } else { v_MessageHeader_Response.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; } if (v_A2A4) { if (f_IMS_PTC_AudioFeatureTagIsRequired() ) { // @sic R5-173021 sic@ @sic R5-177127 sic@ v_MandatoryContactParams := f_SemicolonParam_List_Add_RX(v_MandatoryContactParams, cr_FeatureParam_Audio); /* @sic R5-165943, R5-170564 sic@ * !!!! A11 is for "Response sent by UE", but when the response is not for INVITE or UPDATE * !!!! there is no Contact header at all !!!! */ } if (v_A18 and v_A20) { /* @sic R5-191117 sic@ */ v_MandatoryContactParams := f_SemicolonParam_List_Add_RX(v_MandatoryContactParams, cr_FeatureParam_Video); } v_MessageHeader_Response.contact := cr_Contact(v_ContactSipUrl, v_MandatoryContactParams); /* @sic R5-165943 sic@ */ } if (v_A20) { /* response for INVITE @sic R5-135004 Change 3 sic@ PROSE ISSUE: prose is not clear about whether there shall be any RecordRoute header for responses to requests other than INVITE */ v_MessageHeader_Response.recordRoute := p_MessageHeader_Request.recordRoute; // @sic R5-135004: same as in the INVITE sent to the UE sic@ } v_MessageHeader_Response.contentType := p_ContentType; return v_MessageHeader_Response; }

function f_IMS_MTCallSetup_Receive200OK_SendACK(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template SDP_Message p_SDP_Message := omit, boolean p_PreconditionRequired200OK := true ) runs on IMS_PTC return IMS_ResponseWithSdp_Type { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5w140112: p_InviteRequestWithSdp instead of p_InviteRequest sic@ */ /* @sic R5w140112: returns IMS_ResponseWithSdp_Type instead of Response sic@ */ /* @sic R5-160903: p_PreconditionIndication sic@ */ /* @sic R5-162930: p_PreconditionIndication := PreconditionRequired => per default there is a Require header if the message contains an SDP sic@ */ var InternetProtocol_Type v_ProtocolUsedForInvite := p_InviteRequestWithSdp.RoutingInfo.Protocol; var IMS_DATA_RSP v_IMS_DATA_RSP; var Response v_Response; var MessageHeader v_MessageHeader; var MessageHeader v_MessageHeader_Invite; var template (present) MessageHeader v_MessageHeader_OtherResponse_200; var template (value) MessageHeader v_MessageHeader_Ack; var template ContentType v_ContentType := omit; var template MessageBody v_MessageBody := omit; var SipUrl v_ContactUrl; var template (omit) SDP_Message v_SDP_Message := omit; if (ispresent(p_SDP_Message)) { v_ContentType := cr_ContentTypeSDP; v_MessageBody := cr_MessageBody_SDP; } // Step 12. Receive 200 OK. The UE responds to INVITE with a 200 OK final response after the user answers the call. v_MessageHeader_Invite := p_InviteRequestWithSdp.Invite.msgHeader; v_MessageHeader_OtherResponse_200 := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Invite, v_ContentType, p_PreconditionRequired200OK); /* @sic R5-16090: p_PreconditionIndication sic@ */ v_IMS_DATA_RSP := f_IMS_ReceiveResponse(v_MessageHeader_Invite, v_ProtocolUsedForInvite, cr_Response(c_statusLine200, v_MessageHeader_OtherResponse_200, v_MessageBody)); /* @sic R5s140350 sic@ */ v_Response := v_IMS_DATA_RSP.Response; v_MessageHeader := v_Response.msgHeader; f_IMS_MessageHeader_CheckContactAddr(v_MessageHeader); /* check whether contact contains "SIP URI with IP address or FQDN" (see @sic R5s120727 sic@) */ v_SDP_Message := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP_Ifpresent(v_Response.messageBody, p_SDP_Message); if (isvalue(f_IMS_PTC_ImsInfo_DialogGetRemoteTag())) { /* normal case: there has been e.g. a 183 response already @sic R5s140017 MCC160 implementation: use remote tag sic@ */ f_IMS_MessageHeader_CheckTo(v_MessageHeader, dialogToTag); } else { /* special case (e.g. 16.X test cases): 200 OK is the only response */ f_IMS_Dialog_CompleteCreationForMTCall(v_MessageHeader); /* @sic R5s140017 MCC160 implementation: use remote tag sic@ */ } // Step 13. Send ACK. The SS acknowledges the receipt of 200 OK for INVITE. v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); /* @sic R5s150908 change 4: dialog sic@ @sic R5s170883: in some cases contact URI may be set just by 20o OK sic@ */ v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderTX(p_InviteRequestWithSdp.Invite); IMS_Client.send(cas_IMS_Ack_Request(cs_IMS_RoutingInfo(v_ProtocolUsedForInvite), cs_ACK_Request(v_ContactUrl, v_MessageHeader_Ack))); return valueof(cs_IMS_ResponseWithSdp(v_Response, v_SDP_Message)); }