3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210655

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1766 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_RRC_Idle_Extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If a UE is configured for IMS and Internet, it can happen that the UE initiates the PDU Session for IMS DNN on same connection, but needs a AT command trigger to establish the PDU Session for Internet DNN. In order to test a WE with such a setting, the PICS pc_noOf_PDUsSameConnection should be set to 1, and pc_noOf_PDUsNewConnection should also be set to 1. It is observed that even if this setting is applied, the prompt for AT command for Internet DNN doesn’t come. This is because the decision to prompt the user is made on condition v_NoOfExistingSessions == 0 in the current TTCN implementation of function f_NR5GC_RRC_Idle_Extension. This needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented TTCN that addresses the case when pc_noOf_PDUsSameConnection should be set to 1, and pc_noOf_PDUsNewConnection should also be set to 1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All NR5GC testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to f_NR5GC_RRC_Idle_Extension	3

# Corrections required

## Correction to f_NR5GC_RRC_Idle_Extension

| Template name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | If a UE is configured for IMS and Internet, it can happen that the UE initiates the PDU Session for IMS DNN on same connection, but needs a AT command trigger to establish the PDU Session for Internet DNN. In order to test a WE with such a setting, the PICS pc_noOf_PDUsSameConnection should be set to 1, and pc_noOf_PDUsNewConnection should also be set to 1. It is observed that even if this setting is applied, the prompt for AT command for Internet DNN doesn’t come. This is because the decision to prompt the user is made on condition v_NoOfExistingSessions == 0 in the current TTCN implementation of function f_NR5GC_RRC_Idle_Extension. This needs to be handled. |
| Summary of change | Implemented TTCN that addresses the case when pc_noOf_PDUsSameConnection should be set to 1, and pc_noOf_PDUsNewConnection should also be set to 1 |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } else { // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return; } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_Internet; } case (IMS_DNN) { v_DNN := pc_APN_ID_IMS; } } f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@ } else if (v_NoOfExistingSessions != 0 and pc_noOf_PDUsNewConnection > 0) { v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@ select (v_PDN_Type) { case (Internet_DNN, None) { v_DNN := pc_APN_ID_IMS; } case (IMS_DNN) { v_DNN := pc_APN_ID_Internet; } } f_UT_RequestPDUSession(UT, 2, v_DNN); }else {return;}/*{ // @sic R5s210084 sic@ // if an establishment request message hasn't been sent, or triggered, then don't need to carry on return; }*/ } } <<SKIPPED CODE>>