3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210662

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1769 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.1.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 11.1.2 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.1.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	10

2.4	Change 4	11

2.5	Change 5	13

2.6	Change 6	16

3	Branches executed	16

4	Execution Log Files	16

4.1	HiSilicon Balong 5000	16

5	References	17

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

  - Verification Test Summary

Test Case: 	11.1.2

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_11_1_2_NR5GC |
| --- | --- |
| Reason for change | - NR side should to use f_NR5GC_IRAT_UECapabilityTransfer to complete LTE registration process at preamble. - It should to set cell power on and UE power on command before NR registration. |
| Summary of change | - Add f_NR5GC_IRAT_UECapabilityTransfer after NR cell setup. - Instead NR preamble to f_NR5GC_Preamble_Steps1_13. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_5_NR5GC() runs on NR5GC_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NwkSupport := cds_NG_NetworkFeatureSupport_NoN26; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_S1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NwkSupport.emc := '00'B; // prose hardcodes these to different values from the default // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, v_NwkSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change

function f_TC_11_1_5_NR5GC() runs on NR5GC_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NwkSupport := cds_NG_NetworkFeatureSupport_NoN26; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (nr_Cell1); v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NwkSupport.emc := '00'B; // prose hardcodes these to different values from the default // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, v_NwkSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

  - Change 2

| Function name | fl_TC_11_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | - Due to delay in f_NR_RRCRelease_Common function, LTE side cannot process subsequent message in time. - Acoording to 38.331 clause 5.3.8.3, UE will ignore any field included in RRCRelease message except waitTime if the AS security is not activated, so it is should add security activate process after step 5. - In order to cooperate with the fl_TAUReject_AttachWithoutN26 function on the LTE side, UECapability should be sent. |
| Summary of change | - Send RRCRelease message only. - Add f_NR_RRC_ActivateSecurity after step5. - Add f_NR5GC_IRAT_UECapabilityTransfer. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_2_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ //@siclog "Step 2" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 3-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 5" siclog@ if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B, -, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo )); // Send authentication paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); // send details of existing PDU sessions f_NR5GC_SendPduSessionInfoListToEUTRA(); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change

function fl_TC_11_1_2_NR5GC_TestBody() runs on NR5GC_PTC { var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ //@siclog "Step 2" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Steps 3-4" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, mo_VoiceCall); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 5" siclog@ if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0001'B, -, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } f_NR_RRC_ActivateSecurity (nr_Cell1, v_ReceivedMsg.SecurityProtection.NasCount); //@siclog "Step 6" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); // Send authentication paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); // send details of existing PDU sessions f_NR5GC_SendPduSessionInfoListToEUTRA(); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

  - Change 3

| Function name | fl_TC_11_1_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - 1. Acoording to TS 23.502 clause 4.11.2.2, when UE performs mobility procedure from 5GS to EPS without N26 interface, UE will performs the UE requested PDN Connectivity Procedure if UE wants to transfer the remaining PDU Sessions in 5GS to EPS. The possible parallel behaviour description is missing after step 20. |
| Summary of change | - Add PDN CONNECTIVITY REQUEST process after step 27. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // @sic R5-210133 sic@ fl_TAUReject_AttachWithoutN26 (); // @siclog "Step 21" siclog@ // Step 7 of 4.5A.6.3-1 // FFS: This step is not needed as SRB2 & DRB already configured above f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(eutra_Cell1, tsc_RRC_TI_Def, {cs_508_SRB2_ToAddMod_DEFAULT}, {cs_508_DRB_ToAddMod_DEFAULT_AM(tsc_DRB1)}); // @siclog "Step 26 - 28" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, tsc_EpsDedicatedBearerId); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var EPS_BearerIdentity v_EpsBearerId; var SRB_COMMON_IND v_ReceivedAsp; var timer t_wait := 5; // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); // @sic R5-210133 sic@ fl_TAUReject_AttachWithoutN26 (); alt { [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2_FB(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_wait.timeout { // @siclog "Step 21" siclog@ // Step 7 of 4.5A.6.3-1 // FFS: This step is not needed as SRB2 & DRB already configured above f_EUTRA38_MultiPDN_RRCConnectionReconfiguration_Common(eutra_Cell1, tsc_RRC_TI_Def, {cs_508_SRB2_ToAddMod_DEFAULT}, {cs_508_DRB_ToAddMod_DEFAULT_AM(tsc_DRB1)}); } } // @siclog "Step 26 - 28" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, tsc_EpsDedicatedBearerId); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

  - Change 4

| Function name | f_TC_11_1_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - It should to use f_IMS_PTC_ImsInfo_GetHomeDomainName after the initialization step. - The IMS Deregistration process is happen on the LTE side. |
| Summary of change | - Use f_IMS_PTC_ImsInfo_GetHomeDomainName after fl_IMSFallback_PreambleRegistrations. - Set the first parameter to OtherIPCAN |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_2_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var template (present) SipUrl v_SipUrl := cr_SipUri_HostPort(f_IMS_PTC_ImsInfo_GetHomeDomainName()); var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var MessageHeader v_MessageHeader_Response183; var IMS_DATA_REQ v_IMS_REGISTER_REQ_ByRef; fl_IMSFallback_PreambleRegistrations(); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); // Exception: C.46 before Invite Request alt { [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request(v_SipUrl)), v_IMS_REGISTER_REQ_ByRef) { IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ_ByRef.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_IMS_REGISTER_REQ_ByRef.Request.Register)))); repeat; } [] IMS_Server.check (receive((car_IMS_Invite_Request(?)))) {} } // @siclog "Step 21 - 28" siclog@ /* Steps 7-14 from the Generic Test Procedure for MTSI MO speech call establishment 36.508 table 4.5A.6.3-1 */ v_InviteRequestWithSdp := f_IMS_MOCallSetup_AnnexC21_Step2_A4(); v_MessageHeader_Response183 := f_IMS_MOCallSetup_AnnexC21_Step3_4(v_InviteRequestWithSdp); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, v_MessageHeader_Response183); // @siclog "Step 29" siclog@ f_Delay(1.0); // @siclog "Step 30" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_1_2_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var template (present) SipUrl v_SipUrl; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var MessageHeader v_MessageHeader_Response183; var IMS_DATA_REQ v_IMS_REGISTER_REQ_ByRef; fl_IMSFallback_PreambleRegistrations(); v_SipUrl := cr_SipUri_HostPort(f_IMS_PTC_ImsInfo_GetHomeDomainName()); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); // Exception: C.46 before Invite Request alt { [] a_IMS_ReceiveRequest(car_IMS_Register_Request(cr_REGISTER_Request(v_SipUrl)), v_IMS_REGISTER_REQ_ByRef) { IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ_ByRef.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_IMS_REGISTER_REQ_ByRef.Request.Register)))); repeat; } [] IMS_Server.check (receive((car_IMS_Invite_Request(?)))) {} } // @siclog "Step 21 - 28" siclog@ /* Steps 7-14 from the Generic Test Procedure for MTSI MO speech call establishment 36.508 table 4.5A.6.3-1 */ v_InviteRequestWithSdp := f_IMS_MOCallSetup_AnnexC21_Step2_A4(); v_MessageHeader_Response183 := f_IMS_MOCallSetup_AnnexC21_Step3_4(v_InviteRequestWithSdp); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, v_MessageHeader_Response183); // @siclog "Step 29" siclog@ f_Delay(1.0); // @siclog "Step 30" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(OtherIPCAN); }

  - Change 5

| Function name | f_IMS_IpcanReleaseWithOptionalDeregistration |
| --- | --- |
| Reason for change | - Modify with change point 4. |
| Summary of change | - Add the parameter p_Port. |
| TTCN module | IMS_Procedures_Registration.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(boolean p_AllowRetransmission := false) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(IPCAN, t_Dummy, -, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

After Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(IMS_IPCAN_CO_ORD_PORT p_Port, boolean p_AllowRetransmission := false) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(p_Port, t_Dummy, -, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

- Branches executed

NR Band 41, nea2, Nia2.

- Execution Log Files

  - HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_1_2_NR5GC_N41-FDD3.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_1_2_NR5GC).xml

- References

| [1] | R5s210663: Supporting information for agreement of NR5GC test case 11.1.2 in FR1 |
| --- | --- |