3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210680

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0713 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_BuildSDP_InitialOfferAudio_TX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current implementation of function f_IMS_BuildSDP_InitialOfferAudio_TX, lines of code to assign media attributes to SDP offer have been removed, possibly due to code restructuring. As a consequence, variable v_SDP_MediaAttributes remains as uninitialized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added back the code that was present in previous ATS 21wk12. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may fail as SDP offer in DL INVITE is incomplete.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NR IMS MT Voice call TCs. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_IMS_BuildSDP_InitialOfferAudio_TX	3

# Corrections required

## Correction to function f_IMS_BuildSDP_InitialOfferAudio_TX

| Function name | f_IMS_BuildSDP_InitialOfferAudio_TX |
| --- | --- |
| Reason for change | In current implementation of function f_IMS_BuildSDP_InitialOfferAudio_TX, lines of code to assign media attributes to SDP offer have been removed, possibly due to code restructuring. As a consequence, variable v_SDP_MediaAttributes remains as uninitialized. |
| Summary of change | Added back the code that was present in previous ATS 21wk12. |
| TTCN module | IMS_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_BuildSDP_InitialOfferAudio_TX(IMS_SDP_AudioCodecList_Type p_AudioCodecList, boolean p_PreconditionsRequired := true, boolean p_RemoteResourcesReservedAlready := false) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s141244: use of f_IMS_BuildSDP_TX sic@ */ /* @sic R5-153610: "sendrecv" media attribute removed from SDP message for C.11 step 7 sic@ */ /* @sic R5-153798: p_SDP_Message183 replacing p_SDP_Attribute_curr_qos_remote and p_Step; p_IsC45 sic@ */ /* @sic R5-153798, R5s150841: split of f_IMS_BuildSDP_AnnexC11_Step1or7, parameter p_AdditionalAudioCodecs for C.45 sic@ */ /* @sic R5-169060: allow no preconditions sic@ */ /* @sic R5-173535: p_AudioCodecList instead of p_AdditionalAudioCodecs sic@ */ /* @sic R5-191731: p_RemoteResourcesReservedAlready sic@ */ var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList; var template (value) SDP_bandwidth_list v_MediaBandwidth_List; var template (omit) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (omit) SDP_attribute_list v_SDP_PreconditionAttributes; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171477 sic@ */ if (p_AudioCodecList[0] != EVS_16000) { /* @sic R5-173535: check of first element rather than any list of additional codecs sic@ */ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); /* media bandwidth acc. to C.11 @sic R5-130751; R5-134270: RR=2500; R5-144700: AS=37; R5-172126: RR=2000 sic@ */ } else { v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(65, 0, 2000); /* media bandwidth acc. to C.45 INVITE (Step 1) */ } v_MediaDescriptionRtpList := f_SDP_MediaDescriptionInitialOfferAudioTX(p_AudioCodecList); /* @sic R5-172930 sic@ @sic R5-165942 sic@ */ if ((p_RemoteResourcesReservedAlready and p_PreconditionsRequired) or /* @sic R5-191731: test case 12.13a sic@ */ (v_IsWLAN and p_PreconditionsRequired)) { /* @sic R5-171477 - WLAN Annex C.11a sic@ */ v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); /* @sic R5s180162 sic@ @sic R5-153798: usage of cs_SDP_PreconditionAttributes sic@ */ } else if (p_PreconditionsRequired) { /* Annex C.11, C.11b, C.45 */ v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); /* @sic R5-153798: usage of cs_SDP_PreconditionAttributes sic@ */ } else { /* Annex C.11b, 34.229-5 A.5.2 */ v_SDP_PreconditionAttributes := omit; } return f_IMS_BuildSDP_TX(cs_SDP_Media_MultiAudio(v_MediaDescriptionRtpList.FmtList), v_MediaBandwidth_List, v_SDP_MediaAttributes, v_SDP_PreconditionAttributes); }

After Change:

function f_IMS_BuildSDP_InitialOfferAudio_TX(IMS_SDP_AudioCodecList_Type p_AudioCodecList, boolean p_PreconditionsRequired := true, boolean p_RemoteResourcesReservedAlready := false) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5s141244: use of f_IMS_BuildSDP_TX sic@ */ /* @sic R5-153610: "sendrecv" media attribute removed from SDP message for C.11 step 7 sic@ */ /* @sic R5-153798: p_SDP_Message183 replacing p_SDP_Attribute_curr_qos_remote and p_Step; p_IsC45 sic@ */ /* @sic R5-153798, R5s150841: split of f_IMS_BuildSDP_AnnexC11_Step1or7, parameter p_AdditionalAudioCodecs for C.45 sic@ */ /* @sic R5-169060: allow no preconditions sic@ */ /* @sic R5-173535: p_AudioCodecList instead of p_AdditionalAudioCodecs sic@ */ /* @sic R5-191731: p_RemoteResourcesReservedAlready sic@ */ var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList; var template (value) SDP_bandwidth_list v_MediaBandwidth_List; var template (omit) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (omit) SDP_attribute_list v_SDP_PreconditionAttributes; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171477 sic@ */ if (p_AudioCodecList[0] != EVS_16000) { /* @sic R5-173535: check of first element rather than any list of additional codecs sic@ */ v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(37, 0, 2000); /* media bandwidth acc. to C.11 @sic R5-130751; R5-134270: RR=2500; R5-144700: AS=37; R5-172126: RR=2000 sic@ */ } else { v_MediaBandwidth_List := cs_SDP_Bandwidth_List_Media(65, 0, 2000); /* media bandwidth acc. to C.45 INVITE (Step 1) */ } v_MediaDescriptionRtpList := f_SDP_MediaDescriptionInitialOfferAudioTX(p_AudioCodecList); /* @sic R5-172930 sic@ @sic R5-165942 sic@ */ v_SDP_MediaAttributes := f_SDP_Attributes_Concat_TX(v_MediaDescriptionRtpList.AttributeList, cs_SDP_MediaAttributes_ptime_maxptime); // @sic R5-165942: to have ptime, maxptime at the end of the list sic@ if ((p_RemoteResourcesReservedAlready and p_PreconditionsRequired) or /* @sic R5-191731: test case 12.13a sic@ */ (v_IsWLAN and p_PreconditionsRequired)) { /* @sic R5-171477 - WLAN Annex C.11a sic@ */ v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); /* @sic R5s180162 sic@ @sic R5-153798: usage of cs_SDP_PreconditionAttributes sic@ */ } else if (p_PreconditionsRequired) { /* Annex C.11, C.11b, C.45 */ v_SDP_PreconditionAttributes := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); /* @sic R5-153798: usage of cs_SDP_PreconditionAttributes sic@ */ } else { /* Annex C.11b, 34.229-5 A.5.2 */ v_SDP_PreconditionAttributes := omit; } return f_IMS_BuildSDP_TX(cs_SDP_Media_MultiAudio(v_MediaDescriptionRtpList.FmtList), v_MediaBandwidth_List, v_SDP_MediaAttributes, v_SDP_PreconditionAttributes); }