3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210681

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1778 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_CheckDataPath_CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the function f_NR_CheckDataPath_CA(), the data is being originated on the SCell whereas the DRB was established on the PSCell, this seems to be contradictory. As the DRB was established on the PSCell and not on the SCell, the DRB data should originate on the PSCell (NR Cell 1) |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to make the DRB data originated from PSCell ( NR Cell 1 ) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN may not be in line with the Prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the ENDC CA test cases where loopback of the DRB data is involved |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s210298 was raised for this earlier |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_NR_CheckDataPath_CA	2

# Corrections required

## Correction to function f_NR_CheckDataPath_CA

| Function name | f_NR_CheckDataPath_CA |
| --- | --- |
| Reason for change | In the current TTCN implementation of the function f_NR_CheckDataPath_CA(), the data is being originated on the SCell whereas the DRB was established on the PSCell, this seems to be contradictory. As the DRB was established on the PSCell and not on the SCell, the DRB data should originate on the PSCell (NR Cell 1) |
| Summary of change | It is proposed to make the DRB data originated from PSCell ( NR Cell 1 ) |
| TTCN module | ENDC\Common_NR\NR_CommonFunctions_ENDC_CA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_SCellIndex_2, boolean p_UL_CA := false ) runs on NR_BASE_PTC { //@sic R5-213516 sic@ var template(value) MacBearerRouting_Type v_MacBearerRoutingDL := crs_MacBearerRouting(p_SCell); //Send DL data oin SCell var template(value) MacBearerRouting_Type v_MacBearerRoutingUL := crs_MacBearerRouting(p_PCell); // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex); } if (p_UL_CA) { v_MacBearerRoutingUL := crs_MacBearerRouting(p_SCell); } // Steps 2 f_NR_CheckDataPath(p_SCell, // for DL PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(p_PCell), cs_RlcBearerRouting_NR(p_PCell), p_StepNo, -, v_MacBearerRoutingDL, v_MacBearerRoutingUL ); }

After Change:

function f_NR_CheckDataPath_CA(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, charstring p_StepNo := "", boolean p_ActivateSCell := true, SCellIndex p_SCellIndex := tsc_SCellIndex_2, boolean p_UL_CA := false ) runs on NR_BASE_PTC { //@sic R5-213516 sic@ var template(value) MacBearerRouting_Type v_MacBearerRoutingDL := crs_MacBearerRouting(p_SCell); //Send DL data oin SCell var template(value) MacBearerRouting_Type v_MacBearerRoutingUL := crs_MacBearerRouting(p_PCell); // Step 1 : The SS transmits an Activation MAC control element to activate SCell. if (p_ActivateSCell) { f_SS_CellConfig_ActivateSCell_CA(p_PCell, p_SCellIndex); } if (p_UL_CA) { v_MacBearerRoutingUL := crs_MacBearerRouting(p_SCell); } // Steps 2 f_NR_CheckDataPath(p_PCell, // for DL PDN_2, dedicatedBearer, tsc_NR_DRB2, cs_RlcBearerRouting_NR(p_PCell), cs_RlcBearerRouting_NR(p_PCell), p_StepNo, -, v_MacBearerRoutingDL, v_MacBearerRoutingUL ); }