3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210683

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4622 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.2.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | T1 should be calculated when UE finds out that RLC PDU is missing. In step3 the SS transmits RLC PDU#3 containing RLC SDU#2. UE starts t-Reordering at this time. Note T1 should start to calculate in step3 not in step1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Calculate T1 in step 3 instead of step 1. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Corrections to calculate T1 in test cases 7.2.2.8	3

1. Correction to ' fl_TC_7_2_2_8_TestBody ()'	3

# Corrections to calculate T1 in test cases 7.2.2.8

## 1. Correction to ' fl_TC_7_2_2_8_TestBody ()'

| Function name | fl_TC_7_2_2_8_TestBody() |
| --- | --- |
| Reason for change | T1 should be calculated when UE finds out that RLC PDU is missing. In step3 the SS transmits RLC PDU#3 containing RLC SDU#2. UE starts t-Reordering at this time. Note T1 should start to calculate in step3 not in step1. |
| Summary of change | Calculate T1 in step 3 instead of step 1. |
| TTCN module | ATS_LTE\ 7_2\RLC_UM_Testcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 1" siclog@ // The SS transmits RLC PDU#1 containing first segment of RLC SDU#1. v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1); // @sic R5s170822 sic@ // Note T1 v_SubFrameTimingT1.HSFN.Number := v_Timing.HSFN.Number; v_SubFrameTimingT1.SFN.Number := v_Timing.SFN.Number; v_SubFrameTimingT1.Subframe.Number := v_SubframeOffset; f_TxUMD_PDUpartofSDU (p_RLC_Rec, cs_TimingInfo_SubFrameOffset(v_Timing.SFN.Number, v_SubframeOffset, v_Timing.HSFN.Number), tsc_RLC_SDU1, 0, 75, tsc_FI_StartOfSDU); //@siclog "Step 2" siclog@ // The SS does not transmit RLC PDU#2 containing last segment of RLC SDU#1. // nothing sent in next subframe //@siclog "Step 3" siclog@ // The SS transmits RLC PDU#3 containing RLC SDU#2. p_RLC_Rec.UM_VTUS := 2; v_Timing := f_SubFrameTiming_IncrSFN(v_Timing); // @sic R5s170822, R5s190187, R5s190398 sic@ f_TxUMD_oneSDU( p_RLC_Rec, cs_TimingInfo_SubFrameOffset(v_Timing.SFN.Number, v_SubframeOffset, v_Timing.HSFN.Number), tsc_RLC_SDU2 ); //@siclog "Step 3A" siclog@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 1" siclog@ // The SS transmits RLC PDU#1 containing first segment of RLC SDU#1. v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1); // @sic R5s170822 sic@ // Note T1 /*COMMENTED OUT v_SubFrameTimingT1.HSFN.Number := v_Timing.HSFN.Number; v_SubFrameTimingT1.SFN.Number := v_Timing.SFN.Number; v_SubFrameTimingT1.Subframe.Number := v_SubframeOffset;*/ f_TxUMD_PDUpartofSDU (p_RLC_Rec, cs_TimingInfo_SubFrameOffset(v_Timing.SFN.Number, v_SubframeOffset, v_Timing.HSFN.Number), tsc_RLC_SDU1, 0, 75, tsc_FI_StartOfSDU); //@siclog "Step 2" siclog@ // The SS does not transmit RLC PDU#2 containing last segment of RLC SDU#1. // nothing sent in next subframe //@siclog "Step 3" siclog@ // The SS transmits RLC PDU#3 containing RLC SDU#2. p_RLC_Rec.UM_VTUS := 2; v_Timing := f_SubFrameTiming_IncrSFN(v_Timing); // @sic R5s170822, R5s190187, R5s190398 sic@ v_SubFrameTimingT1.HSFN.Number := v_Timing.HSFN.Number; v_SubFrameTimingT1.SFN.Number := v_Timing.SFN.Number; v_SubFrameTimingT1.Subframe.Number := v_SubframeOffset; f_TxUMD_oneSDU( p_RLC_Rec, cs_TimingInfo_SubFrameOffset(v_Timing.SFN.Number, v_SubframeOffset, v_Timing.HSFN.Number), tsc_RLC_SDU2 ); //@siclog "Step 3A" siclog@ <<SKIPPED CODE>>