3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210730

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1802 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EN-DC RRC test case 8.2.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For step 4 : CounterCheckResponse message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path? The current TTCN implementation is incorrect. The received CounterCheckResponse message will have the same contents when received over the SRB1 MCG and SCG paths, and will contain DRB count infor for both MCG and SCG DRBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the expected contents of CounterCheckResponse so that duplicate contents are expected on MCG and SCG paths |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | For step 4 : CounterCheckResponse message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path? The current TTCN implementation is incorrect. The received CounterCheckResponse message will have the same contents when received over the SRB1 MCG and SCG paths, and will contain DRB count infor for both MCG and SCG DRBs. |
| Summary of change | Modified the expected contents of CounterCheckResponse so that duplicate contents are expected on MCG and SCG paths |
| TTCN module | RRCReconfig_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5-191203 R5-201204 R5-213472 sic@

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (omit) octetstring v_NR_SCGConfig := omit;

var integer v_MCG_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var integer v_SCG_DRB_Id := tsc_DRB2;

var template (value) DRB_CountMSB_InfoList v_CountList;

….

// @siclog "Step 4" siclog@

// Check: Does the UE send CounterCheckResponse message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, // Table 8.2.2.2.1.3.2-3: Parallel behaviour

tsc_SRB1,

cr_508_CounterCheckResponse(-, {cr_DRB_Count_Info(v_MCG_DRB_Id) }),

-,

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_CounterCheckResponse(-, { cr_DRB_Count_Info(v_SCG_DRB_Id)}),

-,

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4");

// @siclog "Step 4A" siclog@

//After steps in table 8.2.2.2.1.3.2-3 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 4 is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

…

}

After Change

function fl_TC_8_2_2_2_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{ // @sic R5-191203 R5-201204 R5-213472 sic@

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var template (omit) octetstring v_NR_SCGConfig := omit;

var integer v_MCG_DRB_Id := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var integer v_SCG_DRB_Id := tsc_DRB2;

var template (value) DRB_CountMSB_InfoList v_CountList;

….

// @siclog "Step 4" siclog@

// Check: Does the UE send CounterCheckResponse message including UE radio access capability information for NR on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, // Table 8.2.2.2.1.3.2-3: Parallel behaviour

tsc_SRB1,

cr_508_CounterCheckResponse(-, {cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}), //82221@ttcn CR

-,

crs_RlcBearerRouting_Eutra(eutra_Cell1))) {};

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,

tsc_SRB1,

cr_508_CounterCheckResponse(-, {cr_DRB_Count_Info(v_MCG_DRB_Id), cr_DRB_Count_Info(v_SCG_DRB_Id)}),  //82221@ttcn CR

-,

cr_RlcBearerRouting_NR(nr_Cell1))) {};

}

f_EUTRA_PreliminaryPass (__FILE__, __LINE__, "Step 4");

// @siclog "Step 4A" siclog@

//After steps in table 8.2.2.2.1.3.2-3 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 4 is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

…

}