3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210735

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1803 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for ENDC RRC test case 8.2.6.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As part of R5-213484, Preamble generic procedure has been changed from ENDC to EUTRA/EPC in accordance with 38.508-1 Section 4.5.4.1. However, changes done under the same will not handle UE capability procedure where UE capability information is requested for NR and EUTRA-NR. This will lead to make TC non-conformant with test procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists change applied for reverting preamble general procedure to make aligned w.r.t. Wk12 TTCN. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.2.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_2_6_2_1_ENDC_EUTRA |
| --- | --- |
| Reason for change | f_EUTRA_Preamble will not handle the UE capability procedure where UE capability information is requested for NR and EUTRA-NR |
| Summary of change | Original Wk12 implementation has been adopted. Please refer the highlighted change. |
| TTCN module | RRC_MRDC_Others_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_6_2_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { // Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (E-UTRA/EPC) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4. // @sic R5-213484 sic@ f_EUTRA_Preamble (eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_8_2_6_2_1_ENDC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change:

| function f_TC_8_2_6_2_1_ENDC_EUTRA() runs on EUTRA_5GS_PTC { // Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and Bearers (MCG(s) only) established and Test Loop Function (On) with UE test loop mode B according to TS 38.508-1 [4], clause 4.5.4. f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTModeB_ON, MCG_Only, RRC_CONNECTED); f_EUTRA_TestBody_Set(true); fl_TC_8_2_6_2_1_ENDC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA_StartDefULGrantTransmission(eutra_Cell1); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); } |  |  |  |
| --- | --- | --- | --- |

## Change 2

| Function name | f_TC_8_2_6_2_1_ENDC_NR |
| --- | --- |
| Reason for change | NR capabilities sent during EUTRA preamble procedure were not handled. |
| Summary of change | Original Wk12 implementation has been adopted. Please refer the highlighted change. |
| TTCN module | RRC_MRDC_Others_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_2_6_2_1_ENDC_NR() runs on ENDC_NR_PTC { // Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_TestBody_Set(true); fl_TC_8_2_6_2_1_ENDC_NR_TestBody (); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_2_6_2_1_ENDC_NR() runs on ENDC_NR_PTC { // Processing delay / PSCell addition / SCG DRB / Success / Latency check / EN-DC f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_TestBody_Set(true); fl_TC_8_2_6_2_1_ENDC_NR_TestBody (); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }