3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210759

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4625 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA multilayer test case 11.2.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-07-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following implementation of TTCN CR R5-191230, optional IMS registration is not handled correctly at step 14-14L. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct handling of optional IMS emergency registration at step 14-14L |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_11_2_7_IMS2() |
| --- | --- |
| Reason for change | Following implementation of TTCN CR R5-191230, optional IMS registration is not handled correctly at step 14-14L. |
| Summary of change | Correct handling of optional IMS emergency registration at step 14-14L |
| TTCN module | IMS_EmergencyCall_IMS.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_11_2_7_IMS2() runs on IMS_PTC

{

var REGISTER_Request v_RegisterRequest;

var INVITE_Request v_InviteRequest;

var IMS_DATA_REQ v_IMS_DATA_REQ;

var REGISTER_Request v_RegisterReq;

var default v_RegistrationHandler := null;

f_IMS_PTC_Init(PDN_2);

// Emergency registration: 34.229-1 Annex C.20

v_RegisterRequest := f_IMS_EmergencyRegistration_C20();

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup_AnnexC22();

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

// 2nd emergency registration may be optional: default handler to deal with C.20 if happening

v_RegistrationHandler := activate(a_IMS_RcvAndHandleRegister(car_IMS_Register_Request(cr_REGISTER_Request())));  //@sic R5-191230 sic@

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup_AnnexC22();

// deactivate default handler to deal with C.20 if happening

deactivate(v_RegistrationHandler);

// Inform EUTRA: IMS Call established.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//IMS call to be released.

f_IMS_IPCAN_WaitForTrigger(IPCAN);

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

}

After change

function f_TC_11_2_7_IMS2() runs on IMS_PTC

{

var REGISTER_Request v_RegisterRequest;

var INVITE_Request v_InviteRequest;

var IMS_DATA_REQ v_IMS_DATA_REQ;

var REGISTER_Request v_RegisterReq;

var default v_RegistrationHandler := null;

var IMS_RoutingInfo_Type v_RoutingInfo_UL;

var InternetProtocol_Type v_Protocol;

var IP_AddrInfo_Type v_UE_Address;

var IP_AddrInfo_Type v_NW_Address;

var IPCAN_INFO_Type v_IpcanInfo;

var IMS_Request_Type v_Request;

f_IMS_PTC_Init(PDN_2);

// Emergency registration: 34.229-1 Annex C.20

v_RegisterRequest := f_IMS_EmergencyRegistration_C20();

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup_AnnexC22();

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//Release Register info

f_IMS_GlobalRelease();

IMS_Server.receive(cr_IMS_DATA_REQ_Any) -> value v_IMS_DATA_REQ;

v_Request := v_IMS_DATA_REQ.Request;

if (ischosen(v_Request.Register))

{

v_RegisterReq := v_IMS_DATA_REQ.Request.Register;

v_RoutingInfo_UL := v_IMS_DATA_REQ.RoutingInfo;

v_UE_Address := v_RoutingInfo_UL.UE_Address;

v_NW_Address := v_RoutingInfo_UL.NW_Address;

v_Protocol := v_RoutingInfo_UL.Protocol;

v_IpcanInfo := f_IMS_IPCAN_Query(IPCAN);

f_IMS_PTC_SetIpcanInfo(v_IpcanInfo);

f_IMS_PTC_ImsInfo_Init(v_UE_Address, v_NW_Address, v_IMS_DATA_REQ.Request.Register, false);

f_IMS_Register_SecurityInit(v_RegisterReq);

IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq))));

v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, true);

v_RegisterReq := v_IMS_DATA_REQ.Request.Register;

IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, true, tsc_IMS_RegisterExpiration)))); // @sic R5s130561 sic@

}

// Emergency call setup: 34.229-1 Annex C.22

v_InviteRequest := f_IMS_EmergencyCallSetup_AnnexC22();

// deactivate default handler to deal with C.20 if happening

// deactivate(v_RegistrationHandler);

// Inform EUTRA: IMS Call established.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//IMS call to be released.

f_IMS_IPCAN_WaitForTrigger(IPCAN);

//Release Emergency Call MO

f_IMS_EmergencyCallRelease(v_InviteRequest);

// Inform EUTRA: IMS Call released.

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

}