3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210762

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4626 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RLC UM mode TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In D21wk24 ttcn, the default value of UM_SN_Size is changed from 10 to 5. This is causing the below listed TCs to fail, which uses 10-bit SN. As per the prose, TCs 7.2.2.2/4/5.2 should use 10-bit SN. Though there is no prose restriction for TCs 7.2.2.6/7/8/9/10/11 to use 5 or 10-bit SN, throughout the TC procedure the TTCN uses functions & templates to handle 10-bit SN only. Ex: fl_GenerateRLC_UM_SDUs, cds_RX_UM_RLC_DRB_SN10_tReord200ms, cr_UMD_PDUoneSDU_SN10, cr_UMD_PDU_SN10 etc. The default value need not be set to 5, as the TCs which uses 5-bit (7.2.2.1, 7.2.2.3 & 7.2.2.5.1) explicitly uses the constant tsc_UM_SN_Size_5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reverted the default value of UM_SN_Size to 10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TCs 7.2.2.2, 7.2.2.4, 7.2.2.5.2, 7.2.2.6, 7.2.2.7, 7.2.2.8, 7.2.2.9, 7.2.2.10 and 7.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_InitRLC_Record |
| --- | --- |
| Reason for change | In D21wk24 ttcn, the default value of UM_SN_Size is changed from 10 to 5. This is causing the below listed TCs to fail, which uses 10-bit SN. As per the prose, TCs 7.2.2.2/4/5.2 should use 10-bit SN. Though there is no prose restriction for TCs 7.2.2.6/7/8/9/10/11 to use 5 or 10-bit SN, throughout the TC procedure the TTCN uses functions & templates to handle 10-bit SN only. Ex: fl_GenerateRLC_UM_SDUs, cds_RX_UM_RLC_DRB_SN10_tReord200ms, cr_UMD_PDUoneSDU_SN10, cr_UMD_PDU_SN10 etc. The default value need not be set to 5, as the TCs which uses 5-bit (7.2.2.1, 7.2.2.3 & 7.2.2.5.1) explicitly uses the constant tsc_UM_SN_Size_5. |
| Summary of change | Reverted the default value of UM_SN_Size to 10 |
| TTCN module | EUTRA_NB_CommonRLC.ttcn |
| MCC160 Comment |  |

Before change:

function f_InitRLC_Record(out RLC_SS_State p_RLC_Rec,

RLC_Mode_Type p_RLC_Mode)

{

if  ( p_RLC_Mode == UM_Mode) {

p_RLC_Rec.t_Reordering := 0.05; // FFS

}

else {

p_RLC_Rec.t_Reordering := 0.08;

}

p_RLC_Rec.t_PollRetransmit := 0.08;

p_RLC_Rec.t_StatusProhibit := 0.06; // @sic R5s100039 sic@

p_RLC_Rec.pollPDU := 128;

p_RLC_Rec.pollByte := 125000;

p_RLC_Rec.maxRetxThreshold := 4;

p_RLC_Rec.UM_SN_Size := 5; // FFS originally 10;  tsc_UM_SN_Size_5 default ??

// Initialisation of other parameters

p_RLC_Rec.AM_VTS := 0;

p_RLC_Rec.AM_VRR := 0;

p_RLC_Rec.AM_VRH := 0;

p_RLC_Rec.UM_VTUS := 0;

p_RLC_Rec.UM_VRUR := 0;

p_RLC_Rec.UM_VRUX := 0;

p_RLC_Rec.UM_VRUH := 0;

p_RLC_Rec.TimeStampLastReceivedPDU := {

SFN := { Number :=0 },

Subframe := { Number := 0 },

HSFN := { Number := 0 },

Slot := { FirstSlot := true }   /* @sic R5w180111: Slot sic@ */

};

p_RLC_Rec.TxPRBS_Pos := 0;

p_RLC_Rec.RxPRBS_Pos := 0;

p_RLC_Rec.TxDataSize := 0;

p_RLC_Rec.RxDataSize := 0;

p_RLC_Rec.Next_PDCP_TX_SN := 0;

p_RLC_Rec.Next_PDCP_RX_SN := 0;

p_RLC_Rec.UL_PDCP_SDU_size := 100;

p_RLC_Rec.UL_PDCP_SDU_scaling := false;

}

After change:

function f_InitRLC_Record(out RLC_SS_State p_RLC_Rec,

RLC_Mode_Type p_RLC_Mode)

{

if  ( p_RLC_Mode == UM_Mode) {

p_RLC_Rec.t_Reordering := 0.05; // FFS

}

else {

p_RLC_Rec.t_Reordering := 0.08;

}

p_RLC_Rec.t_PollRetransmit := 0.08;

p_RLC_Rec.t_StatusProhibit := 0.06; // @sic R5s100039 sic@

p_RLC_Rec.pollPDU := 128;

p_RLC_Rec.pollByte := 125000;

p_RLC_Rec.maxRetxThreshold := 4;

p_RLC_Rec.UM_SN_Size := 10; // FFS originally 10;  tsc_UM_SN_Size_5 default ??

// Initialisation of other parameters

p_RLC_Rec.AM_VTS := 0;

p_RLC_Rec.AM_VRR := 0;

p_RLC_Rec.AM_VRH := 0;

p_RLC_Rec.UM_VTUS := 0;

p_RLC_Rec.UM_VRUR := 0;

p_RLC_Rec.UM_VRUX := 0;

p_RLC_Rec.UM_VRUH := 0;

p_RLC_Rec.TimeStampLastReceivedPDU := {

SFN := { Number :=0 },

Subframe := { Number := 0 },

HSFN := { Number := 0 },

Slot := { FirstSlot := true }   /* @sic R5w180111: Slot sic@ */

};

p_RLC_Rec.TxPRBS_Pos := 0;

p_RLC_Rec.RxPRBS_Pos := 0;

p_RLC_Rec.TxDataSize := 0;

p_RLC_Rec.RxDataSize := 0;

p_RLC_Rec.Next_PDCP_TX_SN := 0;

p_RLC_Rec.Next_PDCP_RX_SN := 0;

p_RLC_Rec.UL_PDCP_SDU_size := 100;

p_RLC_Rec.UL_PDCP_SDU_scaling := false;

}