3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210770

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1822 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN at Step 2 the UE performs a Mobility Registration, but the Registration Accept message does not contain the list of non-allowed areas. This results in the UE deleting the Service Area list which contains non-allowed areas. According to 24.501 cl 5.3.5.2, Quote If the network does not convey the service area restrictions to the UE in the Service area list IE of a REGISTRATION ACCEPT message, the UE shall treat all tracking areas in the registered PLMN and its equivalent PLMN(s) in the registration area as allowed area and delete the stored list of "allowed tracking areas" or the stored list of "non-allowed tracking areas". Unquote Thus, after the Mobility Registration, the UE will consider the NR Cell 11 as “allowed”. This will cause failure in Step 9, when the UE will trigger the Emergency call, whereas the TTCN does not expect it. In order to address this issue, it is proposed to include the TAI of NR Cell 11 in the IE ServiceAreaIdList in the Registration Accept at Step 4. A Prose CR will be raised for this. In the current TTCN implementation, there is a problem with Emergency PDU Session establishment when the UE supports s1 capability. In the function fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_NR5GC_PDUSessionEstablishment f_NR5GC_PDUSessionEstablishment_InnerLoop in case the s1 capability is supported, then following code is executed : if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); } } This results in the EPSBearer ID incrementing. When the variable v_GSM_MobilityInfo is passed to function f_Get_NG_PDUSessionEstablishmentAccept, the IE Mapped EPS BearerContext IE has EPS Bearer ID set as somevalue (depending on whats passed), but the IE AuthorizedQoSFlowDecriptionIE parameters always hardcodes the EPS Bearer Identity to 5. Thus a PDU Session Establishment Accept message is sent with mismatching EPS Bearer ID in MappedEPSBearerContexts and AuthorizedQoSFlowDecription for the Emergency PDU Session. This causes the UE to transmit a PDU Session Modification REQ, which causes testcase failure. It is proposed that in the function f_BuildDefaultQosFlow for the case when flow number =2 instead of hardcoding ‘50’O, p_EPSBearerId is sent. A Prose CR will be raised for this. An alternative solution to this is to run this testcase with InterWorkWithEPS set as FALSE. Since this is a NR only testcase, this could be a possibel solution to problem explained above. In the current TTCN implementation, after Step 9, the NR Cell 11 is still in non-allowedlist. Thus during postamble, the UE will not send a De-registration REQ , but will locally de-register upon Switch OFF. This needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Registration Accept IE in Step 4 to include the TAI of NR Cell 11 in the IE ServiceAreaIdList. Updated the implementation of function f_BuildDefaultQosFlow to remove the hardcoding of ‘50’O. Changed the postamble state from STATE_IDLE_1A DEREGISTERED. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_11_4_6_NR5GC_TestBody	3

1.2	Correction to function f_NR5GC_MobilityRegistration	6

1.3	Correction to function f_NR5GC_MobilityRegistration_Steps4_6	7

1.4	Correction to function f_BuildDefaultQosFlow	10

1.5	Correction to function f_TC_11_4_6_NR5GC	11

# Corrections required

## Correction to function fl_TC_11_4_6_NR5GC_TestBody

| Function name | fl_TC_11_4_6_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN at Step 2 the UE performs a Mobility Registration, but the Registration Accept message does not contain the list of non-allowed areas. This results in the UE deleting the Service Area list which contains non-allowed areas. According to 24.501 cl 5.3.5.2, Quote If the network does not convey the service area restrictions to the UE in the Service area list IE of a REGISTRATION ACCEPT message, the UE shall treat all tracking areas in the registered PLMN and its equivalent PLMN(s) in the registration area as allowed area and delete the stored list of "allowed tracking areas" or the stored list of "non-allowed tracking areas". Unquote Thus, after the Mobility Registration, the UE will consider the NR Cell 11 as “allowed”. This will cause failure in Step 9, when the UE will trigger the Emergency call, whereas the TTCN does not expect it. In order to address this issue, it is proposed to include the TAI of NR Cell 11 in the IE ServiceAreaIdList in the Registration Accept at Step 4. A Prose CR will be raised for this. |
| Summary of change | Updated the Registration Accept IE in Step 4 to include the TAI of NR Cell 11 in the IE ServiceAreaIdList. Please see below. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_4_6_NR5GC_TestBody() runs on NR5GC_PTC { //@siclog "Steps 1" siclog@ // NR Cell 11 as "Serving cell", NR Cell 1/2 as "Non-Suitable "Off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 2" siclog@ // Check: Does in the next 5 sec the UE transmit an RRCSetupRequest message? f_NR5GC_MobilityRegistration(nr_Cell11); // @sic R5s210635 sic@ f_IP_ChangeNrCell(IP, nr_Cell11); // @sic R5s210635 sic@ //@siclog "Steps 3 - 7" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 2"); //@siclog "Steps 8" siclog@ // Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT); //@siclog "Steps 9" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell11, 10.0); }

After Change:

function fl_TC_11_4_6_NR5GC_TestBody() runs on NR5GC_PTC { var TrackingAreaCode v_TAC_cell11 := f_NR_CellInfo_GetTAC (nr_Cell11); //WA#860433 WA#11_4_6 var NAS_PlmnId v_PLMN_cell11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); //WA#860433 WA#11_4_6 var template (value) ServiceAreaIdList v_ServiceAreaIdList := cs_NG_ServiceAreaIdList(v_PLMN_cell11, {bit2oct(v_TAC_cell11)}); //WA#860433 WA#11_4_6 //@siclog "Steps 1" siclog@ // NR Cell 11 as "Serving cell", NR Cell 1/2 as "Non-Suitable "Off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 2" siclog@ // Check: Does in the next 5 sec the UE transmit an RRCSetupRequest message? f_NR5GC_MobilityRegistration(nr_Cell11, -, -, -, -, -, -, v_ServiceAreaIdList); // @sic R5s210635 sic@ //WA#860433 WA#11_4_6 f_IP_ChangeNrCell(IP, nr_Cell11); // @sic R5s210635 sic@ //@siclog "Steps 3 - 7" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 2"); //@siclog "Steps 8" siclog@ // Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT); //@siclog "Steps 9" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell11, 10.0); }

## Correction to function f_NR5GC_MobilityRegistration

| Function name | f_NR5GC_MobilityRegistration |
| --- | --- |
| Reason for change | Change 1.1 above |
| Summary of change | Extended the argument list to include IE ServiceAreaIdList, and pass it further within the function Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_MobilityRegistration (NR_CellId_Type p_NR_CellId, template (present) NG_MobileIdentity p_RxdGUTI := ?, template NG_TrackingAreaId p_TAI := *, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) PLMN_List p_EPLMNs := omit) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, p_RxdGUTI, -,-,-,-,-,-, p_TAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(p_NR_CellId, v_GMM_MobilityInfo, p_GUTIToSend, p_TAIList, p_ReleaseConnection, -, -, p_EPLMNs); }

After Change:

function f_NR5GC_MobilityRegistration (NR_CellId_Type p_NR_CellId, template (present) NG_MobileIdentity p_RxdGUTI := ?, template NG_TrackingAreaId p_TAI := *, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) PLMN_List p_EPLMNs := omit, template (omit) ServiceAreaIdList p_ServiceAreaIdList:= omit /*WA#860433 WA#11_4_6*/) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, p_RxdGUTI, -,-,-,-,-,-, p_TAI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobilityRegistration_Steps4_6(p_NR_CellId, v_GMM_MobilityInfo, p_GUTIToSend, p_TAIList, p_ReleaseConnection, -, -, p_EPLMNs, p_ServiceAreaIdList /*WA#860433 WA#11_4_6*/); }

## Correction to function f_NR5GC_MobilityRegistration_Steps4_6

| Function name | f_NR5GC_MobilityRegistration_Steps4_6 |
| --- | --- |
| Reason for change | Change 1.2 above |
| Summary of change | Extended the argument list to include IE ServiceAreaIdList, and pass it further within the function Please see below. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_MobilityRegistration_Steps4_6(NR_CellId_Type p_NR_CellId, GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (omit) PLMN_List p_EPLMNs := omit) runs on NR5GC_PTC // @sic R5-20692 sic@ { //@sic R5-199036: added p_NwkFeatureSupport sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; if (isvalue(p_GUTIToSend)) { v_GUTIToSend := valueof(p_GUTIToSend); } if (isvalue(p_TAIList)) { v_TAIList := valueof(p_TAIList); } // Step 4 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, p_GMM_MobilityInfo_Type, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, p_EPLMNs, -, -, -, p_NwkFeatureSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(p_NR_CellId, p_SRB); // No SRB2 so must be on SRB1 //Step 6 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } }

After Change:

function f_NR5GC_MobilityRegistration_Steps4_6(NR_CellId_Type p_NR_CellId, GMM_MobilityInfo_Type p_GMM_MobilityInfo_Type, template (omit) NG_MobileIdentity p_GUTIToSend := omit, template (omit) NG_TrackingAreaIdList p_TAIList := omit, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (omit) PLMN_List p_EPLMNs := omit, template (omit) ServiceAreaIdList p_ServiceAreaIdList:=omit /*WA#860433 WA#11_4_6*/) runs on NR5GC_PTC // @sic R5-20692 sic@ { //@sic R5-199036: added p_NwkFeatureSupport sic@ var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (p_NR_CellId); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; if (isvalue(p_GUTIToSend)) { v_GUTIToSend := valueof(p_GUTIToSend); } if (isvalue(p_TAIList)) { v_TAIList := valueof(p_TAIList); } // Step 4 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, p_GMM_MobilityInfo_Type, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList, p_EPLMNs, -, -, -, p_NwkFeatureSupport, - , -, -, - , -, -, p_ServiceAreaIdList);//WA#860433 WA#11_4_6 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Step 5 f_NR5GC_Registration_Complete(p_NR_CellId, p_SRB); // No SRB2 so must be on SRB1 //Step 6 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } }

## Correction to function f_BuildDefaultQosFlow

| Function name | f_BuildDefaultQosFlow |
| --- | --- |
| Reason for change | In the current TTCN implementation, there is a problem with Emergency PDU Session establishment when the UE supports s1 capability. In the function fl_EmergencyCallAndRelease f_NR5GC_EmergencyCallWithRegistration f_NR5GC_EmergencyCallPDUSession f_NR5GC_PDUSessionEstablishment f_NR5GC_PDUSessionEstablishment_InnerLoop in case the s1 capability is supported, then following code is executed : if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); } } This results in the EPSBearer ID incrementing. When the variable v_GSM_MobilityInfo is passed to function f_Get_NG_PDUSessionEstablishmentAccept, the IE Mapped EPS BearerContext IE has EPS Bearer ID set as somevalue (depending on whats passed), but the IE AuthorizedQoSFlowDecriptionIE parameters always hardcodes the EPS Bearer Identity to 5. Thus a PDU Session Establishment Accept message is sent with mismatching EPS Bearer ID in MappedEPSBearerContexts and AuthorizedQoSFlowDecription for the Emergency PDU Session. This causes the UE to transmit a PDU Session Modification REQ, which causes testcase failure. It is proposed that in the function f_BuildDefaultQosFlow for the case when flow number =2 instead of hardcoding ‘50’O, p_EPSBearerId is sent. A Prose CR will be raised for this. An alternative solution to this is to run this testcase with InterWorkWithEPS set as FALSE. Since this is a NR only testcase, this could be a possible solution to problem explained above. |
| Summary of change | Updated the implementation of function f_BuildDefaultQosFlow to remove the hardcoding of ‘50’O. Please see below. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId /*WA#860433 WA#11_4_6 '50'O*/); } } <<SKIPPED CODE>>

## Correction to function f_TC_11_4_6_NR5GC

| Function name | f_TC_11_4_6_NR5GC |
| --- | --- |
| Reason for change | In the current TTCN implementation, after implementing changes 1.1/2/3 at Step 9 the NR Cell 11 is still in non-allowedlist. Thus during postamble, the UE will not send a De-registration REQ , but will locally de-register upon Switch OFF. This needs to be handled. |
| Summary of change | Changed the postamble state from STATE_IDLE_1A DEREGISTERED. Please see below. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_IDLE_1A); //@sic R5s210635 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell11, STATE_DEREGISTERED /*WA#860433 WA#11_4_6 STATE_IDLE_1A*/); //@sic R5s210635 sic@ <<SKIPPED CODE>>