3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210771

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1823 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallPDUSession |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementaiton of this function f_NR5GC_EmergencyCallPDUSession f_NR5GC_PDUSessionEstablishment is called. This expects that the ssc mode be set to crs_SSC_Mode(-, '001'B). This is not inline with 38.508 default message contents which specifies that the IE SSC mode is optional and that the contents should not be checked. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced crs_SSC_Mode(-, '001'B) with -. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC, and other testcases that need Emergency PDU Session |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_NR5GC_EmergencyCallPDUSession	2

# Corrections required

## Correction to function f_NR5GC_EmergencyCallPDUSession

| Function name | f_NR5GC_EmergencyCallPDUSession |
| --- | --- |
| Reason for change | In the current TTCN implementaiton of this function f_NR5GC_EmergencyCallPDUSession f_NR5GC_PDUSessionEstablishment is called. This expects that the ssc mode be set to crs_SSC_Mode(-, '001'B). This is not inline with 38.508 default message contents which specifies that the IE SSC mode is optional and that the contents should not be checked. This needs to be addressed. |
| Summary of change | Replaced crs_SSC_Mode(-, '001'B) with -. Please see below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, p_SRB, -, // @sic R5s210518 sic@ omit, omit, crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2", -, noRrcConnectionRelease); // @sic R5s210515 sic@ don't need the extra delay here as there's further IMS signalling // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); // @sic R5s210518 sic@

After Change:

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, p_SRB, -, // @sic R5s210518 sic@ omit, omit, crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, -, //WA#860433 WA#11_4_6 crs_SSC_Mode(-, '001'B), // @sic R5s210518 sic@ -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2", -, noRrcConnectionRelease); // @sic R5s210515 sic@ don't need the extra delay here as there's further IMS signalling // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); // @sic R5s210518 sic@