3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210795

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4632 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.1.1.2a (TDD mode) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_extDRX-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In step 15, TDD subframe numbers should be 1 associated with PRACH Config Index 51 according to TS 36.211 table 5.7.1-2. But current TDD subframe numbers is 7 associated with PRACH Config Index 51 random access configurations for preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change TDD subframe numbers in random access configurations for preamble in step 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Corrections required

## 1.1 Change 1

| Function name | testcase f_TC_8_1_1_2a_EUTRA () |
| --- | --- |
| Reason for change | In step 15, TDD subframe numbers should be 1 associated with PRACH Config Index 51 according to TS 36.211 table 5.7.1-2. But current TDD subframe numbers is 7 associated with PRACH Config Index 51 random access configurations for preamble. |
| Summary of change | Change TDD subframe numbers in random access configurations for preamble in step 15. |
| TTCN module | ATS_LTE_A_PRO\8_1\RRC_Paging_eDRX.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_1_2a_EUTRA ( ) runs on EUTRA_PTC

{

…

//@siclog "Step 15" siclog@

//The SS sets eDRX-Allowed-r13 IE to true in SystemInformationBlockType1 and changes changes prach-ConfigIndex in SystemInformationBlockType2

//Configure SS to enable report of PRACH preamble reception

f_EUTRA_SS_ConfigRachPreambleIndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable);

//Change the prach config index in system information.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

f_EUTRA_CellInfo_SetPRACH_ConfigurationIndex (eutra_Cell1, 3, true); /* @sic R5s170923 ch.6 sic@ */

v_SubFrameNum_FDD_1 := 1; // FDD subframe number associated with PRACH Config Index 3 according to TS 36.211 table 5.7.1-2

}

else if (v_EUTRA_FDD_TDD_Mode == TDD) {

f_EUTRA_CellInfo_SetPRACH_ConfigurationIndex (eutra_Cell1, 51, true); /* @sic R5s170923 ch.6 sic@ */

v_SubFrameNum_TDD := 7; // in @sic R5s110197 sic@ indicated as associated with default PRACH Config Index 51

}

f_Set_SIB1_eDRXAllowed(eutra_Cell1); //SIB1 set to include eDRX-Allowed-r13 IE

//Configure PRACH at SS

f_EUTRA_SS_CommonCellConfig (eutra_Cell1,  /* @sic R5s170923 ch.5 sic@ */

cas_CellConfig_UpdatePRACH_Info_REQ(eutra_Cell1,

cs_TimingInfo_Now,

f_EUTRA_CellInfo_GetPrach_Config(eutra_Cell1),

f_EUTRA_SS_InitialisePRACH_Config_v1310(eutra_Cell1)));

…

}

After change

function f_TC_8_1_1_2a_EUTRA ( ) runs on EUTRA_PTC

{

…

//@siclog "Step 15" siclog@

//The SS sets eDRX-Allowed-r13 IE to true in SystemInformationBlockType1 and changes changes prach-ConfigIndex in SystemInformationBlockType2

//Configure SS to enable report of PRACH preamble reception

f_EUTRA_SS_ConfigRachPreambleIndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable);

//Change the prach config index in system information.

if (v_EUTRA_FDD_TDD_Mode == FDD) {

f_EUTRA_CellInfo_SetPRACH_ConfigurationIndex (eutra_Cell1, 3, true); /* @sic R5s170923 ch.6 sic@ */

v_SubFrameNum_FDD_1 := 1; // FDD subframe number associated with PRACH Config Index 3 according to TS 36.211 table 5.7.1-2

}

else if (v_EUTRA_FDD_TDD_Mode == TDD) {

f_EUTRA_CellInfo_SetPRACH_ConfigurationIndex (eutra_Cell1, 51, true); /* @sic R5s170923 ch.6 sic@ */

v_SubFrameNum_TDD := 1; // in @sic R5s110197 sic@ indicated as associated with default PRACH Config Index 51

}

f_Set_SIB1_eDRXAllowed(eutra_Cell1); //SIB1 set to include eDRX-Allowed-r13 IE

//Configure PRACH at SS

f_EUTRA_SS_CommonCellConfig (eutra_Cell1,  /* @sic R5s170923 ch.5 sic@ */

cas_CellConfig_UpdatePRACH_Info_REQ(eutra_Cell1,

cs_TimingInfo_Now,

f_EUTRA_CellInfo_GetPrach_Config(eutra_Cell1),

f_EUTRA_SS_InitialisePRACH_Config_v1310(eutra_Cell1)));

…

}