3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210796

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4633 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_GetAdditionalUpdateType |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently ttcn is not correctly handling addUpdateType IE in attachRequest for CIoT optimization testcases. CP & UP CIoT optimization testcases would fail because of mismatch in pnb_CIOT field, as the field is currently hardcoded to ‘00’B. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Template cr_AdditionalUpdateType_CIOT is used in case of CIoT optimization testcases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TC 23.1.x, 23.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_GetAdditionalUpdateType |
| --- | --- |
| Reason for change | Currently ttcn is not correctly handling addUpdateType IE in attachRequest for CIoT optimization testcases. CP & UP CIoT optimization testcases would fail because of mismatch in pnb_CIOT field, as the field is currently hardcoded to ‘00’B. |
| Summary of change | Template cr_AdditionalUpdateType_CIOT is used in case of CIoT optimization testcases |
| TTCN module | EUTRA_NASSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_GetAdditionalUpdateType(EUTRA_ATTACH_Type p_ForcedAttach := NORMAL) runs on EUTRA_PTC return template AdditionalUpdateType

{ /* @sic R5-103681 sic@ */

var template AdditionalUpdateType v_AdditionalUpdateType := omit;

if (f_EUTRA_AttachTypeCheck(p_ForcedAttach) or f_GetTestcaseAttrib_CIOT_CP( testcasename()) or f_GetTestcaseAttrib_CIOT_UP( testcasename())) { // @sic R5s110176, R5-176872 sic@

v_AdditionalUpdateType := cr_AdditionalUpdateTypeAny ifpresent;

}

return v_AdditionalUpdateType;

}

After change:

function f_GetAdditionalUpdateType(EUTRA_ATTACH_Type p_ForcedAttach := NORMAL) runs on EUTRA_PTC return template AdditionalUpdateType

{ /* @sic R5-103681 sic@ */

var template AdditionalUpdateType v_AdditionalUpdateType := omit;

if(f_GetTestcaseAttrib_CIOT_CP(testcasename()) or f_GetTestcaseAttrib_CIOT_UP(testcasename())) {

v_AdditionalUpdateType := cr_AdditionalUpdateType_CIOT ifpresent;

} else if (f_EUTRA_AttachTypeCheck(p_ForcedAttach)) { // @sic R5s110176, R5-176872 sic@

v_AdditionalUpdateType := cr_AdditionalUpdateTypeAny ifpresent;

}

return v_AdditionalUpdateType;

}