3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210808

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1841 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	6

4.1	HiSilicon Balong 5000	6

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.4.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_4_4_NR5GC |
| --- | --- |
| Reason for change | For step 12, signalling for PDU session release should be sent on SRB 2 |
| Summary of change | Correct SRB ID |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { …………………….. // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 subject to prose CR approval at RAN5#92 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } // @siclog "Step 13" siclog@ // Void // @siclog "Step 14" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 15" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) // Prose CR RRC Release fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 16"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16 "); }

After Change

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { …………………….. // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 subject to prose CR approval at RAN5#92 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } // @siclog "Step 13" siclog@ // Void // @siclog "Step 14" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 15" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) // Prose CR RRC Release fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 16"); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16 "); }

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_4_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_4_PIXIT.xml

# References

| [1] | R5s210809 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.4 |
| --- | --- |