3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210853

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0737 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall over IMS test case 21.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To verify eCall over IMS test case 21.6 using IWD 21wk24 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to eCall over IMS test case 21.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 34.229-1 CR 1473 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	f_IMS_ECall_InfoRequest_MessageHeaderRX	3

2.2	f_ECall_MSDUpdate_AnnexC47	4

2.3	cr_EmergencyCallData_Ack_ActionResult	6

2.4	cr_AckType	7

3	Branches executed	7

4	Execution Log Files	8

4.4	Qualcomm SA515M	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 21.6 which is part of the iwd-TTCN3-B2020-09_D21wk24.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case:	21.6

ATS Version:	iwd-TTCN3-B2020-09_D21wk24.zip

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SA515M

Verification Status:	PASS

# Corrections required

## f_IMS_ECall_InfoRequest_MessageHeaderRX

| Function name | f_IMS_ECall_InfoRequest_MessageHeaderRX |
| --- | --- |
| Reason for change | Keeping different expected values for callInfo “purpose” as per condition A3 and A4 |
| Summary of change | Adding the check for condition A4 and the expected callInfo “purpose” value to “EmergencyCallData.Control” |
| TTCN module | IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before change

function f_IMS_ECall_InfoRequest_MessageHeaderRX() runs on IMS_PTC return template (present) MessageHeader

{

var template (present) MessageHeader v_MessageInfo_Message := cr_MessageHeader_Dummy;

var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogRemote);

var charstring v_PackageName := "EmergencyCallData.eCall.MSD";

v_MessageInfo_Message.via             := f_IMS_SipRequest_ViaRX();

v_MessageInfo_Message.fromField       := f_IMS_RequestInDialog_FromHeaderRX();

v_MessageInfo_Message.toField         := f_IMS_RequestInDialog_ToHeaderRX();

v_MessageInfo_Message.callId          := f_IMS_PTC_ImsInfo_DialogGetCallId();

v_MessageInfo_Message.cSeq            := cr_CseqDef(v_CSeqValue, "INFO");

v_MessageInfo_Message.maxForwards     := cr_MaxForwardsDef;

v_MessageInfo_Message.contentType     := cr_ContentTypeMIME;

v_MessageInfo_Message.contentDisposition :=  cr_ContentDisposition("Info-Package");

v_MessageInfo_Message.callInfo        := cr_CallInfo(-, v_PackageName);

v_MessageInfo_Message.infoPackage     := cr_InfoPackage(v_PackageName);

return v_MessageInfo_Message;

}

After change

function f_IMS_ECall_InfoRequest_MessageHeaderRX(EmergencyCallData_Control p_EmergencyCallData_Control) runs on IMS_PTC return template (present) MessageHeader

{

var template (present) MessageHeader v_MessageInfo_Message := cr_MessageHeader_Dummy;

var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogRemote);

var charstring v_PackageName := "EmergencyCallData.eCall.MSD";

var charstring v_ControlPackageName := "EmergencyCallData.Control";

v_MessageInfo_Message.via             := f_IMS_SipRequest_ViaRX();

v_MessageInfo_Message.fromField       := f_IMS_RequestInDialog_FromHeaderRX();

v_MessageInfo_Message.toField         := f_IMS_RequestInDialog_ToHeaderRX();

v_MessageInfo_Message.callId          := f_IMS_PTC_ImsInfo_DialogGetCallId();

v_MessageInfo_Message.cSeq            := cr_CseqDef(v_CSeqValue, "INFO");

v_MessageInfo_Message.maxForwards     := cr_MaxForwardsDef;

v_MessageInfo_Message.contentType     := cr_ContentTypeMIME;

v_MessageInfo_Message.contentDisposition :=  cr_ContentDisposition("Info-Package");

if (match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control.choice.request.datatype))

{

v_MessageInfo_Message.callInfo := cr_CallInfo(-, v_ControlPackageName);

}

else

{

v_MessageInfo_Message.callInfo := cr_CallInfo(-, v_PackageName);

}

v_MessageInfo_Message.infoPackage     := cr_InfoPackage(v_PackageName);

return v_MessageInfo_Message;

}

## f_ECall_MSDUpdate_AnnexC47

| Function name | f_ECall_MSDUpdate_AnnexC47 |
| --- | --- |
| Reason for change | Adding the new function parameter in the function call and “received” in ack is optional as per RFC 8147 |
| Summary of change | Added the new parameter and made “received” of ack as optional |
| TTCN module | IMS_eCall_Functions.ttcn |
| MCC160 Comment |  |

Before change

function f_ECall_MSDUpdate_AnnexC47(template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := cs_IMS_RoutingInfo(f_IMS_PTC_ImsInfo_GetTransportProtocol());

var template (value) MessageHeader v_MessageHeader_Step4;

var template (value) MessageBody v_MessageBody_Step4;

var template (present) MessageHeader v_MessageHeader_Step6;

var InternetProtocol_Type v_Protocol;

var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

var charstring v_BoundaryStep4 := "boundaryXXX";

var charstring v_CidUrl_Step4 := "test-info@3gpp.org";

var charstring v_CidUrl_Step4_header := "<cid:test-info@3gpp.org>";

var charstring v_CidUrl_Step6;

var charstring p_cURI := px_IMS_CalleeContactUri;

var SipUrl v_ContactUrlUEExpected := f_SIP_BuildSipUri_TX(p_cURI);

// Step 4. SS sends SIP_INFO to request MSD update

v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4_header);

v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_CidUrl_Step4, p_EmergencyCallData_Control_Step4);

IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4)));

//Step 5. UE responds with 200 OK

v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol();

f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4)));

// Step 6. UE sends SIP_INFO with MSD update.

v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX();

v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(v_ContactUrlUEExpected, v_MessageHeader_Step6, cr_MessageBody_MIME)));

v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol);

if (match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype)) {

// Special case for TC 21.6: A.2.19 with condition A4

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataControlXML,

v_CidUrl_Step6,

-,

cr_EmergencyCallData_Control_Ack(true, v_CidUrl_Step4));

} else {

// Normal case: UE sends MSD according to A.2.19 with condition A3

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataMSD,

v_CidUrl_Step6);

}

//Step 7 SS responds to INFO with 200 Ok

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_IMS_DATA_REQ.Request.Info.msgHeader))));

}

After change

function f_ECall_MSDUpdate_AnnexC47(template (value) EmergencyCallData_Control p_EmergencyCallData_Control_Step4 := cs_EmergencyCallData_Control_Request) runs on IMS_PTC

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL := cs_IMS_RoutingInfo(f_IMS_PTC_ImsInfo_GetTransportProtocol());

var template (value) MessageHeader v_MessageHeader_Step4;

var template (value) MessageBody v_MessageBody_Step4;

var template (present) MessageHeader v_MessageHeader_Step6;

var InternetProtocol_Type v_Protocol;

var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

var charstring v_BoundaryStep4 := "boundaryXXX";

var charstring v_CidUrl_Step4 := "test-info@3gpp.org";

var charstring v_CidUrl_Step4_header := "<cid:test-info@3gpp.org>";

var charstring v_CidUrl_Step6;

var charstring p_cURI := px_IMS_CalleeContactUri;

var SipUrl v_ContactUrlUEExpected := f_SIP_BuildSipUri_TX(p_cURI);

// Step 4. SS sends SIP_INFO to request MSD update

v_MessageHeader_Step4 := f_IMS_ECall_InfoRequest_MessageHeaderTX(v_BoundaryStep4, v_CidUrl_Step4_header);

v_MessageBody_Step4 := f_IMS_ECall_MIME_MessageBodyTX(v_BoundaryStep4, v_CidUrl_Step4, p_EmergencyCallData_Control_Step4);

IMS_Client.send(cas_IMS_Info_Request(v_RoutingInfo_DL, cs_INFO_Request(v_ContactUrlUE, v_MessageHeader_Step4, v_MessageBody_Step4)));

//Step 5. UE responds with 200 OK

v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol();

f_IMS_ReceiveResponse(v_MessageHeader_Step4, v_Protocol, cr_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Step4)));

// Step 6. UE sends SIP_INFO with MSD update.

v_MessageHeader_Step6 := f_IMS_ECall_InfoRequest_MessageHeaderRX(p_EmergencyCallData_Control_Step4);

v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Info_Request(cr_INFO_Request(v_ContactUrlUEExpected, v_MessageHeader_Step6, cr_MessageBody_MIME)));

v_CidUrl_Step6 := v_IMS_DATA_REQ.Request.Info.msgHeader.callInfo.callInfoBody[0].url;

f_IMS_MessageHeader_Request_CheckVia(v_IMS_DATA_REQ.Request.Bye.msgHeader, v_IMS_DATA_REQ.RoutingInfo.Protocol);

if (match(tsc_EmergencyCallData_DataType_invalidMSD, p_EmergencyCallData_Control_Step4.choice.request.datatype)) {

// Special case for TC 21.6: A.2.19 with condition A4

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataControlXML,

v_CidUrl_Step6,

-,

cr_EmergencyCallData_Control_Ack(*, v_CidUrl_Step4));

} else {

// Normal case: UE sends MSD according to A.2.19 with condition A3

f_IMS_ECall_MIME_DecodeAndCheckEmergencyCallData(v_IMS_DATA_REQ.Request.Info.messageBody.mimeMessageBody,

tsc_MimeType_EmergencyCallDataMSD,

v_CidUrl_Step6);

}

//Step 7 SS responds to INFO with 200 Ok

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_IMS_DATA_REQ.Request.Info.msgHeader))));

}

## cr_EmergencyCallData_Ack_ActionResult

| Function name | cr_EmergencyCallData_Ack_ActionResult |
| --- | --- |
| Reason for change | Incorrect Ack element “action” value |
| Summary of change | Corrected the value as per RFC8147 |
| TTCN module | IMS_XML_eCall_Templates.ttcn |
| MCC160 Comment |  |

Before change

template (present) EmergencyCallData_Ack_ActionResult_Type cr_EmergencyCallData_Ack_ActionResult(template (present) charstring p_Reason,

boolean p_Success) :=

{ /* @status    APPROVED (IMS) */

action_ := "send_data",

details := omit,

reason := p_Reason,

success := p_Success,

attr := omit

};

After change

template (present) EmergencyCallData_Ack_ActionResult_Type cr_EmergencyCallData_Ack_ActionResult(template (present) charstring p_Reason,

boolean p_Success) :=

{ /* @status    APPROVED (IMS) */

action_ := "send-data",

details := omit,

reason := p_Reason,

success := p_Success,

attr := omit

};

## cr_AckType

| Function name | cr_AckType |
| --- | --- |
| Reason for change |  |
| Summary of change | Making elem_list as optional |
| TTCN module | IMS_XML_eCall_Templates.ttcn |
| MCC160 Comment |  |

Before change

template (present) AckType cr_AckType(boolean p_AckRvd,

charstring p_Ref,

template (present) EmergencyCallData_Ack_ActionResultList_Type p_ActionResultList) :=

{ /* @status    APPROVED (IMS) */

received := p_AckRvd,

ref := p_Ref,

attr := omit,

sequence_list := {

{

actionResult_list := p_ActionResultList,

elem_list := {}

}

}

};

After change

template (present) AckType cr_AckType(boolean p_AckRvd,

charstring p_Ref,

template (present) EmergencyCallData_Ack_ActionResultList_Type p_ActionResultList) :=

{ /* @status    APPROVED (IMS) */

received := p_AckRvd,

ref := p_Ref,

attr := omit,

sequence_list := {

{

actionResult_list := p_ActionResultList,

elem_list := *

}

}

};

# Branches executed

The test case was executed on EUTRA band 4

# Execution Log Files

## Qualcomm SA515M

The Qualcomm SA515M IMS UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_21_6_LOG.html

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- PICS/PIXIT parameter file:

TC_21_6_PIXIT.xml

# References

| [1] | R5s210854: Supporting information for addition of eCall over IMS test case 21.6 |
| --- | --- |