3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210860

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1867 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The test case may fail during UECapability information procedure due to the problem highlighted in R5s210315. Due to the implementation of R5s210315 the default BSR configuration is used in the preamble and later reverted to use the standard L2 test case settings defined in TS 38.523-1 section 7.1.0. This is done by calling f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1). This update to BSR is not handled as the RRCSetup message is only updated for test cases that perform early contention resolution, for other cases the RRCSetup message is encoded and provided during cell configuration. Therefore the update of BSR configuration needs to be passed into f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS similar to other test cases (i.e. TC 7.1.1.3.4). - When the DRBs are configured SDAP related information is not provided. - According to the test prose N1 320-octet RLC SDUs on DRB1, N2 320-octet RLC SDUs on DRB2, and N3 320-octet RLC SDUs on DRB3 shall be sent to UE, TTCN implementation provides 5x320 +5*320 +5*320 data to be sent on a specific activation time / slot and further data to be sent in 10msec spacing as provided in NR_L2DataReq The DL grant configuration is configured to automatic mode with allowed PRBs (7-17), with mcs 28 this results in TB size of 12552. The configured data to be sent in a slot is around (38400) and therefore exceeds the TB size. SS cannot fulfil the requirement of sending the given data in a particular slot. - During the test body the bsr timer is changed to sf10240, as a result at the end of the test case when there is still pending data to be sent (after the fixed allocation of UL grant) the UE will only trigger SR upon this timer expiry, the watchdog timer of 1sec will result in TC failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduced new variable v_MAC_CellGroupConfig. Updated the variable using f_NR_CellInfo_GetMAC_CellGroupConfig and passed as a parameter to f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS. - Added cs_SDAP_Configuration_None to provide SDAP configuration. - It is proposed to change the DL grant to max allowed PRB / mcs for the bandwidth configured and this would allow the SS to send the data in one slot and the subsequent data as provided by NR_L2DataReq - Extended the timer to 12sec so that any pending grant can be received when SR is triggered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2.NR5GC, 7.1.1.3.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Change 1.2 is required for both ENDC and NR5GC versions of the testcase. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required to test case 7.1.1.3.2	3

1.1	Correction to f_TC_7_1_1_3_2_NR5GC	3

1.2	Correction to f_TC_7_1_1_3_2_NR_TestBody	8

# Corrections required to test case 7.1.1.3.2

## Correction to f_TC_7_1_1_3_2_NR5GC

| Function name | f_TC_7_1_1_3_2_NR5GC |
| --- | --- |
| Reason for change | - The test case may fail during UECapability information procedure due to the problem highlighted in R5s210315. Due to the implementation of R5s210315 the default BSR configuration is used in the preamble and later reverted to use the standard L2 test case settings defined in TS 38.523-1 section 7.1.0. This is done by calling f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1). This update to BSR is not handled as the RRCSetup message is only updated for test cases that perform early contention resolution, for other cases the RRCSetup message is encoded and provided during cell configuration. Therefore the update of BSR configuration needs to be passed into f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS similar to other test cases (i.e. TC 7.1.1.3.4). - When the DRBs are configured SDAP related information is not provided. |
| Summary of change | - Introduced new variable v_MAC_CellGroupConfig. Updated the variable using f_NR_CellInfo_GetMAC_CellGroupConfig and passed as a parameter to f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS. - Added cs_SDAP_Configuration_None to provide SDAP configuration |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; //@sic R5-196745 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; //@sic R5s191054 sic@ var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-197028 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ //@sic R5-197028 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // Now that at least one PDU session has been established, get all DRB ids required v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(3); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; v_DRBId3 := v_ListOfDRBs[2]; //@sic r5s190783 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3) }; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100)) ; //@sic R5s190347 sic@ v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100)) ;//@sic R5s190347 sic@ v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100)) ;//@sic R5s190347 sic@ //@sic R5s190347 r5-1905696 r5s191054 sic@ v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), //@sic r5s200553 R5-213455 sic@ cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), //@sic r5s200553 R5-213455 sic@ cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId3, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit))}; //@sic r5s200553 R5-213455 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, -, //MAC_CellGroupConfig v_DRBToAddModList, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

After Change:

function f_TC_7_1_1_3_2_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling var DRB_Identity v_DRBId1; var DRB_Identity v_DRBId2; var DRB_Identity v_DRBId3; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var IntegerList_Type v_ListOfDRBs; //@sic R5-196745 sic@ var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; //@sic R5s191054 sic@ var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-197028 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); // f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); //@sic R5s210315 R5s210434 sic@ removed //@sic R5-197028 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // Now that at least one PDU session has been established, get all DRB ids required v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(3); v_DRBId1 := v_ListOfDRBs[0]; v_DRBId2 := v_ListOfDRBs[1]; v_DRBId3 := v_ListOfDRBs[2]; //@sic r5s190783 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId1,-,-,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId2,-,-,cs_SDAP_Configuration_None), f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, v_DRBId3,-,-,cs_SDAP_Configuration_None) }; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100)) ; //@sic R5s190347 sic@ v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100)) ;//@sic R5s190347 sic@ v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100)) ;//@sic R5s190347 sic@ //@sic R5s190347 r5-1905696 r5s191054 sic@ v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); v_SDAP_Config_DRB.defaultDRB := false; v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit; v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId1, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), //@sic r5s200553 R5-213455 sic@ cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId2, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit)), //@sic r5s200553 R5-213455 sic@ cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB, v_DRBId3, cs_38508_PDCP_Config(ms1500,-,-,ms80,omit))}; //@sic r5s200553 R5-213455 sic@ // f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1); v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, v_RLC_BearerConfigList, v_MAC_CellGroupConfig, //MAC_CellGroupConfig v_DRBToAddModList, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true);

## Correction to f_TC_7_1_1_3_2_NR_TestBody

| Function name | f_TC_7_1_1_3_2_NR_TestBody |
| --- | --- |
| Reason for change | - According to the test prose N1 320-octet RLC SDUs on DRB1, N2 320-octet RLC SDUs on DRB2, and N3 320-octet RLC SDUs on DRB3 shall be sent to UE, TTCN implementation provides 5x320 +5*320 +5*320 data to be sent on a specific activation time / slot and further data to be sent in 10msec spacing as provided in NR_L2DataReq The DL grant configuration is configured to automatic mode with allowed PRBs (7-17), with mcs 28 this results in TB size of 12552. The configured data to be sent in a slot is around (38400) and therefore exceeds the TB size. SS cannot fulfil the requirement of sending the given data in a particular slot. - During the test body the bsr timer is changed to sf10240, as a result at the end of the test case when there is still pending data to be sent (after the fixed allocation of UL grant) the UE will only trigger SR upon this timer expiry, the watchdog timer of 1sec will result in TC failure. |
| Summary of change | - It is proposed to change the DL grant to max allowed PRB / mcs for the bandwidth configured and this would allow the SS to send the data in one slot and the subsequent data as provided by NR_L2DataReq - Extended the timer to 12sec so that any pending grant can be received when SR is triggered. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_2_NR_TestBody(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3) runs on NR_BASE_PTC { //Number of SDUs for each run const integer const_N1[4] := {13, 13, 7, 104}; const integer const_N2[4] := {25, 25, 50, 25}; const integer const_N3[4] := {50, 50, 50, 50}; const integer const_T1[4] := {20, 20, 20, 10}; const integer const_T2[4] := {500, 700, 500, 500}; //Amount of data to compare with (octets) const integer const_D1[4] := {4160, 4160, 2240, 33000}; const integer const_D2[4] := {8000, 8000, 10260, 8000}; const integer const_D3[4] := {16000, 7790, 16000, 16000}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var integer j; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; timer t_WatchDog := 1.0; // Local wait Timer //@sic R5s190347 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(4608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_NR_ResourceAllocation[3] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 for (j := 0; j <= 3; j:= j+1) { f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N1[j]); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N2[j]); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N3[j]); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing);

After Change:

function f_TC_7_1_1_3_2_NR_TestBody(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3) runs on NR_BASE_PTC { //Number of SDUs for each run const integer const_N1[4] := {13, 13, 7, 104}; const integer const_N2[4] := {25, 25, 50, 25}; const integer const_N3[4] := {50, 50, 50, 50}; const integer const_T1[4] := {20, 20, 20, 10}; const integer const_T2[4] := {500, 700, 500, 500}; //Amount of data to compare with (octets) const integer const_D1[4] := {4160, 4160, 2240, 33000}; const integer const_D2[4] := {8000, 8000, 10260, 8000}; const integer const_D3[4] := {16000, 7790, 16000, 16000}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_Timing; var integer j; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList1; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList2; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DRB_DataPerSlotList3; // to allocate DL grant based on nprb var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); timer t_WatchDog := 12.0; // Local wait Timer //@sic R5s190347 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(4608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); v_NR_ResourceAllocation[3] := f_NR_ResourceAllocation_UplinkGrant(9224, v_NR_UplinkBWP); // DL grant to be specified based on max NPRB / SCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_Now, resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(0, 4), cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_1_Params(v_SCS)); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // runs 1 to 4 for (j := 0; j <= 3; j:= j+1) { f_NR_HarqErrorHandling_Set(raiseInconc); // Start 'Harq Errors not tolerant' //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N1[j]); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N2[j]); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(crs_NR_PDCP_SDU_317B, const_N3[j]); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing);